3GPP TSG-RAN WG3 #121	R3-234321

# Toulouse, France, 21st  – 25th  August 2023

Agenda Item:	26.2

Source:	Ericsson

Title:	Discussion on NR Timing Resiliency and URLLC enhancements

Document for:	Discussion, Decision

# 1	Introduction

In RP#99, a new RAN Rel-18 WID is approved [1]. At RAN3#119bis, we discussed and endorsed some IE structure.

This paper continues the discussion.

# 2	Discussion

### 2.1	5GS network timing synchronization status and reporting

For the gNB reporting to AMF the timing synchronization status information, there are a few FFS in the IE definition in the endorsed baseline CRs, and the procedure and messaged used over NGAP are not concluded.

Related to Clock accuracy, we think it is beneficial to define the following choice, to reflect the gNB implementation:

| Parameters | IE strcuture |
| --- | --- |
| Clock accuracy | Choice structure: Case 1: ENUMERATED (not-applicable) Case 2: one Rough Value (unit ns) Case 3: Value range (low, high) Case 4: exact value (unit ns) |

Proposal 1: It is proposed RAN3 to discuss and agree that over NGAP, the choice structure with the above “choices” is used to allow gNB to report in different situations its “clock accuracy”.

Related to which procedure and message are used for gNB to perform reporting to AMF, a few proposals were mentioned at the last RAN3 meeting.

Our view is that the existing non UE associated message can be used.

Proposal 2: It is proposed RAN3 to discuss and agree that over NGAP, the non-UE associated NG Update procedure is used for gNB to send the time status report to AMF.

The continuous reporting of the 5G Clock quality information and the consistency are important and should be ensured in RRC_Connected during mobility.

The clock quality criteria, control information and the latest clock quality information sent to the UE should be included from the source RAN to the target during handover preparation. The target RAN should decide to accept or reject the handover based on the received information and its own capability. The target RAN feedbacks it is clock quality information so the source RAN could compare and choose the “right” target.

Proposal 3: It is proposed RAN3 to discuss and agree the continuous reporting of the 5G Clock quality information to the UE during mobility. The clock quality related information is sent to the target RAN during handover procedure, and the target RAN feedbacks its clock quality information.

### 2.2	Interworking with TSN network deployed in the transport network

SA2 has concluded and agreed the TSN topic, Some are impacting NG-RAN node, e.g.

  - N3 tunnel end point addresses that are used for the QoS Flow. The SMF/CUC may instruct the UPF and NG-RAN to assign a separate N3 tunnel end point address for each QoS Flow that may carry TSC streams so that the TN can distinguish the QoS Flows based on the N3 tunnel destination IP addresses.

  - When NG-RAN and UPF has signalled the support of AN-TL and CN-TL, the SMF/CUC may use the TL-Container to send…

SA2 also agreed the “mask-and-match stream identification function” at the last meeting.

When indicated by SMF, NG-RAN would need to support TSN TNL IP per QoS flow and per DRB.

SMF shall sent AN-TL container to RAN, the container may include get-request and set-request. A new non-UE associated procedure would be much clean approach

Proposal 4: It is proposed RAN3 to discuss and agree that upon the indication from SMF, one TNL IP is set up per QoS flow. And the new procedures to transfer AN-TL container.

# 3	Proposal

Proposal 1: It is proposed RAN3 to discuss and agree that over NGAP, the choice structure with the above “choices” is used to allow gNB to report in different situations its “clock accuracy”.

Proposal 2: It is proposed RAN3 to discuss and agree that over NGAP, the non-UE associated NG Update procedure is used for gNB to send the time status report to AMF.

Proposal 3: It is proposed RAN3 to discuss and agree the continuous reporting of the 5G Clock quality information to the UE during mobility. The clock quality related information is sent to the target RAN during handover procedure, and the target RAN feedbacks its clock quality information.

Proposal 4: It is proposed RAN3 to discuss and agree that upon the indication from SMF, one TNL IP is set up per QoS flow. And the new procedures to transfer AN-TL container.

The Text Proposals is submitted in [2].

# 4	Reference

[1]	RP-230754		New WID on NR Timing Resiliency and URLLC enhancements

[2]	R3-234322		Text Proposals on Support NR Timing Resiliency and URLLC enhancements, Ericsson