3GPP TSG-RAN WG3 Meeting #121	R3-234341

Toulouse, France, 21 – 25 August, 2023

Agenda item:	12.2.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS 38.423) Discussion on AI/ML Load Balancing

Document for:	Text Proposal

# 1	Introduction

In RAN3 #120 we made the following agreements and captured some open aspects:

Partial Reporting

There is consensus on the benefits of enabling the requesting node to optimize the measurement request by indicating whether requested measurements can be reported in full or in part. It needs to be further discussed whether such enhancements can be introduced in Rel18.

It was discussed to select a solution for partial reporting and two options remain to be further discussed:

- Option 1: Introduce the indicator in the request message that informs the requested node if the partial reporting is allowed or not allowed.

- Option 2: Introduce the characteristic bitmap in the request message that informs the requested node which measurements must be reported.

- Option 3: No explicit IE in the request message.

Cause Values

Introduce a Cause IE for the Measurement Failed Report Characteristics per cell and Measurement Failed Report Characteristics per node.

The group discussed and had the common understanding as below:

Cause value measurement not supported -> is covered by legacy cause “Measurement not Supported For The Object,”

Cause value measurement temporarily not available -> is covered by legacy cause “Measurement Temporarily not Available,”

It is FFS whether any more cause values should be added to the legacy list of causes.

UE Performance Feedback

It needs to be further discussed whether and how any further granularity needs to be introduced for the selection of measurements to be reported as part of the UE Performance Feedback.

The problem of how to signal to the reporting node time configurations for the UE Performance Feedback measurement reporting is acknowledged.

Time configuration for UE Performance Feedback measurement reporting consists at least of:

- Time duration of the UE Performance Feedback measurement collection (the time duration starting at handover execution and including the last UE Performance Feedback report)

- Whether, within such time duration, measurements are reported periodically or one time

- In case of periodic reporting, the period of UE Performance Feedback measurements (whether existing of new IE)

In this contribution, we address the open aspects above and provide a TP for TS 38.423.

# 2	Partial Reporting

In the RAN3 #120 it was agreed that the partial reporting mechanism is supported in the agreed AI/ML information procedures (AI/ML INFORMATION REQUEST/RESPONSE/UPDATE, name is FFS), but the exact solutions need to be discussed. If a number of measurements are requested and only some of those can be provided by a neighbour it may be more efficient not to fail the whole procedure. Based on discussions from previous RAN3 meetings the following options were down-selected (eliminating Option 2):

- Option 1: Introduce the indicator in the request message that informs the requested node if the partial reporting is allowed or not allowed.

- Option 3: No explicit IE in the request message.

In Option 1, a node requesting measurements indicates in the request message through a binary indication, whether it supports to receive successfully only some of the requested measurements or not. The motivation behind this is that if the measurements are needed, for example, to execute model inference, a node may wish to receive all of the requested measurements in full. In this case, partial success is not supported. Otherwise, if partial success is allowed, the responding node may provide only some of the requested measurements without leading to an error message.

In practice, however, it is possible that a node places a measurement request, out of which some measurements are mandatory, while the rest may be only optional. The binary partial success flag does not have sufficient granularity to indicate to a neighbour “which” are the mandatory measurements from the list of requesting measurements and which measurements could be optionally received.

Observation 1: Partial success flag is not granular enough to indicate mandatory combinations of measurements.

Besides, a node may request a number of measurements from a neighbouring node and if the neighbour does not provide all the needed requested measurements then it may cancel the reporting. Therefore, the usefulness of introducing a partial success flag is unclear in the presence of current reporting mechanism.

One possible benefit from introducing a flag in the request is to introduce a prioritization in the requested measurements in case the responding node does not have enough available resources to provide them all; the responding node will try to provide the measurements with the highest priority to the requesting node if its resources are inadequate. However, a binary flag is not sufficient to enable a prioritization in the requested measurements since it may only prioritize them in terms of an “all” or “none” approach. In our view, there is no other useful enhancement to the existing partial reporting mechanism besides possibly a measurement priority level introduction. If such prioritization does not seem necessary, then Option 3, where no explicit IE is introduced in the request message, is our preferred solution.

Proposal 1: RAN3 to discuss whether a measurement priority level should be associated with a requested measurement. If the introduction of a priority level seems unnecessary then we support Option 3, where no explicit IE is introduced in the request message, as our preferred solution.

# 3	Additional Cause Values

The cause values being proposed over Xn are meant to be given by the target NG-RAN node along with the failed measurement indication to enable the source NG-RAN node to take follow up actions. The final aim is to make that measurement procedure succeed whenever possible. However, a cause value alone will not enable the source NG-RAN node to take a corrective action since it just indicates what is not supported by the target node; it does not provide a node capability in what it may support. Without such capability information, the node would need to explore different requests until it receives a measurement successfully and any possible retry of the measurement procedure by the source may also fail.  For example, if a measurement is temporarily not available, this indication does not tell anything to the requesting node about when the measurement will become available. If a measurement is not available with the requested periodicity or with current combination of requested information, the requesting node cannot know which the correct parameters are to request those measurements, e.g., in terms of a required periodicity, correct input combination, etc. So, this information provided by the responding node does not help the requesting node to place in a more efficient way measurement requests in the future.

Observation 2: If a node indicates it does not temporarily support a measurement or that it does not support a measurement with a certain combination of parameters, a neighbouring node requesting this information does not know when the measurement will be supported or with which parameters the measurement shall be requested again in the future.

Introducing failure causes for all possible measurement periodicities, input combinations or other model parameters is not scalable. Furthermore, cause values are typically used for statistic purposes and KPIs. They are not used to model network behaviour. Therefore, we think that we should consider simple failure cause values for AI/ML information such as “measurement not supported” and “measurement not available” and thus, to reuse the existing cause values “Measurement not Supported For The Object” and “Measurement Temporarily not Available” described in TS 38.423, section 9.2.3.2.

Proposal 2: The legacy list of cause values is sufficient for AI/ML reporting purposes and there is no need to introduce new cause values.

In addition, for AI/ML functionality to work correctly in a real network deployment, operator would need to configure the nodes such that they have an understanding of what information/predictions their neighbours can provide. For instance, a node should be aware of what is a feasible set of periodicities with which a neighbour can provide predictions and no separate cause values for such would need to be introduced.

On the other hand, if measurement requesting procedure needs to become truly automated over the Xn, NG-RAN nodes could indicate their supported capabilities in advance to their neighbours to optimize measurement requests according to what a node is capable to provide at each time instant and to optimize the requested information. However, this could be an enhancement for consideration in Rel-19.

Proposal 3: Consider further enhancements in the measurement request procedure according to node capability in Rel-19.

# 4	Time Configuration for UE Performance Feedback

In the last meeting, we acknowledged the issue that a node requesting UE Performance Feedback from another node needs to indicate to this node time configurations for UE Performance Feedback measurement reporting, where we defined a Time Configuration to consist at least of the following information:

- Time duration of the UE Performance Feedback measurement collection (the time duration starting at handover execution and including the last UE Performance Feedback report)

- Whether, within such time duration, measurements are reported periodically or one time

- In case of periodic reporting, the period of UE Performance Feedback measurements (whether existing of new IE)

In this contribution we address the remaining issues related to timing information of signalling UE Performance Feedback. UE Performance Feedback is collected by a target NG-RAN node after a Handover. There has been discussions whether UE Performance Feedback should be sent in one time or be reported periodically.

Figure 1 illustrates a Handover example where UE Performance feedback is sent from a target node to a source node. In the figure, we indicate by t0 the time instant when Handover is successfully executed at the target NG-RAN node (gNB2) and the Time Duration of UE Performance measurement collection by T. The time tk = t0 + T indicates the time when the last UE Performance measurement report can be sent by the target node. In case of periodic reporting, UE Performance measurements can be collected during also intermediate times t1 ,t2 ,.., tk-1. As can be seen in the figure, depending on the value of T, cell sizes, and the UE mobility pattern with respect to velocity and direction, the received UE Performance Feedback may reflect different information of UE Performance that may not be relevant when it comes to evaluating the specific UE handover event. For example, in Figure 1 at time instant tk the UE has moved closer to another NG-RAN node. In addition, as can be seen in the figure, the correlation of UE Performance feedback to the handover under evaluation decreases during subsequent periodically reported time instants.

Observation 3: The correlation of UE Performance feedback to the handover under evaluation decreases during subsequent periodically reported time instants.

Figure 1 UE Performance feedback reporting from gNB2 to gNB1. Time t0 corresponds to the time when handover is successfully executed at gNB2. The time duration during which measurements for UE Performance are collected is indicated by T. Time last UE Performance measurement report can be sent at time t1.

In our view, UE Performance Feedback is intended to provide feedback shortly after the handover execution in order to reflect whether the handover decision was a good one or not. Providing UE performance feedback periodically may provide misleading information to the source node since the correlation of the reflected reported UE performance to the actual handover that needs to be evaluated decreases during subsequent reporting time instants.

Proposal 4: UE Performance feedback should be based on one-time reporting.

Furthermore, as can be seen in the figure, it is hard for the source node to set the time duration accurately in order to evaluate the UE Performance of a certain handover event. Even though it may be hard for a node to know the exact time duration during which UE Performance is reported a maximum time duration can be defined by the source node. This maximum time duration can also be a time interval that the source node is willing to keep the UE Context or part of the UE Context that is sufficient to correlate the UE to the received UE Performance.

Proposal 5: A source NG-RAN node configures a maximum duration during which it expects UE Performance Feedback.

In order to allow the target node to provide UE Performance Feedback during the right time instant we propose that the actual reporting is triggered through event-based conditions in the Time Configuration, configured by the source NG-RAN node, that allow the target NG-RAN node determine when to send UE Performance to the source node based on its experienced events, e.g., if another handover is triggered for this UE towards another cell.

Proposal 6: Actual reporting of UE Performance Feedback is triggered according to event configuration allowing the target NG-RAN node to determine when to send UE Performance to the source node.

An example of such event could be a subsequent handover triggered for this UE or the radio conditions of the UE drop below or exceed some threshold values.

Finally, we provide a TP for BL CR for TS 38.423 in the Annex.

Proposal 7: Discuss and agree the TP for BL CR for TS 38.423 in the Annex.

# 5	Conclusion

In this paper we make the following observations and proposals:

Observation 1: Partial success flag is not granular enough to indicate mandatory combinations of measurements.

Proposal 1: RAN3 to discuss whether a measurement priority level should be associated with a requested measurement. If the introduction of a priority level seems unnecessary then we support Option 3, where no explicit IE is introduced in the request message, as our preferred solution.

Observation 2: If a node indicates it does not temporarily support a measurement or that it does not support a measurement with a certain combination of parameters, a neighbouring node requesting this information does not know when the measurement will be supported or with which parameters the measurement shall be requested again in the future.

Proposal 2: The legacy list of cause values is sufficient for AI/ML reporting purposes and there is no need to introduce new cause values.

Proposal 3: Consider further enhancements in the measurement request procedure according to node capability in Rel-19.

Observation 3: The correlation of UE Performance feedback to the handover under evaluation decreases during subsequent periodically reported time instants.

Proposal 4: UE Performance feedback should be based on one-time reporting.

Proposal 5: A source NG-RAN node configures a maximum duration during which it expects UE Performance Feedback.

Proposal 6: Actual reporting of UE Performance Feedback is triggered according to event configuration allowing the target NG-RAN node to determine when to send UE Performance to the source node.

Proposal 7: Discuss and agree the TP for BL CR for TS 38.423 in the Annex.

# References

[1]		R3-233535, (BLCR to 38.423) for AI/ML for NG-RAN

# Annex	- TP for BLCR to TS 38.423

<<< start of changes >>>

### 8.4.AA	AI/ML Information Reporting Initiation (FFS on the name)

#### 8.4.AA.1	General

This procedure is used by an NG-RAN node to request the reporting of AI/ML related information to another NG-RAN node.

The procedure uses non UE-associated signalling.

Editor’s Note: FFS other information that can be requested using this procedure.

Editor’s Note: FFS content of AL/ML related information.

#### 8.4.AA.2	Successful Operation

Figure 8.4.AA.2-1: AI/ML Information Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the AI/ML INFORMATION REQUEST message to NG-RAN node2 to start AI/ML related information reporting and stop AI/ML related information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested AI/ML related information reporting according to the parameters given in the request in case the Registration Request IE is set to “start”; or

-	shall stop all cells AI/ML related information reporting and terminate the reporting in case the Registration Request IE is set to “stop”; or

-	FFS

If the Registration Request IE is set to “start” in the AI/ML INFORMATION REQUEST message and the Report Characteristics IE indicates cell specific AI/ML related information reporting, the Cell To Report List IE shall be included.

If NG-RAN node2 is capable to provide all of the requested information, it shall initiate the AI/ML related information reporting as requested by NG-RAN node1 and respond with the AI/ML INFORMATION RESPONSE message.

If NG-RAN node2 is capable to provide some but not all of the requested information, it shall initiate the AI/ML related information reporting for the admitted requested information and include the Node Measurement Initiation Result IE or the Per Cell Measurement Initiation Result IE or both in the AI/ML INFORMATION RESPONSE message.

If the Reporting Periodicity IE in the AI/ML INFORMATION REQUEST is present, this indicates the periodicity for the reporting of periodic AI/ML related information. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

If the Requested Prediction Time IE in the AI/ML INFORMATION REQUEST message is present, it indicates the specific point in time to which the prediction of the information requested applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

If the UE Performance Reporting Configuration IE in the AI/ML INFORMATION REQUEST message is present, it indicates a configuration for reporting of UE Performance measurements. NG-RAN node2 shall take this configuration into account when reporting UE Performance information.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the AI/ML INFORMATION REQUEST indicates the type of objects NG-RAN node2 shall perform measurements or prediction on. NG-RAN node2 shall include in the AI/ML INFORMATION UPDATE message:

-	the Predicted Radio Resource Status IE, if the first bit, “Predicted Radio Resource Status” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”. FFS on the details of Predicted Radio Resource Status IE.

-	the Predicted Number of Active UEs IE, if the second bit, “Predicted Number of Active UEs” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”;

-	the Predicted RRC Connections IE, if the third bit, “Predicted RRC Connections” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

-	the Average UE Throughput DL IE, if the fourth  bit, “Average UE Throughput DL” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

-	the Average UE Throughput UL IE, if the fifth bit, “Average UE Throughput UL” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

-	the Average Packet Delay IE, if the sixth bit, “Average Packet Delay” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

-	the Average Packet Loss IE, if the seventh bit, “Average Packet Loss” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

#### 8.4.AA.3	Unsuccessful Operation

Figure 8.4.AA.3-1: AI/ML Information Reporting Initiation, unsuccessful operation

If none of the requested AI/ML related information reporting cannot be initiated, NG-RAN node2 shall send the AI/ML INFORMATION FAILURE message with an appropriate cause value.

#### 8.4.AA.4	Abnormal Conditions

For the same Measurement ID, if the initiating NG-RAN node1 does not receive either the AI/ML INFORMATION RESPONSE message or the AI/ML INFORMATION FAILURE message, the NG-RAN node1 may reinitiate the AI/ML Information Reporting Initiation procedure towards the same NG-RAN node, provided that the content of the new AI/ML INFORMATION REQUEST message is identical to the content of the previously unacknowledged AI/ML INFORMATION REQUEST message.

If the NG-RAN node2 receives an AI/ML INFORMATION REQUEST message which includes the Registration Request IE set to “stop” and if the NG-RAN node2 Measurement ID value received in the AI/ML INFORMATION REQUEST message is not used, the NG-RAN node2 shall initiate AI/ML INFORMATION FAILURE message with an appropriate cause value.

If the Report Characteristics IE bitmap is set to “0” (all bits are set to “0”) in the AI/ML INFORMATION REQUEST message then NG-RAN node2 shall initiate an AI/ML INFORMATION FAILURE message with an appropriate cause value.

If the NG-RAN node2 receives a AI/ML INFORMATION REQUEST message which includes the Registration Request IE set to “start” and the NG-RAN node1 Measurement ID IE corresponding to an existing on-going AI/ML information reporting, then NG-RAN node2 shall initiate a AI/ML INFORMATION FAILURE message with an appropriate cause value.

<<< next change >>>

#### 9.1.3.CC	AI/ML INFORMATION REQUEST (FFS on the name)

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested AI/ML related information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) (FFS on others) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss Eighth Bit = Energy Cost FFS on the coding | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | FFS |  | YES | ignore |
| UE Performance Reporting Configuration |  |  |  |  | YES | ignore |
| >Time Duration | M |  | INTEGER (1..1024) | Maximum duration during which UE Performance Feedback is provided |  |  |
| >Outgoing Handover Event | O |  | ENUMERATED (true, ...) |  |  |  |
| >Radio Condition Under Threshold Event | O |  |  | Event is triggered if UE radio conditions drop below the threshold |  |  |
| >>RSRP threshold | O |  | INTEGER (0..127) | Corresponds to information provided in the RSRP-Range IE as defined in TS 38.331 [10]. |  |  |
| >>RSRQ threshold | O |  | INTEGER (0..127) | Corresponds to information provided in the RSRQ-Range IE as defined in TS 38.331 [10]. |  |  |
| >>SINR | O |  | INTEGER (0..127) | Corresponds to information provided in the SINR-Range IE as defined in TS 38.331 [10]. |  |  |
| >Radio Condition Over Threshold Event | O |  |  | Event is triggered if UE radio conditions go over the threshold |  |  |
| >>RSRP threshold | O |  | INTEGER (0..127) | Corresponds to information provided in the RSRP-Range IE as defined in TS 38.331 [10]. |  |  |
| >>RSRQ threshold | O |  | INTEGER (0..127) | Corresponds to information provided in the RSRQ-Range IE as defined in TS 38.331 [10]. |  |  |
| >>SINR | O |  | INTEGER (0..127) | Corresponds to information provided in the SINR-Range IE as defined in TS 38.331 [10]. |  |  |

<<< end of changes >>>