3GPP TSG-RAN WG3 Meeting #121	R3-234351

Toulouse, France, August 21st – 25th, 2023

Agenda item:	20.2

Source: 	LG Electronics Inc.

Title: 	Discussions for open issues related to DL notification and data volume

Document for:	Discussion and Decision

# Introduction

In the last RAN3-120 meeting, the following was achieved for MT-SDT:

# 15_R18SDT

- Discuss the open issues above and take all the contributions into account

- Capture agreements and open issues

(moderator - ZTE)

Summary of offline disc R3-233330 noted

(TP for TS 37.483) Update of MT-SDT information over E1 in R3-233372 Agreed

(TP to MT-SDT TS 38.401) consideration on MT-SDT in R3-233404

- If determining that DL data packets are only mapped to SDT bearers as requested in step 1, the gNB-CU-UP includes the MT-SDT information in the DL DATA NOTIFICATION message.

Rev in R3-233485 Agreed unseen

(TP to BL CR TS 38.423 on MT-SDT) in R3-233405 Agreed

(TP for MT-SDT BL CR to TS 38.300) Handling of non-SDT during MT-SD in R3-233413 Agreed

(TP for MT-SDT BLCR to TS 37.480) Introduction of MT-SDT R3-233436 Agreed

In E1AP: MT-SDT indicator IE within MT-SDT information IE is not needed.

In XnAP: MT-SDT data size calculation includes total of both SDT signalling and SDT user plane data. Try to capture it into the TP.

In XnAP: Both MT-SDT indicator IE and MT-SDT Data Size IE are “Mandatory”.

In F1AP: MT-SDT indicator IE is “Mandatory”.

In F1AP: gNB-CU makes the MT-SDT decision, gNB-DU shall follow the decision.

When new DL data is coming through non-SDT bearer, the gNB-CU-UP shall send DL DATA NOTIFICATION message. 2

FFS on either excluding MT-SDT Information, or introducing a new indicator (e.g., Non MT-SDT Data) or other method.

When large size of new DL data is coming through SDT bearer, the gNB-CU-UP shall send DL DATA NOTIFICATION message.

FFS on either excluding MT-SDT Information, or introducing a new indicator (e.g., MT-SDT Oversize), or other method. To be

FFS: How to calculate MT-SDT Data Size for SDT DL data packets.

In this contribution, we discuss some open issues (highlighted above) related to DL notification and data volume, and provide our views.

# Discussion

## E1AP DL notification

Regarding the below agreement and FFS for non-SDT data arrival:

When new DL data is coming through non-SDT bearer, the gNB-CU-UP shall send DL DATA NOTIFICATION message.

FFS on either excluding MT-SDT Information, or introducing a new indicator (e.g., Non MT-SDT Data) or other method.

From the last meeting SOD [1], it is observed that, in case of non-SDT DL data arrival, all companies already agreed for CU-UP to send DL notification without MT-SDT info.

If there is no size to measure because of no DL SDT data arrival, there is no need to include this MT-SDT info. There seems no need to capture anything as this is already the specified behavior.

Proposal 1: E1AP DL Data Notification shall be sent without MT-SDT info, if new DL data arrives through non-SDT bearer.

Regarding the below agreement and FFS for oversize:

When large size of new DL data is coming through SDT bearer, the gNB-CU-UP shall send DL DATA NOTIFICATION message.

FFS on either excluding MT-SDT Information, or introducing a new indicator (e.g., MT-SDT Oversize), or other method.

We first would like to remind that the main purpose of MT-SDT Data Size in E1 Notification lies in “assistance info” to help CU-CP to decide whether to trigger MT-SDT paging or legacy paging. This size info over E1 Notification naturally indicates DL SDT data arrival.

Overall, we are not sure how “96000 bytes” came up from the beginning. Our guess is that 96000 bytes came from Rel-17 MO-SDT, from RRC IE of sdt-DataVolumeThreshould-r17, which was designed for UL direction where SDT is triggered from UE.

However, as mentioned above, here we are talking about “assistance info” from CU-UP to CU-CP. Based on which, CU-CP will decide what to do (either trigger MT-SDT paging or legacy paging). Namely, the decision is purely on CU-CP where CU-UP just provides some helpful information. The context of 96000 bytes discussed here is not the same as in RAN2 where byte96000 is used as one of MO-SDT triggering condition for a UE.

Moreover, looking at its INTEGER definition of the MT-SDT Data Size IE over E1AP and XnAP BL CRs, it is already defined with an extension marker (highlighted below):

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

#### 9.3.1.xxx	MT-SDT Information

This IE provides the assistant information for MT-SDT.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MT-SDT Data Size | M |  | INTEGER (1..96000, …) | Indicates the total data size for all SDT bearers. Unit: byte. |

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

This means that MT-SDT Data Size IE can represent any positive integer value. There is no maximum value that can be reported from CU-UP. Due to its contraint extension, this INTEGER information object will be encoded with a 1-bit prefix (extension flag), and when set to 0, the value will be encoded in an optimized form of 17 bitstring to represent the value range from 1 to 96000 (or could be bytes aligned depending on encoding rule chosen). Any value larger than 96000 can be conveyed with 1-bit prefix set to 1 with proper extension header indicating their size and content.

We think that there is no need to discuss this oversize issue!

Proposal 2: The MT-SDT Data Size IE in E1AP/XnAP BL CRs are already defined as an extensible INTEGER, and thus it can convey any value beyond 96000 at no problem. No need to discuss the oversize issue nor any additional indicator..

## How to calculate MT-SDT DL data volume

Regarding the following FFS:

FFS: How to calculate MT-SDT Data Size for SDT DL data packets.

[2] discussed on DL data volume calculation and provided background on how Rel-17 MO-SDT calculated UL data volume in the UE side. Based on which, for DL SDT data volume calculation, [2] proposed to take SDAP header into account, while excluding other L2 headers.

It is quite straightforward that DL data volume should be at least based on the received IP packet sizes (i.e. T-PDUs included in received G-PDUs over NG-U).

But with respect to taking SDAP header into account, there are a few things to consider.

According to TS 37.324 [3], for DL, SDAP header is included only when reflective QoS is used for NAS-level (i.e. SDF to QoS flow remapping) or AS-level (i.e. flow to DRB remapping). If NAS reflective QoS is used, then RQI bit is set in the DL PDU Session Information frame (defined in TS 38.415 [4]) carried in GTP-U extension header, which needs to be reflected onto DL SDAP header accordingly.

On the other hand, flow-to-DRB remapping is completely decided by CU-CP, for which CU-UP is oblivious at the time when DL SDT data arrives, and hence it has no idea whether it needs to account for DL SDAP header size for data volume calculation when notifying DL data arrival to CU-CP over E1.

Other than reflective QoS, DL SDAP header is not included. Given that reflective QoS is not expected to be frequently executed in most of the cases, DL SDAP header would be rarely used. And it is just 1 byte anyway, so it won’t make any critical difference.

As a result, we think that it is better not to take into account SDAP header when CU-UP calculates data volume for DL SDT data arrival. CU-UP just needs to consider the size of T-PDUs (in the received G-PDUs over NG-U).

Proposal 3: For DL data volume calculation in CU-UP, calculate based on IP packet sizes (i.e. T-PDUs included in the received G-PDUs over NG-U for DL SDT data). No need to take DL SDAP header into account given that it is just 1 byte and not commonly used (only used for NAS/AS reflective QoS).

# Conclusion

Based on the above discussions and analysis in the present contribution, we propose the followings:

Proposal 1: Agree that E1AP DL Data Notification shall be sent without MT-SDT info, if new DL data arrives through non-SDT bearer.

Proposal 2: The MT-SDT Data Size IE in E1AP/XnAP BL CRs are already defined as an extensible INTEGER, and thus it can convey any value beyond 96000 at no problem. No need to discuss the oversize issue nor any additional indicator..

Proposal 3: For DL data volume calculation in CU-UP, calculate based on IP packet sizes (i.e. T-PDUs included in the received G-PDUs over NG-U for DL SDT data). No need to take DL SDAP header into account given that it is just 1 byte and not commonly used (only used for NAS/AS reflective QoS).

# Reference

[1] R3-233330, “Summary of Offline Discussion on CB: SDT_MTSDT”, ZTE

[2] R3-233294, “Discussion on Support of MT-SDT in Split Architecture”, China Telecom

[3] TS 37.324, E-UTRA and NR; Service Data Adaptation Protocol (SDAP) specification (Release 17)

[4] TS 38.415, NG-RAN; PDU Session User Plane Protocol (Release 17)