3GPP TSG-RAN WG3 Meeting #121	R3-234353

Toulouse, France, August 21st – 25th, 2023

Agenda item:	9.2

Source: 	LG Electronics Inc.

Title: 	Correction on wrong abnormal description for conditional mobility in F1AP UE Context Modification procedure

Document for:	Discussion and Decision

# Introduction

In the F1AP UE Context Modification procedure, there is one abnormal description related to modification of conditional mobility based on an IE that has never been used in the UE CTXT MOD REQ message, and thus proposed to be corrected.

# Discussion

In the F1AP UE Context Modification procedure, there is one abnormal condition related to modification of conditional mobility (highlighted below):

#### 8.3.4.4 Abnormal Conditions #### //////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////// If one or more candidate cells in the Candidate Cells To Be Cancelled List IE included in the UE CONTEXT MODIFICATION REQUEST message were not prepared using the same UE-associated signaling connection, the gNB-DU shall ignore those non-associated candidate cells. In case of “CHO-replace” when the Target gNB-DU UE F1AP ID IE is included, if the candidate cell in the SpCell ID IE included in the UE CONTEXT MODIFICATION REQUEST message was not prepared using the same UE-associated signalling connection, the gNB-DU shall ignore this candidate cell.

The same appears in the UE Context Setup procedure:

#### 8.3.1.4 Abnormal Conditions #### //////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////// In case of “CHO-replace” when the Target gNB-DU UE F1AP ID IE is included, if the candidate cell in the SpCell ID IE included in the UE CONTEXT SETUP REQUEST message was not prepared using the same UE-associated signaling connection, the gNB-DU shall ignore this candidate cell.

However, searching the related Target gNB-DU UE F1AP ID IE, this is only used in UE CTXT SETUP REQ message and never used in UE CTXT MOD REQ message.

In fact, the usage of this IE is correct for the UE Context Setup procedure for conditional mobility (CHO/CPAC). For inter-DU case, the parallel UE Context Setup procedures haven been used to set up and modify candidate cells configurations in the target DU, each for each candidate cell (indicated by SpCell ID IE). Given that DU UE F1AP ID has not been assigned yet by the DU when initiated, it is possible that multiple UE-associated signallings could be established for the same UE when setting up candidate cells configurations, where for modification (i.e. “replace”), CU UE F1AP ID and candidate cell OD in the UE CTXT SETUP REQ message are not enough to uniquely identify the same UE. This is why, for the case of “replace”, DU UE F1AP ID has been additionally indicated from CU.

On the other hand, for candidate cell preparation within the source DU, the UE-associated signalling has been already established between CU and DU. Unless we are mistaken, CU/DU UE F1AP IDs are fixed in parallel UE Context Modification procedures, which handles conditional mobility initiation/replace/cancellation triggered by CU. As a result, for the case of “replace”, there was no need to indicate DU UE F1AP ID from CU.

Checking history, we observe that this abnormal description in the UE Context Modification procedure came from Rel-18 CHO F1AP BL CR [1], and has been in place from F1AP v16.2.0. It is highly likely that this condition was neglected to be cleaned up at that time. We thus propose to remove and correct from Rel-16 F1AP specs.

Proposal 1: Remove the “CHO-replace” abnormal description in the UE Context Modification procedure, where the said Target gNB-DU UE F1AP ID IE has never been used in the UE CTXT MOD REQ message for the modification of conditional mobility.

# Conclusion

Based on the above discussions and analysis, we propose the following:

Proposal 1: Remove the “CHO-replace” abnormal description in the UE Context Modification procedure, where the said Target gNB-DU UE F1AP ID IE has never been used in the UE CTXT MOD REQ message for the modification of conditional mobility.

The corresponding correction CRs for TS 38.473 v16.14.0 and v17.5.0 can be found in [2] and [3], respectively.

# Reference

[1] R3-204474, “Baseline CR for introducing Rel-16 NR mobility enhancement”, Ericsson, CATT, Huawei

[2] R3-234354, “Rel-16 Correction in the UE Context Modification procedure abnormal description for conditional mobility modification”, LG Electronics Inc.

[3] R3-234355, “Rel-17 Correction in the UE Context Modification procedure abnormal description for conditional mobility modification”, LG Electronics Inc.