3GPP TSG-RAN WG3 #121                                                                                 R3-234358

Toulouse, France, 21 – 25 August, 2023

Agenda Item:	17.2

Source:	CATT, OPPO

Title:	Consideration on support of common signalling in (C)HO

Document for:	Decision

1Introduction

For mobility enhancement in Rel-18 NR NTN, in order to reduce signalling overhead of handover command which is sent using dedicated (per UE) signalling, RAN2 has discussed the solution for source cell to broadcast in advance servingCellConfigCommon from the target cell (as common (C)HO signalling). In this way, the target´s cell servingCellConfigCommon does not need to be included in each RRCReconfiguration message.

During RAN2’s discussion, it was indicated this might have RAN3 impact, e.g., for the inter-gNB handover case, the target cell’s servingCellConfigCommon might need to be transferred to the source cell using different means than what’s available in the legacy specification. In the LS [1], RAN2 ask RAN3 to answer the following question:

- In case target cell’s servingCellConfigCommon needs to be broadcasted in the source cell, whether it is feasible that the target cell’s servingCellConfigCommon is provided independently to the source cell for the purpose of inter-gNB handover in Rel-18 with or without specification changes?

In this contribution, we will discuss the whether and how to provide the servingCellConfigCommon of the target cell to the source cell from RAN3 perspective.  Base on the discussion, we provide the draft reply LS accordingly.

2. Discussion

From the LS [1], RAN2 has discussed the solution for source cell to broadcast in advance servingCellConfigCommon from the target cell (as common (C)HO signalling). In this way, the target´s cell servingCellConfigCommon does not need to be included in each RRCReconfiguration message, which could reduce signalling overhead of handover command.

Observation 1: It’s beneficial to support common HO signalling to reduce the signalling overhead of handover command.

From RAN3 perspective, to make support of the common (C)HO, the key issue is how the source gNB can get the neighbour cell’s servingCellConfigCommon from the target gNB, some potential solutions are provided below:

- Solution 1: Pure OAM based solution.

- Solution 2: Signalling based solution

- Solution 3: Implementation based solution

For the solution 1, the neighbour cell’s servingCellConfigCommon is configured via OAM. In this case the source NG-RAN node could broadcast neighbour cell’s common configuration in the SIB, the UEs could obtain the common configuration which could be used for the handover towards the neighbour cell. The target NG-RAN node does not need to include the servingCellConfigCommon in the HandoverCommand message, which could reduce the signalling overhead as expected.

In Rel-17, as specified in TS 38.300, in SIB19, a cell could broadcast serving cell’s ephemeris info, it could also broadcast neighbour cell’s NTN Payload ephemeris data. Currently, the neighbour cell’s ephemeris data could only be configured to the source NG-RAN node via OAM.

The NTN ephemeris is provided in SIB19. It includes serving cell's NTN payload ephemeris and optionally neighbouring cell's NTN payload ephemeris.

Observation 2: In Rel-17, Neighbour cell’s NTN payload ephemeris could be configured to the source NG-RAN node.

In RAN3, we discussed a lot on how to exchange the NTN neighbour relations between the gNBs, and it’s finally agreed to go for OAM based solution if the neighbour relations are changeable with time, e.g. in earth moving cell case.

Observation 3: For NTN, the neighbour relations between the gNBs are relied on OAM if the neighbour relations are changeable with time.

Based on the above observations, we understand the OAM based solutions could also be applied to configure the neighbour cell’s common figuration. And this could be done together when configure the neighbour relations.

Observation 4: OAM based solution (solution 1) is feasible to configure neighbour cell’s servingCellConfigCommon information.

For the signalling based solution (solution 2), gNBs exchange the servingCellConfigCommon per cell for the cells which have neighbour relations. Then we should consider non-UE associated signalling or UE associated signalling should be used.

As this is cell specific information, using non-UE procedure seems ok, but as mentioned in the O3 above, when the neighbour relations are changeable in NTN, the neighbour relations configuration is relied on OAM. Thus, reusing the existing non-UE associated procedures, i.e. Xn/NG Setup, Configuration Update is not always feasible. And we understand it’s not economic to define new non-UE associated signalling(s) to transfer the neighbour cell’s servingCellConfigCommon.

Observation 5: For signalling based solution (solution 2), reuse the existing non-UE procedures, e.g. Xn/NG Setup, Configuration Update may not be feasible in case of earth moving cell case, where the neighbour relations change with time. Define the new non-UE signalling(s) is not an economic way.

Considering the UE-associated signalling, the straightforward way is to transfer the servingCellConfigCommon via the existing Xn/NG handover signallings, e.g. add a separate IE in XnAP Handover Request ACK, or NGAP Target NG-RAN Node to Source NG-RAN Node Transparent Container.

Observation 6: For signalling based solution (solution 2), it’s possible to explicitly transfer the servingCellConfigCommon in Xn/NG handover signalling.

As the servingCellConfigCommon is already included in the RRC Container of the XnAP Handover Request ACK, and the NGAP Target NG-RAN Node to Source NG-RAN Node Transparent Container, it’s also possible for the source NG-RAN node to obtain the servingCellConfigCommon info by implementation, i.e. decoding the RRC Container.

Observation 7: For implementation based solution (solution 3), it is feasible for the source gNB to obtain the target cell’s servingCellConfigCommon by decoding the RRC Container generated by the target gNB.

Base on the discussion above, all the 3 solutions as above are feasible. There’s no specification impact to support the solution 1, however, for the solution 2 and solution 3, we still need to consider how to notify the target NG-RAN node to include the servingCellConfigCommon in the RRC Container (HO Command), and how to coordinate between gNBs when the servingCellConfigCommon is updated in the target cell. More details need to be discussed.

Observation 8: For signalling based solution (solution 2) and implementation based solution (solution 3), more details need to be considered on how to notify the target gNB whether to include servingCellConfigCommon in the RRC Container, and how to coordinate between the gNBs when the servingCellConfigCommon is updated.

Compared the solutions above, OAM based solution is preferred to avoid/minimize the RAN3 impact, and we would like to reply the LS to RAN2, the draft LS is attached.

Proposal 1: From RAN3 perspective, OAM based solution is preferred to support the common signalling in (C)HO.

Proposal 2: Reply the LS to RAN2, draft LS in annex could be taken as start point.

3. Conclusion

In this contribution, we discussed whether and how to provide the target cell’s servingCellConfigCommon to the source NG-RAN node for the purpose of inter gNB handover. Based on the discussion, we provided the following observations and proposals:

Observation 1: It’s beneficial to support common HO signalling to reduce the signalling overhead of handover command.

Observation 2: In Rel-17, Neighbour cell’s NTN payload ephemeris could be configured to the source NG-RAN node.

Observation 3: For NTN, the neighbour relations between the gNBs are relied on OAM if the neighbour relations are changeable with time.

Observation 4: OAM based solution (solution 1) is feasible to configure neighbour cell’s servingCellConfigCommon information.

Observation 5: For signalling based solution (solution 2), reuse the existing non-UE procedures, e.g. Xn/NG Setup, Configuration Update may not be feasible in case of earth moving cell case, where the neighbour relations change with time. Define the new non-UE signalling(s) is not an economic way.

Observation 6: For signalling based solution (solution 2), it’s possible to explicitly transfer the servingCellConfigCommon in Xn/NG handover signalling.

Observation 7: For implementation based solution (solution 3), it is feasible for the source gNB to obtain the target cell’s servingCellConfigCommon by decoding the RRC Container generated by the target gNB.

Observation 8: For signalling based solution (solution 2) and implementation based solution (solution 3), more details need to be considered on how to notify the target gNB whether to include servingCellConfigCommon in the RRC Container, and how to coordinate between the gNBs when the servingCellConfigCommon is updated.

Proposal 1: From RAN3 perspective, OAM based solution is preferred to support the common signalling in (C)HO.

Proposal 2: Reply the LS to RAN2, draft LS in annex could be taken as start point.

4. References

- R3-233721 LS on common signalling in (C)HO, From: RAN2 (OPPO)

5. Draft Reply LS to RAN2

Title:	[Draft] LS Response on common signalling in (C)HO

Release:	Rel-18

Work Item:	NR_NTN_enh

Source:	CATT, OPPO (to be RAN3)

To:	RAN WG2

Cc:

Contact Person:

#### Name:	Jiancheng Sun

#### E-mail Address:	sunjiancheng@catt.cn

Attachments:	n/a

1. Overall Description:

Thanks RAN2 for the LS on support of common signalling in (C)HO. RAN3 discussed the question raised by RAN2 and would provide the answer below:

From RAN3 perspective, it’s feasible to provide the target cell’s servingCellConfigCommon to the source cell for purpose of inter-gNB handover. RAN3 assumes the OAM based solution, signalling based solution and implementation based solution are all feasible. However, to avoid the specification impact, RAN3 prefers to go for OAM based solution, i.e. the target cell’s servingCellConfigCommon could be configured to the source NG-RAN node via OAM.

2. Actions:

To RAN2:

ACTION: RAN3 respectfully requests RAN2 to take the above response into consideration in their future work.

3. Date of Next TSG-RAN WG3 Meetings:

RAN3#121bis	   		9 - 13 October 			2023			Xiamen, China

RAN3#122	       		13-17 November 	2023			Chicago, US