3GPP TSG-RAN WG3 Meeting #121	R3-234376

# Toulouse, France, 21st – 25th August, 2023

Agenda item:		12.2.1

Source:	ZTE

Title:	(TP to 38.423 & 38.300) Further discussion on Timing Information

Document for:		Others

Introduction

Following are the agreement and left issues on timing information in the last meeting:

The requested prediction time includes the specific point in time for which the prediction information is being requested with the assumption that this specific point in time is in the reasonable future. The encoding needs to be further discussed. FFS on the encoding and exact values of the specific time point. Down-select the options: - INTEGER TYPE - ENUMERATED TYPE FFS on the benefits of the predicted information over a period of time. The specific point in time for which the prediction information is being requested with the assumption that this specific point in time is in the reasonable future, can be configured for both one-time reporting and periodic reporting.

It was agreed that the requested prediction time can be configured for both one-time reporting and periodic reporting in the agreed new request message, and the requested prediction time includes the specific point in time for which the prediction information is being requested with the assumption that this specific point in time is in the reasonable future. The encoding of that IE needs to be further discussed.

Besides, the meaning of the other information, e.g., time duration, validity time, accuracy should also be understood and consider whether it should be conveyed in the procedure.

In this contribution, we provide our views on the encoding format and other information on timing, along with the corresponding TP.

Discussion

Requested Prediction Time

Encoding Format

In the last meeting, following options are considered for the encoding of the requested prediction time:

- Option 1: INTERGER TYPE

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Requested prediction time |  |  | INTEGER (0..4095, ...) | Unit: seconds |

- Option 2: ENUMERATED TYPE

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Requested prediction time |  |  | ENUMERATED(0s, 5s, 10s, 20s, 40s, 80s, 160s, 320, 640, …) | Unit: seconds |

- It has been acknowledged that the requested prediction time refers to a specific point in the reasonable future, which implies that the time range for the requested prediction should not be too long. As a result, the relative time type is excluded. While both options above can work, option 1 encompasses option 2 and offers greater flexibility in implementation. AI/ML models can generate predicted information based on the specified requested prediction time. On the other hand, option 2 restricts the predicted values, allowing the requesting node to choose only from these discrete values.

- Option 1 covers Option 2 and offers greater flexibility in implementation.

- Using Option 1 INTERGER TYPE to encode the requested prediction time.

Other Timing Information

One-time Reporting:

- Whether the requested timing information should involve the time duration in the request message, was also discussed in the offline discussion. The requested node generates the average information according to the introduced time duration, but several companies do not catch the benefits of the average value. Here, we give our understanding why predicted time duration is necessary. The Fig.1 shows the requested timing information with time duration in the case of one-time reporting.

- Figure 1. The requested timing information with time duration

- Firstly, from AI/ML perspective, AI/ML model can be trained to make predictions over a period of time. These predictions can be in form of average information, where the model estimates the average value. For example, for the temperature prediction, an AI/ML model trained with historical data can predict the average temperature in next day, or in a next month. From a predictive standpoint, predicted information not only represents a single prediction at a specific time but also represents the average prediction over a period of time.

- The AI/ML model is able to generate the average information over a period of time.

- Predicted information not only represents a single prediction at a specific time but also represents the average prediction over a period of time.

- Then, in certain scenarios, a single predicted value at a specific point in time might not fully represent the complete prediction status. For instance, at future time 10 seconds, the predicted value is 20, but at future time 15 seconds, the value drastically increases to 100. Such variations can have a significant impact on decision-making processes. To address this issue, the requesting node considers the average prediction rather than relying solely on the predicted value at a single point in time. This approach allows for a more comprehensive assessment of the prediction's overall trend, especially when dealing with fluctuating data. The average predicted information reflects the status/data distribution over a period of time, and then the requested node can generate the appropriate decision based on the average predicted information (e.g., predicted resource status).

- The average predicted information reflects the status/data distribution over a period of time, and then the requested node can generate the appropriate decision based on the average predicted information (e.g., predicted resource status).

For one-time reporting, the requested prediction time specified in the AI/ML INFORMATION REQUEST message can include both the time instant and the duration of time. Hence, the definition of the requested prediction time is clarified as follows:

-	Start time: The specific point in time for which the prediction information is being requested. (Agreed)

- 	Time duration: The length of time over which the predicted information is being requested.

Note: The time duration must be configured together with the time instant; otherwise, the configuration is considered invalid

- In the case of one-time reporting, the requested prediction time in the AI/ML INFORMATION REQUEST message involves the start time and time duration.

- Extend the definition of the requested timing information as follows:

-	Requested prediction time: The specific point in time for which the prediction information is being requested (Agreed).

- 	Time duration: The length of time window over which the predicted information is being requested.

Note: The time duration must be configured together with the Requested prediction time; otherwise, the configuration is considered invalid.

Periodic Reporting:

- In terms of periodic reporting, it has been agreed that the requested prediction time can be configured for both one-time and periodic reporting. In this context, we present our understanding of how the proposed requested prediction time works in conjunction with the periodicity. Firstly, Figure 2 illustrates that the requested prediction time only encompasses the specific time instant as previously agreed upon.

- - Scenario 1: The requested prediction time only includes the time instant.

Figure 2. The timing information in the request message involves the time instant without the time duration, in the case of periodic reporting

- Regarding scenario 1, within the realm of periodic reporting, once the requesting node receives the requested prediction time and periodicity values, it will include predicted information for {Trequested prediction time + N * Tperiodicity, where N = 0, 1, 2, 3, 4, 5, ...} in each of the report messages.

- In the case periodic reporting, with the configured periodicity and requested prediction time, the predicted information in the each report message is for {Trequested prediction time + N * Tperiodicity, where N = 0, 1, 2, 3, 4, 5, ...}.

- For periodic reporting, if the requested prediction time is configured with a periodicity, the requested node should report the predicted information every time interval specified by the periodicity.

- - Scenario 2: The requested prediction time includes the time instant and time duration.

- We provide provides an example of how periodic reporting works with the AI/ML INFORMATION REQUEST message, using the requested prediction time instant, duration of time, and periodicity, as shown in the Figure 4. In this example, the requesting node sets the requested prediction time instant to 10s with a duration of time of 20s and a periodicity of 5s.

- The requested node generates the predicted information at every 5-second interval, starting from 5s, and sends it to the requesting node. The transmission continues until the duration of time is exceeded (in this case, after 5 reports have been sent), at which point the transmission stops.

Figure 3. The timing information in the request message involves the time instant with the time duration, in the case of periodic reporting

- The duration of time is specified with time instant in the request message for determining when the transmission should stop in periodic reporting.

- In the case of periodic reporting, from our understanding, there is no need to report the average information over a period of time since predicted information, every time interval specified by the periodicity, can completely represent the data distribution.

- The requested prediction time and time duration can be configured for both one-time reporting and periodic reporting.

The time information in the request message can not only be used to configure the prediction time but can also be used to configure other data such as measured EC or UE performance feedback. The requested base station can periodically or once report the requested information within the time range specified in the message.

- RAN3 is kindly asked to discuss whether the timing information can be used for other type information e.g. UE performance feedback.

- Possible Implementation to 38.423:

    - 9.1.3.CC	AI/ML INFORMATION REQUEST (FFS on the name)

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested AI/ML related information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| **************** skip unchanged part ******************* |  |  |  |  |  |  |
| Requested Prediction Time | O |  | INTEGER (0..4095, ...) | If Periodicity IE is present, the predicted information in the each report message is for {Trequested prediction time + N * Tperiodicity, where N = 0, 1, 2, 3, 4, 5, ...}. Unit: seconds | YES | ignore |
| Requested Time Duration | O |  | INTEGER (0..90060, …) | Unit: seconds | YES | ignore |

The corresponding TP to 38.423 is provided in the Annex-A.

Validity time

- Several companies think the validity time refers to the time period during which each requested prediction information is considered valid. In other words, it is the time period for which each prediction is relevant and accurate. On the other hand, time duration refers to the overall time range for which prediction information is requested. Figure 5 as an example for clearly illustrated.

Figure 5. The timing information in the request message involves the time instant, the time duration and validity time, in the case of periodic reporting

- Here, the figure 5 show the potential scenario, that if the requesting node configures the time instant, time duration and validity time with periodicity in the request message, the requested node will generate the predicted information within 5 seconds at every 10-second interval, starting from 09:00:20.

- However, we don’t see benefits of indicating the validity time in the request message here. When the requesting node receives the report message, it already knows that the predicted information contained in the message was predicted based on the requested prediction time configured in the request message. The requesting node can also determine the validity of the predicted message based on the actual time.

- In other words, indicating the validity time in the request message may not add any significant benefit in the context of periodic reporting. This is because periodic reporting is a continuous process that involves generating and transmitting predicted information at regular intervals. The requesting node is aware of the requested prediction time and periodicity configured in the request message. Therefore, it can infer the validity time of each report message based on the actual time and the configured periodicity.

- The requesting node is aware of the requested prediction time and periodicity configured in the request message, so it can infer the validity time of each report message based on the actual time and the configured periodicity.

- For the periodic reporting, it is not necessary to indicate the validity time in the request message.

- For the one-time reporting, it seems the validity time has the same meaning of the time duration. Hence, it is also not necessary to be indicated in the request message. The predicted information keep valid until the requested prediction time.

- For the one-time reporting, it is also not necessary to indicate the validity time in the request message, because the time duration has the same meaning as validity time.

Stage2 impact

Currently, we see that the new agreed AI/ML INFORMATION REQUEST message can involve the predicted information and other measurements. The performance feedback and predicted information could be involved in the agreed new procedure.

Furthermore, based on the TR37.817 [1], to support the agreed use cases, the solutions involve the same signaling flow when AI/ML training and AI/ML inference are in the NG-RAN nodes. For clear illustration of the usage of AI/ML information procedure, following figure could be captured in the 38.300.

Figure 1. The procedure flow for AI/ML function between NG-RAN nodes

- Capture the figure above in TS38.300 that the new agreed procedure could be used to request the predicted information or measurement information.

Conclusion

We propose the following observations and proposals:

- Option 1 covers Option 2 and offers greater flexibility in implementation.

- Using Option 1 INTERGER TYPE to encode the requested prediction time.

- The AI/ML model is able to generate the average information over a period of time.

- Predicted information not only represents a single prediction at a specific time but also represents the average prediction over a period of time.

- The average predicted information reflects the status/data distribution over a period of time, and then the requested node can generate the appropriate decision based on the average predicted information (e.g., predicted resource status).

- In the case of one-time reporting, the requested prediction time in the AI/ML INFORMATION REQUEST message involves the start time and time duration.

- Extend the definition of the requested timing information as follows:

-	Requested prediction time: The specific point in time for which the prediction information is being requested (Agreed).

- 	 Time duration: The length of time window over which the predicted information is being requested.

Note: The time duration must be configured together with the Requested prediction time; otherwise, the configuration is considered invalid.

- In the case periodic reporting, with the configured periodicity and requested prediction time, the predicted information in the each report message is for {Trequested prediction time + N * Tperiodicity, where N = 0, 1, 2, 3, 4, 5, ...}.

- For periodic reporting, if the requested prediction time is configured with a periodicity, the requested node should report the predicted information every time interval specified by the periodicity.

- The duration of time is specified with time instant in the request message for determining when the transmission should stop in periodic reporting.

- The requested prediction time and time duration can be configured for both one-time reporting and periodic reporting.

- RAN3 is kindly asked to discuss whether the timing information can be used for other type information e.g., measured EC or UE performance feedback.

- The requesting node is aware of the requested prediction time and periodicity configured in the request message, so it can infer the validity time of each report message based on the actual time and the configured periodicity.

- For the periodic reporting, it is not necessary to indicate the validity time in the request message.

- For the one-time reporting, it is also not necessary to indicate the validity time in the request message, because the time duration has the same meaning as validity time.

- Capture the figure above in TS38.300 that the new agreed procedure could be used to request the predicted information or measurement information.

Annex-A (TP to BLCR for 38.423)

**************** Start of Change ****************

    - 8.4.AA	AI/ML Information Reporting Initiation (FFS on the name)

    - 8.4.AA.1	General

This procedure is used by an NG-RAN node to request the reporting of AI/ML related information to another NG-RAN node.

The procedure uses non UE-associated signalling.

Editor’s Note: FFS other information that can be requested using this procedure.

Editor’s Note: FFS content of AL/ML related information.

    - 8.4.AA.2	Successful Operation

Figure 8.4.AA.2-1: AI/ML Information Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the AI/ML INFORMATION REQUEST message to NG-RAN node2 to start AI/ML related information reporting and stop AI/ML related information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested AI/ML related information reporting according to the parameters given in the request in case the Registration Request IE is set to “start”; or

-	shall stop all cells AI/ML related information reporting and terminate the reporting in case the Registration Request IE is set to “stop”; or

-	FFS

If the Registration Request IE is set to “start” in the AI/ML INFORMATION REQUEST message and the Report Characteristics IE indicates cell specific AI/ML related information reporting, the Cell To Report List IE shall be included.

If NG-RAN node2 is capable to provide all of the requested information, it shall initiate the AI/ML related information reporting as requested by NG-RAN node1 and respond with the AI/ML INFORMATION RESPONSE message.

If NG-RAN node2 is capable to provide some but not all of the requested information, it shall initiate the AI/ML related information reporting for the admitted requested information and include the Node Measurement Initiation Result IE or the Per Cell Measurement Initiation Result IE or both in the AI/ML INFORMATION RESPONSE message.

If the Reporting Periodicity IE in the AI/ML INFORMATION REQUEST is present, this indicates the periodicity for the reporting of periodic AI/ML related information. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

If the Requested Prediction Time IE in the AI/ML INFORMATION REQUEST message is present, it indicates the specific point in time to which the prediction of the information requested applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

If the Requested Time Duration IE in the AI/ML INFORMATION REQUEST message is present, it indicates the time duration to which the prediction of the information requested applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the AI/ML INFORMATION REQUEST indicates the type of objects NG-RAN node2 shall perform measurements or prediction on. NG-RAN node2 shall include in the AI/ML INFORMATION UPDATE message:

-	the Predicted Radio Resource Status IE, if the first bit, “Predicted Radio Resource Status” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”. FFS on the details of Predicted Radio Resource Status IE.

-	the Predicted Number of Active UEs IE, if the second bit, “Predicted Number of Active UEs” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”;

-	the Predicted RRC Connections IE, if the third bit, “Predicted RRC Connections” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

-	the Average UE Throughput DL IE, if the fourth  bit, “Average UE Throughput DL” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

-	the Average UE Throughput UL IE, if the fifth bit, “Average UE Throughput UL” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

-	the Average Packet Delay IE, if the sixth bit, “Average Packet Delay” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

-	the Average Packet Loss IE, if the seventh bit, “Average Packet Loss” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

****************Next Change ****************

    - 9.1.3.CC	AI/ML INFORMATION REQUEST (FFS on the name)

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested AI/ML related information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) (FFS on others) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss Eighth Bit = Energy Cost FFS on the coding | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | INTEGER (0..4095, ...) | If Periodicity IE is present, the predicted information in the each report message is for {Trequested prediction time + N * Tperiodicity, where N = 0, 1, 2, 3, 4, 5, ...}. Unit: seconds | YES | ignore |
| Requested Time Duration | O |  | INTEGER (0..90060, …) | Unit: seconds | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

**************** End of Change ****************

Annex-B (TP to BLCR for 38.300)

**************** Start of Change ****************

- X.X AI/ML for NG-RAN

- X.X.1 General

AI/ML for NG-RAN, as a RAN internal function, is achieved by using Artificial Intelligence (AI) and Machine Learning (ML) techniques.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

- X.X.2 Mechanisms and Principles

The AI/ML function requires inputs from neighbour NG-RAN nodes (e.g. predicted information, feedback information, measurements) and/or UEs (e.g. measurement results), in support to AI/ML processes such as AI/ML Model Inference and AI/ML Model Training.

AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.

For the deployments of RAN intelligence, following scenarios may be supported:

•	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

•	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

For the AI/ML use cases, the unified AI/ML related procedure is used to support to transfer predicted information or other measurements, as shown in Figure X.

Figure X. The procedure flow for AI/ML function between NG-RAN nodes

**************** End of Change ****************