3GPP TSG-RAN WG3 Meeting #121															 R3-234378

# Toulouse, France, 21st – 25th August, 2023

Agenda item:		12.2.2.2

Source:	ZTE

Title:	Discussion on left issues of AI based mobility optimization

Document for:		Others

Introduction

In the last meeting, we have agreed that the measured UE trajectory, which is limited to the next one hop target NG-RAN node, transferred from the target NG-RAN node to the source NG-RAN node after the concerned UE(s) handover.

In Rel18, the UE trajectory prediction is only limited to the next one hop target NGRAN node, and the source NGRAN node can collect measured UE trajectory (in the form of UE History Information) related to one or multiple cells within the single next hop target NGRAN node after handing over the concerned UE(s).

Take the procedure for UE performance feedback report as the baseline to also support the measured UE trajectory collection by the source gNB.

The details on how to request the measured UE trajectory collection needs to be further discussed.

In this contribution, we provide our understanding on how to design the signaling to support the transmission of the measured UE trajectory over Xn interface. Moreover, we also provide the corresponding TP in the Annex.

Discussion

Measured UE trajectory

During the previous meeting, it was acknowledged that the UE trajectory prediction is only limited to the next hop target NG-RAN node. Based on this assumption, the source NG-RAN node can collect the measured UE trajectory related to one or multiple cells within the single next hop target NG-RAN node after handing over the concerned UE(s).

- Focus the scenario that the actual UE trajectory feedback information is limited in the first target NG-RAN node in Rel-18.

Another issue discussed left was whether the predicted trajectory of the UE would span across multiple NG-RAN nodes. If the predicted UE trajectory spans across multiple NG-RAN nodes, it would mean that the actual UE trajectory information may need to be transferred from the final NG-RAN node to the source NG-RAN node, as depicted in Figure 1.

Figure 1. Examples of predicted UE trajectory across multiple NG-RAN nodes

From the figure, the actual UE trajectory should include the trajectory from gNB 2 to gNB 6. The issue here is that the UE context between gNB 1 and gNB 6 is not established, so it is hard to collect the actual UE trajectory by gNB 1 via Xn interface. The solution may involved the Uu and NG interface. In this release, we suggest to focus the scenario that the actual UE trajectory feedback information is limited in the first target NG-RAN node.

- Discuss the issue on whether the predicted trajectory of the UE spans across multiple NG-RAN nodes and how to support collection of the measured UE trajectory from multiple hop target NG-RAN nodes in the Rel19.

When UE trajectory feedback information is limited in the first target NG-RAN node, maintaining UE context between the source and target NG-RAN nodes after a UE handover becomes challenging. From a technical perspective, this challenge can be overcome by gNB implementation. Specifically, the target NG-RAN node can begin collecting UE trajectory information after a successful handover, provided that the source NG-RAN node had requested the trajectory information before the handover.

- It is up to gNB implementation to keep the UE context between the source NG-RAN node and the target NG-RAN node, if the UE trajectory feedback is requested from the source NG-RAN node.

It has been agreed that the agreed class1 procedure (AI/ML INFORMATION REQUEST/RESPONSE, the name needs further discussion) is used to configure UE performance feedback reporting. According to the agreement in the last meeting “Take the procedure for UE performance feedback report as the baseline to also support the measured UE trajectory collection by the source gNB. ”, the same principle can be applied to UE trajectory feedback as well.

- The agreed class1 procedure (AI/ML INFORMATION REQUEST/RESPONSE, the name needs further discussion) is used to configure UE trajectory reporting.

To do this, the source NG-RAN node generates a predicted UE trajectory and initiates a new request message to the potential target NG-RAN node to configure the measured UE trajectory. Then the predicted UE trajectory is transferred to the target NG-RAN node via a HANDOVER REQUEST message. The target NG-RAN node can understand that the UE trajectory of the handover-ed UE should be collected by the Measurement ID in the Handover Request message, which is configured in the request message before. Then the target NG-RAN node then begins collecting subsequent UE trajectory information after the successful handover, and report the cell-level trajectory to the source NG-RAN node via AI/ML INFORMATION UPDATE message.

- Introduce the UE trajectory IE in the agreed class2 AI/ML INFORMATION UPDATE message.

- Introduce the UE trajectory bit in the Request Characteristic IE in the AI/ML INFORMATION REQUEST message.

Possible Implementation

Following is the possible implementation to the actual UE trajectory IE.

    - 9.1.3.FF	AI/ML INFORMATION UPDATE (FFS on the name)

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested AI/ML related information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell AI/ML Info Result (FFS on the name) |  | 0..1 |  |  | YES | ignore |
| >Cell AI/ML Info Result Item(FFS on the name) |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  |  |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| >>Predicted Number of Active UEs | O |  | 9.2.2.62 |  | –- |  |
| >>Predicted RRC Connections | O |  | 9.2.2.56 |  | – |  |
| UE Associated Info Result |  | 0..1 |  |  |  |  |
| > UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  |  |  |
| >> UE Assistant Identifier (FFS on the name) | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1 |  |  |
| >>UE Performance | O |  | 9.2.3.Y |  |  |  |
| >>UE trajectory | O |  | FFS 9.2.3.P1 |  |  |  |
| Energy Cost | O |  | FFS | It represents the actual measurement of the Energy Cost (FFS) |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEReports | Maximum no. UE that can be served by a NG-RAN node. Value is FFS. |

    - 9.2.3.P1	UE Trajectory

The UE Trajectory Information IE contains the list of NG-RAN cells where the UE has visited after being handed over to the target gNB.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UE Trajectory List |  | 1..<maxnoofCellsTrajectory> |  | List of cells where the UE connected, in chronological order. |
| >Trajectory Cell Information | M |  | 9.2.3.P2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsTrajectory | Maximum number of cells that can be reported for UE trajectory. Value is FFS. |

    - 9.2.3.P2	Trajectory Cell Information

The Trajectory Cell Information contains the cell ID of the NG-RAN cell that a UE has visited after being handed over to the target gNB..

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Trajectory Cell Information |  |  |  |  |
| >NG-RAN Cell |  |  |  |  |
| >>Global NG-RAN Cell Identity | M |  | 9.2.2.27 |  |
| >>Time UE Stays in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, or set of NG-RAN cells in seconds. If the duration is more than 4095s, this IE is set to 4095. |

The corresponding TP is provided in the Annex.

- Agree the TP provided in the Annex.

Conclusion

We propose the following observations and proposals:

- Focus the scenario that the actual UE trajectory feedback information is limited in the first target NG-RAN node in Rel-18.

- Discuss the issue on whether the predicted trajectory of the UE spans across multiple NG-RAN nodes and how to support collection of the measured UE trajectory from multiple hop target NG-RAN nodes in the Rel19.

- It is up to gNB implementation to keep the UE context between the source NG-RAN node and the target NG-RAN node, if the UE trajectory feedback is requested from the source NG-RAN node.

- The agreed class1 procedure (AI/ML INFORMATION REQUEST/RESPONSE, the name needs further discussion) is used to configure UE trajectory reporting.

- Introduce the UE trajectory IE in the agreed class2 AI/ML INFORMATION UPDATE message.

- Introduce the UE trajectory bit in the Request Characteristic IE in the AI/ML INFORMATION REQUEST message.

- Agree the TP provided in the Annex.

- Annex - TP to TS38.423

- TP to 38.423 BLCR

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

<Unchanged Text Omitted>

    - 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. Note: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| >MBS Session Information List | O |  | 9.2.1.36 |  | YES | ignore |
| >5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| >UE Slice Maximum Bit Rate List | O |  | 9.2.3.167 |  | YES | ignore |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Cell Based UE Trajectory Prediction | O |  | 9.2.3.x | This IE contains cell based UE trajectory prediction within the cells of the target NG-RAN node. | YES | ignore |
| AI/ML Measurement ID (FFS on the name) | O |  | 9.2.3.M |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

<Unchanged Text Omitted>

    - 8.4.AA AI/ML Information Reporting Initiation (FFS on the name)

    - 8.4.AA.1	General

This procedure is used by an NG-RAN node to request the reporting of AI/ML related information to another NG-RAN node.

The procedure uses non UE-associated signalling.

Editor’s Note: FFS other information that can be requested using this procedure.

Editor’s Note: FFS content of AL/ML related information.

    - 8.4.AA.2	Successful Operation

Figure 8.4.AA.2-1: AI/ML Information Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the AI/ML INFORMATION REQUEST message to NG-RAN node2 to start AI/ML related information reporting and stop AI/ML related information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested AI/ML related information reporting according to the parameters given in the request in case the Registration Request IE is set to "start"; or

-	shall stop all cells AI/ML related information reporting and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	FFS

If the Registration Request IE is set to "start" in the AI/ML INFORMATION REQUEST message and the Report Characteristics IE indicates cell specific AI/ML related information reporting, the Cell To Report List IE shall be included.

If NG-RAN node2 is capable to provide all or part of (exact details of if and how to support partial reporting are FFS) requested information, it shall initiate the AI/ML related information reporting as requested by NG-RAN node1 and respond with the AI/ML INFORMATION RESPONSE message.

If the Reporting Periodicity IE in the AI/ML INFORMATION REQUEST is present, this indicates the periodicity for the reporting of periodic AI/ML related information. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the AI/ML INFORMATION REQUEST indicates the type of objects NG-RAN node2 shall perform measurements or prediction on. NG-RAN node2 shall include in the AI/ML INFORMATION UPDATE message:

- 	the Predicted Radio Resource Status IE, if the first bit, "Predicted Radio Resource Status" of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to "1". FFS on the details of Predicted Radio Resource Status IE.

- 	the Predicted Number of Active UEs IE, if the second bit, "Predicted Number of Active UEs" of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to "1";

-	the Predicted RRC Connections IE, if the third bit, "Predicted RRC Connections" of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to "1".

-    the Average UE Throughput DL IE, if the fourth  bit, "Average UE Throughput DL" of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to "1".

-    the Average UE Throughput UL IE, if the fifth bit, "Average UE Throughput UL" of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to "1".

-    the Average Packet Delay IE, if the sixth bit, "Average Packet Delay" of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to "1".

-    the Average Packet Loss IE, if the seventh bit, "Average Packet Loss" of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to "1".

-    the Energy Cost IE, if the eighth bit, "Energy Cost" of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to "1".

-    the UE Trajectory IE, if the ninth bit, " UE Trajectory" of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to "1".

<Unchanged Text Omitted>

    - 9.1.3.CC	AI/ML INFORMATION REQUEST (FFS on the name)

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested AI/ML related information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) (FFS on others) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss Eighth Bit = Energy Cost Ninth Bit = UE trajectory FFS on the coding | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | FFS |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

    - 9.1.3.DD	AI/ML INFORMATION RESPONSE (FFS on the name)

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested AI/ML related information, for all or part of the measurement objects included in the reporting is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Node Measurement Initiation Result |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | ignore |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | EACH | ignore |
| >>Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(128)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Energy Cost. Other bits shall be ignored by the NG-RAN node1. |  |  |
| >>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Per Cell Measurement Initiation Result |  | 0..1 |  | List of measurement objects that failed to be initiated per cell. | YES | ignore |
| >Per Cell Measurement Initiation Result Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | EACH | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the cell. | – |  |
| >>>Measurement Failure Cause Item |  | 1 .. <maxFailedMeasObjects> |  |  |  |  |
| >>>>Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(128)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss. Eigth Bit = UE trajectory Other bits shall be ignored by the NG-RAN node1. | – |  |
| >>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxFailedMeasObjects | Maximum number of measurement objects that can fail per measurement. Value is 124. |
| maxFailedMeasPerNode | Maximum number of measurement objects that can fail per node. Value is 124. |

    - 9.1.3.EE	AI/ML INFORMATION FAILURE (FFS on the name)

This message is sent by the NG-RAN node2 to NG-RAN node1 to indicate that for all of (exact details of if and how to support partial reporting are FFS) the requested objects the reporting cannot be initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

    - 9.1.3.FF	AI/ML INFORMATION UPDATE (FFS on the name)

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested AI/ML related information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell AI/ML Info Result (FFS on the name) |  | 0..1 |  |  | YES | ignore |
| >Cell AI/ML Info Result Item (FFS on the name) |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  |  |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| >>Predicted Number of Active UEs | O |  | 9.2.2.62 |  | –- |  |
| >>Predicted RRC Connections | O |  | 9.2.2.56 |  | – |  |
| UE Associated Info Result |  | 0..1 |  |  |  |  |
| > UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  |  |  |
| >> UE Assistant Identifier (FFS) | M |  | FFS |  |  |  |
| >> UE Performance | O |  | 9.2.3.Y |  |  |  |
| >> UE trajectory | O |  | 9.2.3.P1 |  |  |  |
| Energy Cost | O |  | FFS | It represents the actual measurement of the Energy Cost (FFS) |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEReports | Maximum no. UE that can be served by a NG-RAN node. Value is FFS. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

    - 9.2.3.P1	UE Trajectory Information

Editors Note: Actual name of this IE is FFS.

The UE Trajectory Information IE contains the list of NG-RAN cells where the UE has visited after being handed over to the target gNB.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UE Trajectory List |  | 1..<maxnoofCellsTrajectory> |  | List of cells where the UE connected, in chronological order. |
| >Trajectory Cell Information | M |  | 9.2.3.P2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsTrajectory | Maximum number of cells that can be reported for UE trajectory. Value is FFS. |

    - 9.2.3.P2	Trajectory Cell Information

The Trajectory Cell Information contains the cell ID of the NG-RAN cell that a UE has visited after being handed over to the target gNB..

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Trajectory Cell Information |  |  |  |  |
| >NG-RAN Cell |  |  |  |  |
| >>Global NG-RAN Cell Identity | M |  | 9.2.2.27 |  |
| >>Time UE Stays in Cell | O |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, or set of NG-RAN cells in seconds. If the duration is more than 4095s, this IE is set to 4095. |

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>