3GPP TSG-RAN WG3 Meeting #121															 R3-234379

# Toulouse, France, 21st – 25th August, 2023

Agenda item:		12.2.2.3

Source:	ZTE

Title:	(TP to 38.423) Further discussion on AI/ML based energy saving

Document for:		Others

Introduction

During the previous meeting, the metric of energy cost (EC) is agreed as follows:

EC is represented as an index, which should be normalized and defined by OAM. The index value could be encoded as an integer from 0 to a maximum. The maximum value should guarantee enough accuracy.

FFS on whether the inferred EC should be introduced in R18.

FFS on whether the additional load needs to be introduced in R18.

In this contribution, we elaborate our analysis on the open issues above related to energy consumption for AI/ML in the RAN, and provide the corresponding TP in the Annex.

Discussion

Encoding of the EC

In the last meeting, it was agreed that the index value is encoded as an integer from 0 to a maximum. With regard to the specification, maximum should be set to a specific value. Considering the method of normalization is various, to guarantee enough accuracy, the maximum can be set to a large value than 100. And the IE can be extensible for future proof.

- The range of the Energy Cost IE in the report message can be set to (0..1000, ...).

Method of transferring EC

The metric of Energy Cost (EC) exchanged between NG-RAN nodes can be an inferred energy consumption related to an additional load or an actual energy consumption value from a neighboring node for either additional load or current load (The details to be further discussed). EC is a value at gNB level.

Following the definition of EC above, it was found several cases are implicitly involved:

After the energy saving decision:

- Case a1: Energy consumption of the NG-RAN node after the energy saving decision.

- Case a2: Energy consumption of the NG-RAN node related to the additional load after the energy saving decision.

Before the energy saving decision:

- Case b1: Predicted Energy consumption related to the additional load before the energy saving decision.

However, upon checking the TR 37.817 [1], I noticed that one case we discussed before was not involved in the current definition.

============================= TR37.817 =============================

Input of AI/ML-based Network Energy Saving:

From local node:

-	UE mobility/trajectory prediction

-	Current/Predicted Energy efficiency

-	Current/Predicted resource status

From the UE:

-	UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

From neighbouring NG-RAN nodes:

-	Current/Predicted energy efficiency

-	Current/Predicted resource status

-	Current energy state (e.g., active, high, low, inactive)

============================= TR37.817 =============================

The yellow-highlighted input information is current/predicted energy efficiency from neighboring NG-RAN nodes. Now based on the current definition of metrics of energy consumption, this kind of input becomes current/predicted energy cost from neighbor NG-RAN nodes. Hence, the following cases should also be involved in the energy cost definition.

- Case b2: Current Energy consumption of the neighboring NG-RAN node for total load before the energy saving decision.

- Case b3: Predicted Energy consumption of the neighboring NG-RAN node for total load before the energy saving decision.

What should be emphasized here is that before AI/ML decision, the AI/ML input information is used for reference by NG-RAN node. Therefore, the current/predicted energy consumption of the NG-RAN node before the energy saving decision should also be considered.

- In the TR 37.817, the current/predicted energy consumption from neighbor NG-RAN nodes for total load is also considered as input information for AI/ML energy saving.

- Current/predicted energy consumption for total load should be transferred between NG-RAN nodes as input information for AI/ML energy saving.

Above all, the scenario of the definition of energy consumption involves following:

After the energy saving decision:

- Case a1: Energy consumption of the NG-RAN node after the energy saving decision.

- Case a2: Energy consumption of the NG-RAN node related to the additional load after the energy saving decision.

Before the energy saving decision:

- Case b1: Predicted Energy consumption related to the additional load before the energy saving decision.

- #R18不讨论

- Case b2: Current Energy consumption of the neighboring NG-RAN node for total load before the energy saving decision.

- Case b3: Predicted Energy consumption of the neighboring NG-RAN node for total load before the energy saving decision.

| - Data Type | - Usage | - Specification Impacts. |
| --- | --- | --- |
| - Case a1 | - Use the energy consumption for the total load to evaluate the performance of AI/ML based energy saving decision. - Reflect the overall EC status, including the offloading load, and other factors which have impacts on EC. | - Spec Impact: - Transfer the measured EC from the offloading target node to the source node via AI/ML INFORMAITON UPDATE message. |
| - Case a2 | - Using the energy consumption for the additional load to evaluate the performance of AI/ML based energy saving decision. - Reflect the EC status related to the offloading load. | - Spec Impact: - Transfer the measured EC related to additional load from the offloading target node to the source node via AI/ML INFORMAITON UPDATE message |
| - Case b1 | - The neighbour node provides the predicted EC related to the additional load, and then the source node evaluates if the energy saving decision is good to execute. | - Spec Impact: - 1. Provide the “additional load ” to the offloading target node - 2. Transfer the inferred EC from the offloading target node to the source node via AI/ML INFORMAITON UPDATE message |
| - Case b2 | - The neighbour nodes provide the source node with EC related to the total load. By collecting this data from neighbour nodes, the source node can generate a more accurate and informed energy saving decision. | - Spec Impact: - Transfer the measured EC from the neighbour node to the source node via AI/ML INFORMATION UPDATE message. |
| - Case b3 | - The neighbour nodes provide the source node with predicted EC related to the total load. By collecting this data from neighbour nodes, the source node can generate a more accurate and informed energy saving decision. | - Spec Impact: - Transfer the predicted EC from the neighbour node to the source node via AI/ML INFORMATION UPDATE message. |

For case a1 , the source node collects the measured EC related to the total load. Additionally, the source node can calculate the delta EC by measured EC collected before energy saving decision and after energy saving decision. How to use the measured EC to evaluate the performance of AI/ML based energy saving decision is up to source node itself.

- The source node can evaluate the executed energy saving decision through either the measured EC related to total load or delta EC calculated by measured EC collected before energy saving decision and after energy saving decision.

- Measured energy cost after energy saving decision is transferred from the offloading target node to the source node. How to use the measured energy cost is up to node itself.

With respect to the case a2, basically UE does offload to cell based on the energy saving decision, so the EC related to the additional load is per-cell level. However now we agree that the Energy Cost is a node level parameter and further EC granularity is out of scope of Rel18. Hence, it is not pursed to transfer the measured energy cost related to the additional load for now. Moreover, if we consider the energy consumption related to the additional load per NG-RAN node, the energy consumption of the current load is much more straightforward.

- Transferring the measured energy cost of the current load of the NG-RAN node after energy decision is much more straightforward.

- It is nor pursed to transfer the measured energy cost related to the additional load after energy decision in this release.

Based on my understanding, in Case b1, the predicted energy cost is linked to the additional load. In order to understand the predicted energy cost associated with an additional load, the source NG-RAN node must first inform the AI/ML energy saving decision e.g., “additional load” to the target NG-RAN node. The target NG-RAN node can then use this information to predict the energy cost based on the decision and communicate the predicted energy cost related to the potential additional load back to the source NG-RAN node.

However, following are a couple of issues:

Firstly, it may not be feasible for the RAN node to make a determination based on predictions over prediction. The inferred energy consumption related to energy consumption is based on the predicted energy saving decision, and the current energy consumption of the NG-RAN node itself. Since finally the source node will determine whether the energy saving decision is feasible, why not the target NG-RAN node provide its current energy efficiency and predicted energy efficiency to the source NG-RAN node. Based on these information, the source NG-RAN node could generate and evaluate the appropriate energy saving decision.

- Providing the current/predicted energy cost before energy saving decision to the node who generate the energy saving decision directly can improve the accuracy of AI/ML based energy saving decision.

- It is unnecessary to transferring the AI/ML based energy saving decision, e.g., additional load, to the neighbor NG-RAN nodes.

Secondly, the specification for the energy saving decision needs to be clearly defined, for example, specifying the number of UE that will be offloaded. It is hard to define the energy saving decision in the specification. In order to completely reflect the ‘additional load’, the potential offloading target node should know much information of the UE to be offloaded. However, it is hard and complex to provide so much information in the signaling to reflect the additional load.

- Transferring predict energy consumption related to additional load may not be possible at this time.

Above all, we propose to transfer the predicted energy consumption and current energy consumption per node over Xn interface. And the agreed AI/ML information Request/Response/Update message could be reused for these kind of information.

- Introduce the bit of predicted energy consumption and the bit of energy consumption in the Report Characteristic IE in the AI/ML INFORMATION REQUEST message.

- Introduce the energy consumption and predicted energy consumption per node in the AI/ML INFORMATION UPDATE message.

Request Type Indication

We can see that in cases a1/a2 and b2, there is a need to transfer energy consumption between NG-RAN nodes. However, from a signaling perspective, the design of energy consumption is the same in both cases, which can create confusion for the target NG-RAN node when it receives the request message. In the current specification, the report characteristics in the request message only indicates the ‘measured energy cost’, the requested node can not understand whether the energy cost is immediately collected or collected after the energy saving decision (e.g., handover).

- The target NG-RAN node cannot distinguish the case between case a1/a2 (energy consumption after energy saving decision) and case b1 (energy consumption before the energy saving decision).

To address this issue, one possible solution is to introduce a request type indication in the request message. By doing so, the target NG-RAN node can understand which type of measurement the source NG-RAN node requires, whether it is feedback information after UE handover or current information. This can help to eliminate the confusion caused by the similarity in the design of energy consumption and ensure that the correct information is conveyed to the target NG-RAN node.

- Introduce the Delayed Collection IE in the request message that the measurement is triggered after the UE(s) handover in order to ensure that the correct information is conveyed to the target NG-RAN node.

Conclusion

We propose the following observations and proposals:

Encoding of the EC:

- The range of the Energy Cost IE in the report message can be set to (0..1000, ...).

Method of transferring EC:

- In the TR 37.817, the current/predicted energy consumption from neighbor NG-RAN nodes for total load is also considered as input information for AI/ML energy saving.

- Current/predicted energy consumption for total load should be transferred between NG-RAN nodes as input information for AI/ML energy saving.

- The source node can evaluate the executed energy saving decision through either the measured EC related to total load or delta EC calculated by measured EC collected before energy saving decision and after energy saving decision.

- Measured energy cost after energy saving decision is transferred from the offloading target node to the source node. How to use the measured energy cost is up to node itself.

- Transferring the measured energy cost of the current load of the NG-RAN node after energy decision is much more straightforward.

- It is nor pursed to transfer the measured energy cost related to the additional load after energy decision in this release.

- Providing the current/predicted energy cost before energy saving decision to the node who generate the energy saving decision directly can improve the accuracy of AI/ML based energy saving decision.

- It is unnecessary to transferring the AI/ML based energy saving decision, e.g., additional load, to the neighbor NG-RAN nodes.

- Transferring predict energy consumption related to additional load may not be possible at this time.

- Introduce the bit of predicted energy consumption and the bit of energy consumption in the Report Characteristic IE in the AI/ML INFORMATION REQUEST message.

- Introduce the energy consumption and predicted energy consumption per node in the AI/ML INFORMATION UPDATE message.

- The target NG-RAN node cannot distinguish the case between case a1/a2 (energy consumption after energy saving decision) and case b1 (energy consumption before the energy saving decision).

- Introduce the Delayed Collection IE in the request message that the measurement is triggered after the UE(s) handover in order to ensure that the correct information is conveyed to the target NG-RAN node.

- References

[1] TR37.817

- Annex-A TP to 38.423

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

    - 8.4.AA	AI/ML Information Reporting Initiation (FFS on the name)

    - 8.4.AA.1	General

This procedure is used by an NG-RAN node to request the reporting of AI/ML related information to another NG-RAN node.

The procedure uses non UE-associated signalling.

Editor’s Note: FFS other information that can be requested using this procedure.

Editor’s Note: FFS content of AL/ML related information.

    - 8.4.AA.2	Successful Operation

Figure 8.4.AA.2-1: AI/ML Information Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the AI/ML INFORMATION REQUEST message to NG-RAN node2 to start AI/ML related information reporting and stop AI/ML related information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested AI/ML related information reporting according to the parameters given in the request in case the Registration Request IE is set to “start”; or

-	shall stop all cells AI/ML related information reporting and terminate the reporting in case the Registration Request IE is set to “stop”; or

-	FFS

If the Registration Request IE is set to “start” in the AI/ML INFORMATION REQUEST message and the Report Characteristics IE indicates cell specific AI/ML related information reporting, the Cell To Report List IE shall be included.

If NG-RAN node2 is capable to provide all of the requested information, it shall initiate the AI/ML related information reporting as requested by NG-RAN node1 and respond with the AI/ML INFORMATION RESPONSE message.

If NG-RAN node2 is capable to provide some but not all of the requested information, it shall initiate the AI/ML related information reporting for the admitted requested information and include the Node Measurement Initiation Result IE or the Per Cell Measurement Initiation Result IE or both in the AI/ML INFORMATION RESPONSE message.

If the Reporting Periodicity IE in the AI/ML INFORMATION REQUEST is present, this indicates the periodicity for the reporting of periodic AI/ML related information. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

If the Requested Prediction Time IE in the AI/ML INFORMATION REQUEST message is present, it indicates the specific point in time to which the prediction of the information requested applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

If the Delayed Collection IE is set to “true” in the AI/ML INFORMATION REQUEST message, this indicates the request type of AI/ML related information. The NG-RAN node2 shall consider the information reported as feedback information and report the requested information after specific AI/ML related decision, e.g. handover.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the AI/ML INFORMATION REQUEST indicates the type of objects NG-RAN node2 shall perform measurements or prediction on. NG-RAN node2 shall include in the AI/ML INFORMATION UPDATE message:

-	the Predicted Radio Resource Status IE, if the first bit, “Predicted Radio Resource Status” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”. FFS on the details of Predicted Radio Resource Status IE.

-	the Predicted Number of Active UEs IE, if the second bit, “Predicted Number of Active UEs” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”;

-	the Predicted RRC Connections IE, if the third bit, “Predicted RRC Connections” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

-	the Average UE Throughput DL IE, if the fourth  bit, “Average UE Throughput DL” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

-	the Average UE Throughput UL IE, if the fifth bit, “Average UE Throughput UL” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

-	the Average Packet Delay IE, if the sixth bit, “Average Packet Delay” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

-	the Average Packet Loss IE, if the seventh bit, “Average Packet Loss” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

-  the Actual Energy Cost IE, if the eighth bit, “Actual Energy Cost” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

-  the Inferred Energy Cost IE, if the ninth bit, “Inferred Energy Cost” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

    - 9.1.3.CC	AI/ML INFORMATION REQUEST (FFS on the name)

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested AI/ML related information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) (FFS on others) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss Eighth Bit = Actual Energy Cost Ninth Bit = Inferred Energy Cost FFS on the coding | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | FFS |  | YES | ignore |
| Delayed Collection | O |  | ENUMERATED(true, …) | This IE indicates the requested AI/ML related information is collected after the action if present. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

    - 9.1.3.DD	AI/ML INFORMATION RESPONSE (FFS on the name)

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested AI/ML related information, for all or part of the measurement objects included in the reporting is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Node Measurement Initiation Result |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | ignore |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | EACH | ignore |
| >>Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(128)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Actual Energy Cost. Second Bit = Inferred Energy Cost. Other bits shall be ignored by the NG-RAN node1. |  |  |
| >>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Per Cell Measurement Initiation Result |  | 0..1 |  | List of measurement objects that failed to be initiated per cell. | YES | ignore |
| >Per Cell Measurement Initiation Result Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | EACH | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the cell. | – |  |
| >>>Measurement Failure Cause Item |  | 1 .. <maxFailedMeasObjects> |  |  |  |  |
| >>>>Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(128)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss. Other bits shall be ignored by the NG-RAN node1. | – |  |
| >>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxFailedMeasObjects | Maximum number of measurement objects that can fail per measurement. Value is 124. |
| maxFailedMeasPerNode | Maximum number of measurement objects that can fail per node. Value is 124. |

    - 9.1.3.EE	AI/ML INFORMATION FAILURE (FFS on the name)

This message is sent by the NG-RAN node2 to NG-RAN node1 to indicate that for all of (exact details of if and how to support partial reporting are FFS) the requested objects the reporting cannot be initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

    - 9.1.3.FF	AI/ML INFORMATION UPDATE (FFS on the name)

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested AI/ML related information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell AI/ML Info Result (FFS on the name) |  | 0..1 |  |  | YES | ignore |
| >Cell AI/ML Info Result Item(FFS on the name) |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  |  |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| >>Predicted Number of Active UEs | O |  | 9.2.2.62 |  | –- |  |
| >>Predicted RRC Connections | O |  | 9.2.2.56 |  | – |  |
| UE Associated Info Result |  | 0..1 |  |  |  |  |
| > UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  |  |  |
| >> UE Assistant Identifier (FFS on the name) | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1 |  |  |
| >>UE Performance | O |  | 9.2.3.Y |  |  |  |
| >>UE trajectory | O |  | FFS |  |  |  |
| Actual Energy Cost | O |  | INTEGER(0..1000,...) | It represents the actual measurement of the Energy Cost |  |  |
| Inferred Energy Cost | O |  | INTEGER(0..1000,...) | It represents the inferred measurement of the Energy Cost |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEReports | Maximum no. UE that can be served by a NG-RAN node. Value is FFS. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>