3GPP TSG-RAN WG3 #121       	                                       R3-234391

21th – 25th Aug 2023 Toulouse, France

Agenda item:		11.3

Source:				CATT

Title:	Discussion on Support for legacy QoE in NR-DC

Document for:		Discussion and Decision

1		Introduction

In RAN3#120 meeting, the NR QoE supporting in NR-DC was discussed. The legacy QoE related agreements and open issues are captured in chair Note[1] as below:

When a management-based QoE configuration is received directly by the SN from the OAM, the SN can explicitly indicate to the MN whether it is going to receive the QoE/RVQoE reports via the SRB5 or SRB4, whether it is transparently via SRB4 needs further discussion.

RAN3 understanding is that the UE can be explicitly instructed whether to report over SRB5 or SRB4.

FFS whether:

- For management-based QoE, the SN can explicitly indicate to the MN whether it wants to configure the UE via SRB3.

- For management-based QoE, the SN can explicitly indicate to the MN whether it wants to configure the UE transparently for the MN via SRB1.

Mobility

In line with the Rel-17 principles, if the MN configured the UE with QoE measurements, every subsequent MN serving the UE can configure and release the RVQoE measurements.

PSCell change

At SN change without MN change, the QoE configuration information is passed from the old/source SN to the new/target SN via the MN.

SN release

If the SN configured a UE with QoE measurements, at SN release, the QoE/RVQoE configuration can be released. Whether the SN-configured QoE/RVQoE configuration information can be passed to the MN in case of SN release needs to be further discussed.

This contribution will further discuss the supporting on the legacy QoE in NR-DC. And provide the view on the open issues .

2		Discussion

2.1 Reporting and configuring leg indication

In last meeting, we agreed: When a management-based QoE configuration is received directly by the SN from the OAM, the SN can explicitly indicate to the MN whether it is going to receive the QoE/RVQoE reports via the SRB5 or SRB4, whether it is transparently via SRB4 needs further discussion.

But there is still one open issue, whether it is transparently via SRB4 needs further discussion. Because we already agree: if a node has configured the UE with QoE measurements, and the other node is receiving the QoE reports from the UE and forwarding them directly to the MCE. So if the MN receive the SN configured QoE report, the report can be sent to MCE directly. It is no necessary to transparently send to SN.

Proposal 1: QoE report to SN transparently via SRB4 is not needed

We agreed that the leg switching command can be sent to the UE by the node that configured that specific QoE configuration. Both the MN node and SN node may encounter the overload status, to avoid introducing delay and reduce the overhead in interface, we should allow both MN and SN can send the indication of switching reporting leg based on its overload status. The SN may send the command via SRB3 or SRB1

Proposal 2: Both MN and SN node can command the UE to switch the reporting leg based on its overload status

For the configuring, we have one FFS left in last meeting. For management-based QoE, the SN can explicitly indicate to the MN whether it wants to configure the UE via SRB3 or transparently for the MN via SRB1. In current specification, the SN sends information to UE transparently for the MN via SRB1 does not need to indicate the MN. So in this case, it can follow the existing rule.

Proposal 3: The SN can explicitly indicate to the MN whether it wants to configure the UE via SRB3

2.2 SN coordination with MN about M-based QMC

In last meeting, the TP for 38.423 is agreed, only the IEs for SN initiate the coordination and modification is included. The exact procedure is not defined yet for the SN express to MN the M-based QoE Measurements. Either defines new procedure or reuse the existing SN node initiated S-NG-RAN node Modification. After checking the message S-NODE MODIFICATION REQUIRED is suitable for the function. So we may straightforwardly use this procedure

Proposal 4: Reuse the existing SN node initiated S-NG-RAN node Modification procedure to express to the MN its interest in configuring a UE with an m-based QoE and RV QoE

The SN may receive the below parameter of m-based QoE configuration from OAM as specified in TS 28.405 [2]: Service Type, area Scope, QoE Collection Entity Address, QoE Reference, MDT Alignment Information, available RAN QoE Metrics and QMC Configuration File. When the SN wants to express to MN its interest in this configuration, we agreed that at least the QoE reference should be included in the information. If only QoE reference is include, we may suppose the MN also received the m-based QoE configuration with same QoE reference. Otherwise the MN does not know what the SN interest QoE is. So the service type should also be included. But the information includes QoE reference and service type only applies the two steps coordination case. The two steps coordination case means the SN only express its interest in the QoE configuration via the QoE ID in first step. And exchange the information for configuration sending and report receiving, forwarding in second step after get MN accepted. If one step used, all the parameters received from OAM should be sent to MN if the MN sends the configuration to UE. If the MN is set to receive the QoE report, the QoE Collection Entity Address should be sent to the MN.

Proposal 5: The SN may send parameters to MN for different purpose in the first message to MN or in the subsequent message after MN accept the configuration. The parameters include QoE Reference, Service Type, area Scope, QoE Collection Entity Address, MDT Alignment Information, available RAN QoE Metrics, QMC Configuration File, and indication of configuring leg and reporting leg.

2.3 Support for alignment of QoE measurements and radio related measurement

In NR-DC, the immediate MDT can be configured in the MN and SN. But the logged MDT can be configured only in MN. The logged MDT aligns with the QoE in idled and inactive state should be discussed in the work item of the idle QoE and the NR-DC will not impact the alignment. For the QoE in connected state align with the immediate MDT in MN and SN, aligned immediate MDT in MN and SN may be the same MDT or different MDT. To simple the specification, we should restrict the alignments to only supporting the same MDT. How to trigger the MDT configured should be enhanced base on the current specification. The QoE start indication should send to SN for the SN MDT configuration.

Proposal 6: aligned immediate MDT in MN and SN should be the same MDT

Proposal 7: The QoE start indication should be sent to SN for the SN MDT configuration initial

3		Conclusion

In the present contribution we make the following observations and proposal:

Proposal 1: QoE report to SN transparently via SRB4 is not needed

Proposal 2: Both MN and SN node can command the UE to switch the reporting leg based on its overload status

Proposal 3: The SN can explicitly indicate to the MN whether it wants to configure the UE via SRB3

Proposal 4: Reuse the existing SN node initiated S-NG-RAN node Modification procedure to express to the MN its interest in configuring a UE with an m-based QoE and RV QoE

Proposal 5: The SN may send parameters to MN for different purpose in the first message to MN or in the subsequent message after MN accept the configuration. The parameters include QoE Reference, Service Type, area Scope, QoE Collection Entity Address, MDT Alignment Information, available RAN QoE Metrics, QMC Configuration File, and indication of configuring leg and reporting leg.

Proposal 6: aligned immediate MDT in MN and SN should be the same MDT

Proposal 7: The QoE start indication should be sent to SN for the SN MDT configuration initial

4		Reference

- RAN3_120_agenda_20230526_EOM

- TS28.405 Quality of Experience (QoE) measurement collection; Control and configuration