3GPP TSG-RAN WG3 #121      	                                  R3-234395

21th – 25th Aug 2023 Toulouse, France

Agenda item:		14.4

Source:				CATT

Title:	Discussion on NR-DC with selective activation of the cell groups

Document for:		Discussion and Decision

1		Introduction

In RAN3 #120 meeting, the below agreements and open issues are captured in [1] as below.

Add a new IE as a sub IE of the Conditional PSCell Change Information Required IE in the S-NODE CHANGE REQUIRED message to indicate that the request is for SCG Selective Activation.

It is FFS (up to RAN2 progress) what form the indication will have (an explicit IE or an RRC container).

Note: This can be revisited based on RAN2 progress.

Add a new IE as a sub IE of the Conditional PSCell Addition Information Request IE in the S-NODE ADDITION REQUEST message to indicate that the request is for SCG Selective Activation.

It is FFS (up to RAN2 progress) what form the indication will have (an explicit IE or an RRC container).

Note: This can be revisited based on RAN2 progress.

Introduce a new indicator for selective activation in the S-NODE MODIFICATION REQUEST message to indicate that the request is for SCG selective activation. FFS on the scenarios.

Introduce a new indicator for selective activation in the Xn-U address indication message to indicate that the request is for SCG selective activation. FFS on the scenarios.

And in RAN2#122 meeting, the below agreements for selective SCG are captured.

For SN-initiated SCG selective activation, candidate SN generates execution conditions for subsequent CPC.

FFS if it shall be possible to do something like MN-initiated CPA/CPC where Candidate SN generate execution conditions for subsequent CPC

The UE shall skip the condition evaluation for a candidate which is a current PScell.

The reference configuration is provided to all candidates involved in preparation, FFS which node initially generates it. Assume it can be provided in MN initiated and in SN initiated procedures.

Will not spend specific efforts for supporting nested configurations for candidate cell configuration.

Rapporteur take initiative on naming offline

Terminology is “Subsequent CPAC”

This contribution will continue discussing the selective activation for cell group based on above agreements and open issue list.

2		Discussion

In last meeting, RAN2 agreed: The reference configuration is provided to all candidates involved in preparation, FFS which node initially generates it. Assume it can be provided in MN initiated and in SN initiated procedures.

According the agreements, the reference configuration should be sent from the node which initially generates it to all candidates. Whatever which node generates it, the reference configuration should be transfer from MN to candidate SNs via S-NODE Addition Request message when the SN added. In R17 and before version, the CG-ConfigInfo IE carried in M-NG-RAN node to S-NG-RAN node Container IE is used to transfer the RAN2 defined CG information from MN to SN. So the reference configuration also can be transferred in this IE from MN to SN.

Proposal 1: Transferring reference configuration from MN to SN in the container in which including RAN2 defined CG-ConfigInfo

The SN will be confirmed whether the reference configuration is used for the SCG delta configuration. The indication can be within the CG-CandidateList IE in the S-NG-RAN node to M-NG-RAN node Container IE in S-NODE Addition Request Acknowledge message.

Proposal 2: The indication for denoting whether the reference configuration is used can be within the CG-CandidateList IE in the S-NG-RAN node to M-NG-RAN node Container IE.

RAN2 agreed: For SN-initiated SCG selective activation, candidate SN generates execution conditions for subsequent CPC. According this agreement, the MN will forward the entire source SN wanted PScell list to all candidate SN nodes. And then all candidate SN nodes generated the execution conditions for all source SN wanted PScells. RAN2 should include this information in the CG-ConfigInfo and CG-CandidateList . It should not impact RAN3. But we need wait for RAN2 progress

Proposal 3: candidate SN generates execution conditions for subsequent CPC should not impact RAN3 spec.

The selected SN should be informed the data forwarding address of other configured and kept candidate SN after one candidate executed to support the early data forwarding. So then the new SN can perform the early data forwarding to those candidate SNs. We agreed: Reuse the Xn-U Address Indication message and the Early Status Transfer message to support early data forwarding for SCG Selective Activation. The data forwarding address of the candidate may be sent to each other at the CPAC configuration phase or after one candidate executed.

Proposal 4: The data forwarding address of the candidates is sent to each other at the CPAC configuration phase or after one candidate executed.

The candidate SN CU-CP should inform the CU-UP via E1 about the data forwarding information when the data forwarding address is changed.

Proposal 5: The Bearer Context Modification (gNB-CU-CP initiated) procedure can be used for the updating of data forwarding address via on E1

3		Conclusion

In the present contribution we make the following observations and proposal:

Proposal 1: Transferring reference configuration from MN to SN in the container in which including RAN2 defined CG-ConfigInfo likely

Proposal 2: The indication for denoting whether the reference configuration is used can be within the CG-CandidateList IE in the S-NG-RAN node to M-NG-RAN node Container IE.

Proposal 3: candidate SN generates execution conditions for subsequent CPC should not impact RAN3 spec.

Proposal 4: The data forwarding address of the candidates is sent to each other at the CPAC configuration phase or after one candidate executed.

Proposal 5: The Bearer Context Modification (gNB-CU-CP initiated) procedure can be used for the updating of data forwarding address via on E1

4		Reference

- RAN3_120_agenda_20230526_EOM