3GPP TSG-RAN WG3 #121   	                      R3-234396

21th – 25th Aug 2023 Toulouse, France

Agenda item:		26.2

Source:				CATT

Title:	Discussion on Network timing synchronization status and reporting

Document for:		Discussion and Decision

1		Introduction

In RAN3#120 meeting, the Network timing synchronization status and reporting was discussed. agreements and open issues captured as below:

In NGAP, introduce a new Timing Synchronisation Status Reporting Initiation procedure (class 1) to enable the AMF to initiate RAN TSS reporting by the gNB?

In NGAP, introduce a new Timing Synchronisation Report procedure (class 2) to enable the gNB to report RAN TSS to the AMF.

In F1AP, introduce a new Timing Synchronisation Report procedure (class 2) to enable the gNB-DU to report RAN TSS to the gNB-CU.

WA: When Clock Quality Detail Level IE has value “clock quality metrics”, all clock quality metrics supported by the gNB implementation are delivered to the UE (may be revisited based on SA2 agreements).

Other open issues:

The Clock Quality Acceptance Criteria IE can reuse the same encoding as the RAN Timing Synchronisation Status IE?

Whether a separate IE is needed?

How to encode The Clock Quality Acceptance Criteria IE?

How to encode the RAN Timing Synchronisation Status IE?

This contribution will discuss open issues and provide the proposals

2		Discussion

2.1 gNB reporting node-level RAN timing synchronization status information towards the AMF

In last RAN3 meeting, we agree introduce a new class 2 procedure to enable the gNB to report the TSS to AMF. But whether introduce new procedure to initiate the RAN reporting is not concluded. Even though the existing AMF Configuration Update procedure can realize this function, we may introduce new class 1 procedure for the reporting initiated due we already agree to use new procedure for reporting. So the specification readability and alignment is better.

Proposal 1: introduce a new Timing Synchronisation Status Reporting Initiation procedure (class 1) to enable the AMF to initiate RAN TSS reporting by the gNB in NGAP

Also align with NGAP, introduce a new Timing Synchronisation Status Reporting Initiation procedure (class 1) to enable the gNB-CU to initiate RAN TSS reporting by the gNB-DU in F1AP

Proposal 2: introduce a new Timing Synchronisation Status Reporting Initiation procedure (class 1) to enable the gNB-CU to initiate RAN TSS reporting by the gNB-DU in F1AP

Bases on the AMF request, the RAN should monitor the time synchronization status and send the report to AMF as descripted in TS23.501

--excerpt from 23.501

gNBs may be pre-configured with thresholds for each timing synchronization status attribute, if supported, that is described in Table 5.27.1.12-1. When the network timing synchronization status exceeds the thresholds (i.e. status degradation), or the network timing synchronization status meets the thresholds again (i.e. status improvement), the gNB notifies the TSCTSF (either using N2 node level signalling via AMF, or via OAM) with the gNB ID, the scope of the timing synchronization status (i.e., all cells or a list of Cell IDs within a single gNB) and the corresponding network timing synchronization status (TSS) attributes as described in this clause. The gNB indicates the status change to the UEs via SIB9:

--end of excerpt from 23.501

From the above description, the RAN will send the node-level status report to AMF when the status changes. The information includes RAN node ID, the scope of status and the details of the status.

Proposal 3: Include RAN Node ID, the scope of TSS and the corresponding network timing synchronization status attributes in TSS report message

Regarding the scope of the timing synchronization status, it should be same as scope of the report ID in the LS from SA2. RAN3 has response in LS that may be a cell list. The scope is defined a gNB ID or a group of cells within the gNB specified with a list of Cell IDs.

Proposal 4: The scope of the timing synchronization status may be gNB ID or cell list within the gNB

3		Conclusion

In the present contribution we make the following observations and proposal:

Proposal 1: introduce a new Timing Synchronisation Status Reporting Initiation procedure (class 1) to enable the AMF to initiate RAN TSS reporting by the gNB in NGAP

Proposal 2: introduce a new Timing Synchronisation Status Reporting Initiation procedure (class 1) to enable the gNB-CU to initiate RAN TSS reporting by the gNB-DU in F1AP

Proposal 3: Include RAN Node ID, the scope of TSS and the corresponding network timing synchronization status attributes in TSS report message

Proposal 4: The scope of TSS may be gNB ID or cell list within the gNB

4		Reference