3GPP TSG-RAN WG3 Meeting #121	R3-234404

Toulouse, 21 Aug – 25 Aug 2023

Title: 	NR QoE continuity during intra-5GC inter-RAT handover

Source: 	China Unicom

Agenda item:	11.4

Document Type:	Discussion

Introduction

The Work Item on QoE reporting has been agreed in RAN#96 (RP-221803) with the following objectives:

- Support the continuity of legacy QoE measurement job for streaming and MTSI service during intra-5GC inter-RAT handover process [RAN2, RAN3].

This paper discusses how to support the continuity of legacy QoE measurement job for streaming and MTSI service during intra-5GC inter-RAT handover process.

Discussion

HO from NR to LTE/5GC

In the current specification, the area scope is configured for only NR CGI, it needs to be enhanced to support E-UTRA CGI to support the intra-5GC inter-RAT handover between LTE/5GC and NR.

Proposal 1: The area scope of QoE configuration should be extend to support the E-UTRA CGI.

During RAN2 Meeting #121, there is a LS send to RAN3:

Agree on the principles of Option 3 and Option 4: - Option 3: For HO from NR to LTE/5GC, the UE can keep and continue measurements for only one configuration for a service type supported in LTE.

In this release, the NR QoE enhancement will not have any impacts to current LTE specification, but the QoE configuration for LTE and NR are different, some enhancements for how to coordinate the configuration between LTE and NR are needed.

In NR, multiple QoE configurations are supported, and in LTE, only one QoE configuration is supported.

In RAN3#120 meeting, RAN3 have the following WA:

WA: From NR to LTE, the source node adapts to the target node on deciding which one of the QoE configuration to keep.

Considering there should be no impacts for LTE specification(TS36.331), the QoE configuration inside the RRC container can only transfer one QoE configuration according to current TS36.331, but multiple QoE configuration can be sent between NR and LTE outside the RRC container, which is already supported in current specification.

There is a scenario that the UE move from NR to LTE, then move back to NR quickly, it is better for the NR to recover all the s-based QoE configuration in NR though only one QoE configuration is supported in LTE. In this scenario, all the s-based QoE configuration need to be sent from NR to LTE, and send back from LTE to NR when UE move back to NR.

Proposal 2: The source NR node should send all the s-based QoE measurements configuration to target LTE node, and all the s-based QoE measurements configuration should be sent back from LTE to NR when UE move back to NR.

Proposal 3: Turn the following WA to agreement:

WA: From NR to LTE, the source node adapts to the target node on deciding which one of the QoE configuration to keep.

Proposal 4: For the HO from NR to LTE/5GC, if the QoE configuration send from the source node is not supported by the target node (inside the RRC container), the target node can choose any QoE configuration outside the RRC container.

HO from LTE/5GC to NR

Since the LTE/5GC only support one QoE configuration and it is no need to use QoE reference and RRC ID to identify different QoE configurations. The target NR can allocate a RRC ID according to the QoE reference send from the source LTE/5GC in HandoverCommand message defined in TS38.331, and the RRC ID can be used by the QoE reporting send from UE to gNB. In Rel-17 NR QoE, the RRC ID is already supported in the TS38.331.

Proposal 5: Turn the following WA to agreement:

WA: For HO from LTE/5GC to NR, there is no impact for RAN3 specification.

Conclusion

Proposal 1: The area scope of QoE configuration should be extend to support the E-UTRA CGI.

Proposal 2: The source NR node should send all the s-based QoE measurements configuration to target LTE node, and all the s-based QoE measurements configuration should be sent back from LTE to NR when UE move back to NR.

Proposal 3: Turn the following WA to agreement:

WA: From NR to LTE, the source node adapts to the target node on deciding which one of the QoE configuration to keep.

Proposal 4: For the HO from NR to LTE/5GC, if the QoE configuration send from the source node is not supported by the target node (inside the RRC container), the target node can choose any QoE configuration outside the RRC container.

Proposal 5: Turn the following WA to agreement:

WA: For HO from LTE/5GC to NR, there is no impact for RAN3 specification.