3GPP TSG-RAN WG3 #121								R3-234406

Toulouse, France,21th – 25th Aug 2023

Agenda Item:	25.2

Source:	China Telecom

Title: 	Discussion on Support of XR Enhancement in Split Architecture

Document for:	Discussion

Introduction

The updated WID on XR Enhancements for NR was agreed in RAN#99 meeting[1]. The objectives on RAN3 are as follows:

- Specify the enhancements related to capacity:

-	Discard operation of PDU Sets for DL and UL (RAN2, RAN3);

- Specify the enhancements for XR Awareness:

-	Signalling by CN of semi-static information per QoS flow (e.g., PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2);

-	Provisioning by UE of XR traffic assistance information e.g., periodicity, UL traffic arrival information (RAN2, RAN3);

-	Support signalling the congestion information from RAN to the CN in alignment with SA2 (RAN3);

The intention of this paper is to discuss the E1 and F1 impact of XR enhancement, and provide some proposals.

Discussion

In order to support XR enhancement, SA2 had agreed that CN needs to provide the following information to NG-RAN:

- The PDU set related information per QoS flow

- dynamic information per PDU set(via user plane)

- End of Data Burst indication

Given that the dynamic information per PDU set including PDU Set Size (in bytes), PDU Set SN, PDU SN within a PDU Set, Indication of End PDU of the PDU Set, and PDU Set Importance (to identify the importance of a PDU Set within a QoS Flow) is provided, there is no need to configure this dynamic information over E1 and F1 interface. In addition, given that the DL End of Data burst Indication is contained in the GTP-U header of the last PDU in downlink while UL indicator uses an implicit way, e.g. (Padding) BSR with BS value equal to zero, the DL/UL indicator can be regarded as a user plane function. Therefore, there is no need to consider Indication of End of Data burst for E1 and F1 interface.  The PDU set related information per QoS flow are provided via control plane:

- Traffic Periodicity (via TSCAI/TSCAC)

- Traffic Jitter (via TSCAI)

- PDU Set QoS parameters (used to support PDU Set level handling in the NG-RAN)

  - PDU Set Error Rate (PSER)

  - PDU Set Delay Budget (PSDB), optional

  - PDU Set Integrated handling Indication (PSIHI)

Here we discuss which parameters are needed in E1 and F1 interface. To configure UE power saving management scheme for connected mode DRX, SA2 agreed that UL/DL Periodicity, the N6 Jitter Information associated with the DL Periodicity also need to be provided to NG-RAN. For UL direction, RAN2 agreed that UE can report the range of jitter associated with UL XR traffic. With such information, the NG-RAN can configure properly radio resources for UE to mitigate the extra access delay caused by jitter. Furthermore, in case XR device and UE are not co-located, it is useful for UE to inform network of its UL jitters information. In RAN2#122, UL assistance information including burst arrival time, UL jitter, and periodicity (FFS) is reported per QoS flow.

For E1 and F1 interface, current TSCAI only includes direction, periodicity, Burst Arrival Time, and Survival Time. Therefore, CU-CP needs to provide N6 Jitter Information and UL jitter in TSCAI to DU and CU-UP to support XR Enhancement.

Proposal 1: CU-CP needs to provide N6 Jitter Information and UL jitter in TSCAI to DU and CU-UP to support XR Enhancement

XR-Awareness relies on PDU Set QoS Parameters specified in TS23.501, such as QoS flows, PDU Sets, Data Bursts and traffic assistance information. When a QoS Flow is enabled with PDU Set based QoS handling by 5GC, PDU Set QoS Parameters may be provided by the SMF to the gNB as part of the QoS profile of the QoS flow:

- PDU Set Delay Budget (PSDB): A QoS Flow is associated with only one PSDB for both UL and DL, and when available, it supersedes the PDB.

- PDU Set Error Rate (PSER): A QoS Flow is associated with only one PSER for both UL and DL, and when available, it supersedes the PER.

- PDU Set Integrated Handling Information (PSIHI): indicates whether all PDUs of the PDU Set are needed for the usage of PDU Set by application layer.

For E1/F1 interface, the PDB and PER are contained in GBR and NGBR QoS configuration. Furthermore, RAN2 agreed that the definitions of PSDB, PSER, PSIHI, PSI, EoDB etc. defined in TS23.501 are not changed in RAN2 specifications. According to the definition of PSIHI, this parameter is not needed in gNB-DU and gNB-CU-UP. Based on this, PSDB and PSER need to be contained in GBR and NGBR QoS configuration over E1/F1 interface.

Proposal 2: PSDB and PSER need to be contained in GBR and NGBR QoS configuration over E1/F1 interface.

In addition, in RAN2#122 meeting, a new DRX cycle based on rational numbers was agreed. Therefore, a new DRX cycle based on rational numbers need to be contained in F1 interface. Details of new DRX cycle depend on RAN2 progress.

Proposal 3: a new DRX cycle based on rational numbers need to be contained in F1 interface.

To support signalling the congestion information from RAN to the CN in alignment with SA2 is another objective of XR Enhancement. According to TS23.501, the NG-RAN may be required to provide the UL and/or DL QoS Flow congestion information. In addition, NG-RAN also supports ECN marking for L4S to expose the congestion information. This function exposes congestion information by marking ECN bits in the IP header of the user IP packets between the UE and the application server to trigger application layer rate adaptation [2]. For split architecture, the gNB-CU-CP requires the gNB-DU to perform congestion monitoring, the gNB-DU reports the congestion indication to gNB-CU-UP, and gNB-CU-CP needs to inform gNB-CU-UP to perform ECN marking for a specific QoS flow.

Proposal 4: the gNB-CU-CP requires the gNB-DU to perform congestion monitoring, the gNB-DU reports the congestion indication to gNB-CU-UP

Proposal 5: gNB-CU-CP needs to inform gNB-CU-UP to perform ECN marking for a specific QoS flow

We believe the Stage-2 TS38.401 also needs to be updated to support XR, thus we provide a TP for TS38.401 in [3].

Proposal 6: the Stage-2 TS38.401 needs to be updated to support XR

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: CU-CP needs to provide N6 Jitter Information and UL jitter in TSCAI to DU and CU-UP to support XR Enhancement

Proposal 2: PSDB and PSER need to be contained in GBR and NGBR QoS configuration over E1/F1 interface.

Proposal 3: a new DRX cycle based on rational numbers need to be contained in F1 interface.

Proposal 4: the gNB-CU-CP requires the gNB-DU to perform congestion monitoring, the gNB-DU report the congestion indication to gNB-CU-UP

Proposal 5: gNB-CU-CP needs to inform gNB-CU-UP to perform ECN marking for a specific QoS flow

Proposal 6: the Stage-2 TS38.401 needs to be updated to support XR

Reference

- RP-230786，Updated WID on XR Enhancements for NR，Nokia, Qualcomm，3GPP TSG RAN Meeting #99 Rotterdam, Netherlands, March 20-23, 2023

- 3GPP TS 23.501 V18.2.2，System architecture for the 5G System (5GS);Stage 2

- (BLCR to 38.401) Introduction of XR enhancements