3GPP TSG-RAN WG3 Meeting #121                                                                  R3-234442

Toulouse, France, Aug 21st - Aug 25th, 2023

Agenda Item:	12.2.2.3

Source:	CMCC

Title:	Further Discussion on Inferred EC for AI/ML for NG-RAN Energy Saving

Document for:	Discussion and Decision

1	Introduction

During meeting #119, an enhanced option on how to transfer Energy Cost through Xn (Option-3a) is proposed, and finally got consensus on the following opinions:

- Introduce the metric of Energy Cost (EC) as the AI/ML metric to be shared over the Xn interface among gNBs.

- Adopt the below Option-3a and exchange Energy Cost (EC) upon request over the Xn interface.

- The metric of Energy Cost (EC) exchanged between NG-RAN nodes can be an inferred energy consumption related to an additional load or an actual energy consumption value from a neighboring node for either additional load or current load (The details to be further discussed). EC is a value at gNB level.

The details on how to define the additional load or current load needs further discussion.

During the last RAN3 #120 meeting, the following agreements and FFS were captured:

EC is represented as an index, which should be normalized and defined by OAM. The index value could be encoded as an integer from 0 to a maximum. The maximum value should guarantee enough accuracy.

FFS on whether the inferred EC should be introduced in R18.

FFS on whether the additional load needs to be introduced in R18.

In this contribution, we provide further considerations on the inferred EC and additional load, and updating the corresponding TP.

2	Discussion

In #120 meeting, RAN3 reached consensus that EC is represented as an index, which should be normalized and defined by OAM. The index value could be encoded as an integer from 0 to a maximum. The maximum value should guarantee enough accuracy.

Proposal 1: The type and scope of Energy Cost IE can be set as INTEGER (0.. 1000, ...) in AI/ML INFORMATION UPDATE message, which means the maximum index value can be at least 1000, and extendible as required.

In meeting#119, we get consensus that both inferred and measured Energy Consumption can be supported. So we need to indicate these two kinds of Energy Consumption differently. The current “Report Characteristics IE” is easy to be extended to indicate them. We suggest to use the eighth bit to indicate the Actual Energy Cost, and use the ninth bit to indicate the Inferred Energy Cost.

Proposal 2: Use eighth bit of Report Characteristics IE to indicate “Actual Energy Consumption”, use ninth bit of Report Characteristics IE to indicate “Inferred Energy Consumption” in AI/ML INFORMATION REQUEST message.

Proposal 3: Use first bit of Measurement Failed Report Characteristics IE in each Node Measurement Initiation Result Item IE to indicate “Actual Energy Consumption” failure, use second bit of Measurement Failed Report Characteristics IE in each Node Measurement Initiation Result Item IE to indicate “Inferred Energy Consumption” failure in AI/ML INFORMATION RESPONSE message.

Proposal 4: Introduce two different IE: Actual Energy Cost IE and Inferred Energy Cost  IE in AI/ML INFORMATION UPDATE message, to allow the target NG-RAN node to report the actual or inferred Energy Cost separately or at the same time.

The above proposals will be presented into TP to 38.423.

However, the details of how to infer the Energy Consumption on target node, whether infer it based on additional load? how to define the addition load need further discussion.

Observation 1: There are two scenarios that request the Actual EC. The source node can request the current actual energy consumption of the target node, to help on AI/ML inference and make HO decision before HO. Also the source node can request the current actual energy consumption of the target node after HO, as the feedback for AI/ML training.

Observation 2: There two scenarios that request the Inferred EC. The source node can request the inferred energy consumption of the target node, assuming additional traffic offloaded to the target node, to help on AI/ML inference and make HO decision; The source node can also request the inferred energy consumption of the target node in a specific future time, to help on AI/ML inference and make HO decision.

Proposal 5: The source NG-RAN node could carry the load metric for the additional load in the request message to target NG-RAN node, if it requests an Inferred Energy Consumption value to serve the additional load.

Proposal 6: The source NG-RAN node could also carry the time information in the request message to target NG-RAN node, if it requests an Inferred Energy Consumption value in the give future.

And the metric of load and Energy Consumption (EC) is not confirmed yet. In previous meeting, the following information options are suggested to support for the definition of “Additional Load”:

-	Number of RRC connections to be offloaded,

-	Number of Active UEs to be offloaded

-	PRB load to be offloaded (the definition needs to be discussed further)

-	Average UL/DL PDCP SDU data volume to be offloaded

-	Target Cell of the offloading action

Among above parameters, “number of active UEs” is the direct way to measure, and it is also a key factor for the NG-RAN node’s energy consumption. It is a easy way to start from “Number of Active UEs” as “Additional Load”.

Proposal 7： It is proposed to use “Number of Active UEs to be offloaded” as base line to describe “Addition Load”. FFS on other parameters.

3	Summary

In this contribution, we provide additional input considerations on the remaining issues on AI/ML for NG-RAN Energy Saving, following observations and proposals are made:

Observation 1: There are two scenarios that request the Actual EC. The source node can request the current actual energy consumption of the target node, to help on AI/ML inference and make HO decision before HO. Also the source node can request the current actual energy consumption of the target node after HO, as the feedback for AI/ML training.

Observation 2: There two scenarios that request the Inferred EC. The source node can request the inferred energy consumption of the target node, assuming additional traffic offloaded to the target node, to help on AI/ML inference and make HO decision; The source node can also request the inferred energy consumption of the target node in a specific future time, to help on AI/ML inference and make HO decision.

Proposal 1: The type and scope of Energy Cost IE can be set as INTEGER (0.. 1000, ...) in AI/ML INFORMATION UPDATE message, which means the maximum index value can be at least 1000, and extendible as required.

Proposal 2: Use eighth bit of Report Characteristics IE to indicate “Actual Energy Consumption”, use ninth bit of Report Characteristics IE to indicate “Inferred Energy Consumption” in AI/ML INFORMATION REQUEST message.

Proposal 3: Use first bit of Measurement Failed Report Characteristics IE in each Node Measurement Initiation Result Item IE to indicate “Actual Energy Consumption” failure, use second bit of Measurement Failed Report Characteristics IE in each Node Measurement Initiation Result Item IE to indicate “Inferred Energy Consumption” failure in AI/ML INFORMATION RESPONSE message.

Proposal 4: Introduce two different IE: Actual Energy Cost IE and Inferred Energy Cost  IE in AI/ML INFORMATION UPDATE message, to allow the target NG-RAN node to report the actual or inferred Energy Cost separately or at the same time.

Proposal 5: The source NG-RAN node could carry the load metric for the additional load in the request message to target NG-RAN node, if it requests an Inferred Energy Consumption value to serve the additional load.

Proposal 6: The source NG-RAN node could also carry the time information in the request message to target NG-RAN node, if it requests an Inferred Energy Consumption value in the give future.

Proposal 7： It is proposed to use “Number of Active UEs to be offloaded” as base line to describe “Addition Load”. FFS on other parameters.

4	Reference

[1] 	3GPP RP-220617 TR 37.817-200, CMCC

[2]       TR 38.423, Xn Application Protocol (XnAP)

Annex: TP to 38.423

### 8.4.AA	AI/ML Information Reporting Initiation (FFS on the name)

#### 8.4.AA.1	General

This procedure is used by an NG-RAN node to request the reporting of AI/ML related information to another NG-RAN node.

The procedure uses non UE-associated signalling.

Editor’s Note: FFS other information that can be requested using this procedure.

Editor’s Note: FFS content of AL/ML related information.

#### 8.4.AA.2	Successful Operation

Figure 8.4.AA.2-1: AI/ML Information Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the AI/ML INFORMATION REQUEST message to NG-RAN node2 to start AI/ML related information reporting and stop AI/ML related information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested AI/ML related information reporting according to the parameters given in the request in case the Registration Request IE is set to “start”; or

-	shall stop all cells AI/ML related information reporting and terminate the reporting in case the Registration Request IE is set to “stop”; or

-	FFS

If the Registration Request IE is set to “start” in the AI/ML INFORMATION REQUEST message and the Report Characteristics IE indicates cell specific AI/ML related information reporting, the Cell To Report List IE shall be included.

If NG-RAN node2 is capable to provide all of the requested information, it shall initiate the AI/ML related information reporting as requested by NG-RAN node1 and respond with the AI/ML INFORMATION RESPONSE message.

If NG-RAN node2 is capable to provide some but not all of the requested information, it shall initiate the AI/ML related information reporting for the admitted requested information and include the Node Measurement Initiation Result IE or the Per Cell Measurement Initiation Result IE or both in the AI/ML INFORMATION RESPONSE message.

If the Reporting Periodicity IE in the AI/ML INFORMATION REQUEST is present, this indicates the periodicity for the reporting of periodic AI/ML related information. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

If the Requested Prediction Time IE in the AI/ML INFORMATION REQUEST message is present, it indicates the specific point in time to which the prediction of the information requested applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the AI/ML INFORMATION REQUEST indicates the type of objects NG-RAN node2 shall perform measurements or prediction on. NG-RAN node2 shall include in the AI/ML INFORMATION UPDATE message:

-	the Predicted Radio Resource Status IE, if the first bit, “Predicted Radio Resource Status” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”. FFS on the details of Predicted Radio Resource Status IE.

-	the Predicted Number of Active UEs IE, if the second bit, “Predicted Number of Active UEs” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”;

-	the Predicted RRC Connections IE, if the third bit, “Predicted RRC Connections” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

-	the Average UE Throughput DL IE, if the fourth  bit, “Average UE Throughput DL” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

-	the Average UE Throughput UL IE, if the fifth bit, “Average UE Throughput UL” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

-	the Average Packet Delay IE, if the sixth bit, “Average Packet Delay” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

-	the Average Packet Loss IE, if the seventh bit, “Average Packet Loss” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

-   the Actual Energy Cost IE, if the eight bit, “Actual Energy Cost” of the Report Characteristics IE included in the AI/ML INOFRMATION REQUEST message is set to “1”.

-   the Inferred Energy Cost IE, if the ninth bit, “Inferred Energy Cost” of the Report Characteristics IE included in the AI/ML INOFRMATION REQUEST message is set to “1”.

#### 8.4.AA.3	Unsuccessful Operation

Figure 8.4.AA.3-1: AI/ML Information Reporting Initiation, unsuccessful operation

If none of the requested AI/ML related information reporting cannot be initiated, NG-RAN node2 shall send the AI/ML INFORMATION FAILURE message with an appropriate cause value.

#### 8.4.AA.4	Abnormal Conditions

For the same Measurement ID, if the initiating NG-RAN node1 does not receive either the AI/ML INFORMATION RESPONSE message or the AI/ML INFORMATION FAILURE message, the NG-RAN node1 may reinitiate the AI/ML Information Reporting Initiation procedure towards the same NG-RAN node, provided that the content of the new AI/ML INFORMATION REQUEST message is identical to the content of the previously unacknowledged AI/ML INFORMATION REQUEST message.

If the NG-RAN node2 receives an AI/ML INFORMATION REQUEST message which includes the Registration Request IE set to “stop” and if the NG-RAN node2 Measurement ID value received in the AI/ML INFORMATION REQUEST message is not used, the NG-RAN node2 shall initiate AI/ML INFORMATION FAILURE message with an appropriate cause value.

If the Report Characteristics IE bitmap is set to “0” (all bits are set to “0”) in the AI/ML INFORMATION REQUEST message then NG-RAN node2 shall initiate an AI/ML INFORMATION FAILURE message with an appropriate cause value.

If the NG-RAN node2 receives a AI/ML INFORMATION REQUEST message which includes the Registration Request IE set to “start” and the NG-RAN node1 Measurement ID IE corresponding to an existing on-going AI/ML information reporting, then NG-RAN node2 shall initiate a AI/ML INFORMATION FAILURE message with an appropriate cause value.

9.1.3.CC	AI/ML INFORMATION REQUEST (FFS on the name)

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested AI/ML related information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) (FFS on others) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss Eighth Bit = Actual Energy Cost Ninth Bit = Inferred Energy Cost FFS on the coding | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | FFS |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

9.1.3.DD	AI/ML INFORMATION RESPONSE (FFS on the name)

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested AI/ML related information, for all or part of   the measurement objects included in the reporting is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Node Measurement Initiation Result |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | ignore |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | EACH | ignore |
| >>Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(128)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Actual Energy Cost. Second Bit = Inferred Energy Cost. Other bits shall be ignored by the NG-RAN node1. |  |  |
| >>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Per Cell Measurement Initiation Result |  | 0..1 |  | List of measurement objects that failed to be initiated per cell. | YES | ignore |
| >Per Cell Measurement Initiation Result Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | EACH | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the cell. | – |  |
| >>>Measurement Failure Cause Item |  | 1 .. <maxFailedMeasObjects> |  |  |  |  |
| >>>>Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(128)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss. Other bits shall be ignored by the NG-RAN node1. | – |  |
| >>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxFailedMeasObjects | Maximum number of measurement objects that can fail per measurement. Value is 124. |
| maxFailedMeasPerNode | Maximum number of measurement objects that can fail per node. Value is 124. |

9.1.3.FF	AI/ML INFORMATION UPDATE (FFS on the name)

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested AI/ML related information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell AI/ML Info Result (FFS on the name) |  | 0..1 |  |  | YES | ignore |
| >Cell AI/ML Info Result Item(FFS on the name) |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  |  |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| >>Predicted Number of Active UEs | O |  | 9.2.2.62 |  | –- |  |
| >>Predicted RRC Connections | O |  | 9.2.2.56 |  | – |  |
| UE Associated Info Result |  | 0..1 |  |  |  |  |
| > UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  |  |  |
| >> UE Assistant Identifier (FFS on the name) | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1 |  |  |
| >>UE Performance | O |  | 9.2.3.Y |  |  |  |
| >>UE trajectory | O |  | FFS |  |  |  |
| Actual Energy Cost | O |  | INTEGER (0.. 1000, ...) FFS on maximum value | The node level Actual measured Energy Consumption index. Value 0 indicates the minimum Actual measured Energy Consumption, the maximum value can be at least 1000, and extended as required. It is measured in linear scale. |  |  |
| Inferred Energy Cost | O |  | INTEGER (0.. 1000, ...) FFS on maximum value | The node level Inferred Energy Consumption index. Value 0 indicates the minimum Inferred Energy Consumption, the maximum value can be at least 1000, and extended as required. It is inferred in linear scale. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEReports | Maximum no. UE that can be served by a NG-RAN node. Value is FFS. |