3GPP TSG-RAN WG3 Meeting RAN3#121	R3-234475

Toulouse, FR, Aug 21 – Aug 25 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1917 | rev | - | Current version: | 15.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on presence of the QoE Area Scope IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, China Telecom, China Unicom, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_QMC_Streaming-Core |  |  |  |  |  | Date: |  |  | 2023-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The presence of the Area Scope IE in NR QoE is optional. Meanwhile, the presence of the Area Scope IE in LTE QoE is mandatory and SA5 does not modify the basic definition of QoE area scope during recent years. The reason why the Area Scope IE of QoE shall be optional is that the target node does not need to receive the area scope from source node during the handover if the target node has already received it, which is the case, for management based QoE. To fix the misalignment of the description for QoE area scope in SA5 spec(e.g. TS28.405) and RAN3 spec(TS36.413, TS36.423), and to sync up the presence of Area Scope IE in LTE QoE and NR QoE, the presence of the Area Scope IE for LTE QoE shall be corrected from M to O. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the presence of the Area Scope IE from M to O. Impact analysis: Impact assessment towards the previous version of the specification (same release): This CR has an impact under protocol point of view. The impact can be considered isolated because the change only affects QoE. The CR is non-backwards compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The presence of Area Scope IE for LTE QoE in Rel-15 does not align the presence of this IE in NR specifications. The TS36.413 can not fully reflect QoE handover mechanism and SA5 definition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.128, 9.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==== Start of Changes ====

9.2.1.128	UE Application layer measurement configuration

The IE defines configuration information for the QoE Measurement Collection (QMC) function.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Container for application layer measurement configuration | M |  | Octet string (1..1000) | Indicates application layer measurement configuration, see Annex L in [43]. | - | - |
| CHOICE Area Scope of QMC | O |  |  |  | - | - |
| >Cell based |  |  |  |  |  | - |
| >>Cell ID List for QMC |  | 1 .. <maxnoofCellIDforQMC> |  |  |  | - |
| >>>E-CGI | M |  | 9.2.1.38 |  | - | - |
| >TA based |  |  |  |  |  | - |
| >>TA List for QMC |  | 1 .. <maxnoofTAforQMC> |  |  |  | - |
| >>>TAC | M |  | 9.2.3.7 | The TAI is derived using the current serving PLMN. | - | - |
| >TAI based |  |  |  |  | - | - |
| >>TAI List for QMC |  | 1 .. <maxnoofTAforQMC> |  |  | - | - |
| >>>TAI | M |  | 9.2.3.16 |  | - | - |
| >PLMN area based |  |  |  |  |  | - |
| >>PLMN List for QMC |  | 1 .. <maxnoofPLMNforQMC> |  |  |  | - |
| >>>PLMN Identity | M |  | 9.2.3.8 |  | - | - |
| Service Type | M |  | ENUMERATED (QMC for streaming service, QMC for MTSI service, ...) | This IE indicates the service type of UE application layer measurements. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforQMC | Maximum no. of Cell ID subject for QMC scope. Value is 32. |
| maxnoofTAforQMC | Maximum no. of TA subject for QMC scope. Value is 8. |
| maxnoofPLMNforQMC | Maximum no. of PLMNs in the PLMN list for QMC scope. Value is 16. |

==== Next Change ====

9.3.4	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

==== Skip Unchanged Part ====

UEAppLayerMeasConfig ::= SEQUENCE {

containerForAppLayerMeasConfig			OCTET STRING (SIZE(1..1000)),

areaScopeOfQMC		AreaScopeOfQMC		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UEAppLayerMeasConfig-ExtIEs} } OPTIONAL,

...

}

UEAppLayerMeasConfig-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

{ID id-serviceType	CRITICALITY ignore	EXTENSION ServiceType	PRESENCE optional},

...

}

==== End of Changes ====