3GPP TSG-RAN WG3 Meeting #121	R3-234487

Toulouse, France, 21st – 25th August 2023

Agenda Item:	26.1

Source: 				LG Electronics Inc.

Title:			Remaining open issue in eNPN

Document for:		Discussion and Decision

1.	Introduction

There is a FFS on whether to include the Selected NID IE into the top level of the S-NODE ADDITION REQUEST and S-NODE MODIFICATION REQUEST message for the dual connectivity [1]. In this contribution, we examine this issue and then provide our view on it.

2.	Discussion

It is still FFS on whether to include the Selected NID IE into the S-NODE ADDITION REQUEST and S-NODE MODIFICATION REQUEST message [1].

TS 37.340: … ## 11.1 Roaming and Access Restrictions The principles for conveying roaming and access restriction info for EN-DC are described in TS 36.300 [2]. For MR-DC with 5GC, SCG (re)selection at the SN is based on roaming and access restriction information in SN. If roaming and access restriction information is not available at the SN, the SN shall consider that there is no restriction for SCG (re)selection. Therefore, MN needs to convey the up-to-date roaming and access restriction information to SN via XnAP messages. …

TS 38.423: … If the S-NODE ADDITION REQUEST message contains the Selected PLMN IE, the S-NG-RAN node may use it for RRM purposes. If the S-NODE ADDITION REQUEST message contains the Expected UE Behaviour IE, the S-NG-RAN node shall, if supported, store this information and may use it to optimize resource allocation. If the S-NODE ADDITION REQUEST message contains the Mobility Restriction List IE, the S-NG-RAN node, if supported, shall store this information and use it to select an appropriate SCG. …

According to the description in the TS 38.423 and TS 37.340, the SN can properly select the PLMN for the SCG based on the Mobility Restriction List IE. We think that same principle can be applied to the equivalent SNPN case. That is, since the equivalent SNPN list and the serving SNPN ID are already included into the Mobility Restriction List IE, the SN itself can properly determine the SNPN ID for the SCG. For the Selected PLMN IE, our understanding is that for the RAN sharing case, this IE is firstly introduced to signal the serving PLMN to the SN in LTE-DC. However, we are not sure whether this can be applied to the equivalent SNPN case. For now, therefore, we propose not to include the Selected NID IE into the top level of the S-NODE ADDITION REQUEST and S-NODE MODIFICATION REQUEST message at this time.

Proposal: There is no need to include the Selected NID IE into the top level of the S-NODE ADDITION REQUEST and S-NODE MODIFICATION REQUEST message.

3.	Conclusion

In this contribution, we focused on the open issue of the Rel-18 eNPN and provided our view on it. The following proposal is kindly suggested to RAN3:

Proposal: There is no need to include the Selected NID IE into the top level of the S-NODE ADDITION REQUEST and S-NODE MODIFICATION REQUEST message.

4.	Reference

- R3-233351, “Summary of offline discussions on CB: # 28_R18,” Ericsson, May, 2023.