3GPP TSG-RAN WG3 Meeting #121	R3-23xxxx

Toulouse, France, 21st– 25th August, 2023

Agenda item:	13.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	(TP for NR_mobile_IAB BL CR for TS 38.470) New F1 setup

Document Type:	Other

# Introduction

This is to provide TP for TS38.470 to support the new IAB procedures.

# Annex: TP for NR_mobile_IAB BL CR for TS 38.470

START OF CHANGES

### 5.2.12	IAB support function

The support for IAB comprises several functions.

The BAP mapping configuration function allows the IAB-donor-CU to provide BAP mapping which includes the backhaul routing configuration and/or BH RLC channel mapping information for IAB-donor-DU or IAB-DU. This function also enables the IAB-donor-CU to provide the BAP header rewriting configuration, or other BAP related configurations to the IAB-DU. This function also enables the IAB-donor-CU to provide the BAP related configurations to the IAB-donor-DU.

The gNB-DU resource configuration function is used by the IAB-donor-CU to provide cell resource configuration for an IAB-donor-DU or an IAB-DU, and/or NA resource configuration of a parent node IAB-DU or IAB-donor-DU serving the co-located IAB-MT for an IAB-DU, and/or information about the child node’s cell resource configuration and other periodic configurations to a parent IAB-node or an IAB-donor-DU. This function also allows the IAB-donor-CU to provide the semi-static cell resource configuration of a neighbour node or a peer parent-node of a child node, whereas this neighbor node or a peer parent can be an IAB-donor-DU or an IAB-DU.

The IAB TNL address configuration function enables the IAB-donor-CU to request IP address(es) to be used for IAB-node(s) from an IAB-donor-DU, or to request from an IAB-donor-DU the removal of IP address(es) used for IAB-node(s). This function is also used by the IAB-donor-CU to provide an IAB-donor-DU with the IP address information of the traffic to be transferred to a peer IAB-donor-DU via an inter-donor-DU tunnel.

The IAB UP configuration update function allows the update of BH information or the UP TNL information between the IAB-donor-CU and an IAB-DU.

The IAB New F1 Setup Trigger function enables the IAB-donor-CU to trigger the new F1 interface establishment between target F1-terminating IAB-donor-CU and target logical IAB-DU.

The IAB New F1 Setup Notify function enables the IAB-DU to report the outcome of the new F1 interface setup between the target IAB-donor-CU and target logical IAB-DU.

NEXT CHANGE

### 6.1.12	IAB procedures

The IAB procedures are listed below:

-	BAP Mapping Configuration procedure

-	gNB-DU Resource Configuration procedure

-	IAB TNL Address Allocation procedure

-	IAB UP Configuration Update procedure

-	New F1 Setup Trigger procedure

-	New F1 Setup Notify procedure

END OF CHANGES