3GPP TSG-RAN WG3 Meeting #121	R3-234594

Toulouse, France, 21st– 25th August, 2023

Agenda item:	13.2

Source: 	Huawei

Title: 	(TP for NR_mobile_IAB BL CR for TS 38.473) New F1 setup

Document Type:	Other

# Introduction

This is to provide the TP to reflect the online agreements, according to the CB:

CB: # MobileIAB

- Agree to New TP to TS38.401 R3-234592

- Agree to TP to NGAP in R3-234593, revision of R3-23 3967

- R3-234141 is revised in R3-234594. Can the TP to TS38.473 to introduce two new class 2 procedures in R3-234594 be agreed?

- New TP to TS38.473 in R3-234595 to introduce the gNB-ID of the MT CU in F1 message. Can R3-234595 be agreed?

(moderator - QC)

# Annex: TP for NR_mobile_IAB BL CR for TS 38.473

START OF CHANGES

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

UNCHANGED PART OMITTED

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| DU-CU Radio Information Transfer | DU-CU RADIO INFORMATION TRANSFER |
| CU-DU Radio Information Transfer | CU-DU RADIO INFORMATION TRANSFER |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Reference Time Information Reporting Control | REFERENCE TIME INFORMATION REPORTING CONTROL |
| Reference Time Information Report | REFERENCE TIME INFORMATION REPORT |
| Access Success | ACCESS SUCCESS |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Positioning Assistance Information Control | POSITIONING ASSISTANCE INFORMATION CONTROL |
| Positioning Assistance Information Feedback | POSITIONING ASSISTANCE INFORMATION FEEDBACK |
| Positioning Measurement Report | POSITIONING MEASUREMENT REPORT |
| Positioning Measurement Abort | POSITIONING MEASUREMENT ABORT |
| Positioning Measurement Failure Indication | POSITIONING MEASUREMENT FAILURE INDICATION |
| Positioning Measurement Update | POSITIONING MEASUREMENT UPDATE |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Context Release Request | BROADCAST CONTEXT RELEASE REQUEST |
| Multicast Context Release Request | MULTICAST CONTEXT RELEASE REQUEST |
| PDC Measurement Report | PDC MEASUREMENT REPORT |
| PDC Measurement Termination | PDC MEASUREMENT TERMINATION COMMAND |
| PDC Measurement Failure Indication | PDC MEASUREMENT FAILURE INDICATION |
| PDC Measurement Termination | PDC MEASUREMENT TERMINATION COMMAND |
| Measurement Activation | MEASUREMENT ACTIVATION |
| QoE Information Transfer | QOE INFORMATION TRANSFER |
| Positioning System Information Delivery | POSITIONING SYSTEM INFORMATION DELIVERY COMMAND |
| Mobile IAB F1 Setup Triggering | MIAB F1 SETUP TRIGGERING |
| Mobile IAB F1 Setup Outcome Notification | MIAB F1 SETUP OUTCOME NOTIFICATION |

NEXT CHANGE

## 8.10	IAB Procedures

### 8.10.X	Mobile IAB F1 Setup Triggering

#### 8.10.X.1	General

The purpose of the Mobile IAB F1 Setup Triggering procedure is to trigger F1 interface establishment between a target logical gNB-DU and a target F1-terminating IAB-donor-CU. The target logical gNB-DU is co-located with the gNB-DU that receives the triggering message. Based on the triggering message, the target logical gNB-DU sets up an F1 interface towards a target F1-terminating IAB-donor-CU, for supporting mobile IAB-DU migration. This procedure uses non-UE associated signalling.

NOTE:	This procedure is applicable for mobile IAB-nodes, where the term "gNB-DU" applies to a mobile IAB-DU, and the term "gNB-CU" applies to a source F1-terminating IAB-donor-CU during mobile IAB-DU migration.

#### 8.10.X.2	Successful Operation

Figure 8.10.X.2-1: Mobile IAB F1 Setup Triggering: Successful Operation

The gNB-CU initiates the procedure by sending the MIAB F1 SETUP TRIGGERING message to the gNB-DU. The gNB-DU initiates the TNL connection establishment and F1 setup to a target F1-terminating IAB-donor-CU indicated by the Target gNB ID, which is included in the MIAB F1 SETUP TRIGGERING message.

If the MIAB F1 SETUP TRIGGERING message contains the Target gNB IP address IE, the gNB-DU shall store the IP address and use it for establishing the SCTP association(s) and for setting up F1 interface towards a target F1-terminating IAB-donor-CU.

If the MIAB F1 SETUP TRIGGERING message contains the Target SeGW IP address IE, the gNB-DU shall store the IP address and use it for establishing the security connection to protect the F1 interface towards the target F1-terminating IAB-donor-CU.

#### 8.10.X.3	Abnormal Conditions

Not applicable.

NEXT CHANGE

### 8.10.Y	Mobile IAB F1 Setup Outcome Notification

#### 8.10.Y.1	General

The purpose of the Mobile IAB F1 Setup Outcome Notification procedure is to report the outcome of the F1 interface setup between a target logical gNB-DU and a target F1-terminating IAB-donor-CU. The target logical gNB-DU is co-located with the gNB-DU that sends the notification message, and can be used to set up another F1 interface for supporting mobile IAB-DU migration. This procedure uses non-UE associated signalling.

NOTE:	This procedure is applicable for mobile IAB-nodes, where the term "gNB-DU" applies to mobile IAB-DU, and the term "gNB-CU" applies to source F1-terminating IAB-donor-CU during mIAB-DU migration.

#### 8.10.Y.2	Successful Operation

Figure 8.10.X.2-1: Mobile IAB F1 Setup Outcome Notification: Successful Operation

The gNB-DU initiates the procedure by sending the MIAB F1 SETUP OUTCOME NOTIFICATION message to the gNB-CU. After the target logical gNB-DU co-located with the gNB-DU has performed the F1 setup procedure with the target F1-terminating IAB-donor-CU, the gNB-DU sends the MIAB F1 SETUP OUTCOME NOTIFICATION message to the gNB-CU, to report the outcome of the F1 setup towards the target F1-terminating IAB-donor-CU.

If the Activated Cells Mapping List is included in the MIAB F1 SETUP OUTCOME NOTIFICATION message, the gNB-CU shall, if supported, take it into account when further performing handover of the connected UEs from this gNB-DU to its co-located target logical gNB-DU.

#### 8.10.Y.3	Abnormal Conditions

Not applicable.

NEXT CHANGE

### 9.2.9	IAB messages

UNCHANGED PART OMITTED

#### 9.2.9.X1	MIAB F1 SETUP TRIGGERING

This message is sent by the gNB-CU to trigger F1 interface setup from the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Target gNB ID | M |  | Global gNB ID 9.3.1.Y |  | YES | reject |
| Target gNB IP address | O |  | Transport Layer Address 9.3.2.3 |  | YES | ignore |
| Target SeGW IP address | O |  | Transport Layer Address 9.3.2.3 |  | YES | ignore |

#### 9.2.9.X2	MIAB F1 SETUP OUTCOME NOTIFICATION

This message is sent by the gNB-DU to notify the gNB-CU about the outcome of F1 interface setup between ithe gNB-DU’s co-located target logical gNB-DU and a target F1-terminating IAB-donor-CU.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| F1 Setup Outcome | M |  | ENUMERATED (success, failure, …) |  | YES | reject |
| Activated Cells Mapping List |  | 0..1 |  |  | YES | ignore |
| >Activated Cells List Mapping Item IEs |  | 1.. <maxCellingNBDU> |  | List of activated cells | EACH | ignore |
| >> NR CGI For Target Logical GNB-DU | M |  | NR CGI 9.3.1.12 | The identity of an activated cell belonging to the target logical gNB-DU of the mobile IAB-node | - |  |
| >> NR CGI For Source Logical GNB-DU | M |  | NR CGI 9.3.1.12 | The identity of an activated cell belonging to the source logical gNB-DU of the mobile IAB-node | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

NEXT CHANGE

### 9.3.1	Radio Network Layer Related IEs

#### 9.3.1.Y	Global gNB ID

This IE is used to globally identify a gNB (see TS 38.300 [6]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.3.1.14 |  |
| CHOICE gNB ID | M |  |  |  |
| >gNB ID |  |  |  |  |
| >>gNB ID | M |  | BIT STRING (SIZE(22..32)) | Equal to the leftmost bits of the NR Cell Identity IE contained in the NR CGI IE of each cell served by the gNB. |

NEXT CHANGE

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

F1AP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM F1AP-CommonDataTypes

Reset,

ResetAcknowledge,

F1SetupRequest,

F1SetupResponse,

F1SetupFailure,

GNBDUConfigurationUpdate,

GNBDUConfigurationUpdateAcknowledge,

GNBDUConfigurationUpdateFailure,

GNBCUConfigurationUpdate,

GNBCUConfigurationUpdateAcknowledge,

GNBCUConfigurationUpdateFailure,

UEContextSetupRequest,

UEContextSetupResponse,

UEContextSetupFailure,

UEContextReleaseCommand,

UEContextReleaseComplete,

UEContextModificationRequest,

UEContextModificationResponse,

UEContextModificationFailure,

UEContextModificationRequired,

UEContextModificationConfirm,

ErrorIndication,

UEContextReleaseRequest,

DLRRCMessageTransfer,

ULRRCMessageTransfer,

GNBDUResourceCoordinationRequest,

GNBDUResourceCoordinationResponse,

PrivateMessage,

UEInactivityNotification,

InitialULRRCMessageTransfer,

SystemInformationDeliveryCommand,

Paging,

Notify,

WriteReplaceWarningRequest,

WriteReplaceWarningResponse,

PWSCancelRequest,

PWSCancelResponse,

PWSRestartIndication,

PWSFailureIndication,

GNBDUStatusIndication,

RRCDeliveryReport,

UEContextModificationRefuse,

F1RemovalRequest,

F1RemovalResponse,

F1RemovalFailure,

NetworkAccessRateReduction,

TraceStart,

DeactivateTrace,

DUCURadioInformationTransfer,

CUDURadioInformationTransfer,

BAPMappingConfiguration,

BAPMappingConfigurationAcknowledge,

BAPMappingConfigurationFailure,

GNBDUResourceConfiguration,

GNBDUResourceConfigurationAcknowledge,

GNBDUResourceConfigurationFailure,

IABTNLAddressRequest,

IABTNLAddressResponse,

IABTNLAddressFailure,

IABUPConfigurationUpdateRequest,

IABUPConfigurationUpdateResponse,

IABUPConfigurationUpdateFailure,

ResourceStatusRequest,

ResourceStatusResponse,

ResourceStatusFailure,

ResourceStatusUpdate,

AccessAndMobilityIndication,

ReferenceTimeInformationReportingControl,

ReferenceTimeInformationReport,

AccessSuccess,

CellTrafficTrace,

PositioningMeasurementRequest,

PositioningMeasurementResponse,

PositioningMeasurementFailure,

PositioningAssistanceInformationControl,

PositioningAssistanceInformationFeedback,

PositioningMeasurementReport,

PositioningMeasurementAbort,

PositioningMeasurementFailureIndication,

PositioningMeasurementUpdate,

TRPInformationRequest,

TRPInformationResponse,

TRPInformationFailure,

PositioningInformationRequest,

PositioningInformationResponse,

PositioningInformationFailure,

PositioningActivationRequest,

PositioningActivationResponse,

PositioningActivationFailure,

PositioningDeactivation,

PositioningInformationUpdate,

E-CIDMeasurementInitiationRequest,

E-CIDMeasurementInitiationResponse,

E-CIDMeasurementInitiationFailure,

E-CIDMeasurementFailureIndication,

E-CIDMeasurementReport,

E-CIDMeasurementTerminationCommand,

BroadcastContextSetupRequest,

BroadcastContextSetupResponse,

BroadcastContextSetupFailure,

BroadcastContextReleaseCommand,

BroadcastContextReleaseComplete,

BroadcastContextReleaseRequest,

BroadcastContextModificationRequest,

BroadcastContextModificationResponse,

BroadcastContextModificationFailure,

MulticastGroupPaging,

MulticastContextSetupRequest,

MulticastContextSetupResponse,

MulticastContextSetupFailure,

MulticastContextReleaseCommand,

MulticastContextReleaseComplete,

MulticastContextReleaseRequest,

MulticastContextModificationRequest,

MulticastContextModificationResponse,

MulticastContextModificationFailure,

MulticastDistributionSetupRequest,

MulticastDistributionSetupResponse,

MulticastDistributionSetupFailure,

MulticastDistributionReleaseCommand,

MulticastDistributionReleaseComplete,

PDCMeasurementInitiationRequest,

PDCMeasurementInitiationResponse,

PDCMeasurementInitiationFailure,

PDCMeasurementReport,

PDCMeasurementTerminationCommand,

PDCMeasurementFailureIndication,

PRSConfigurationRequest,

PRSConfigurationResponse,

PRSConfigurationFailure,

MeasurementPreconfigurationRequired,

MeasurementPreconfigurationConfirm,

MeasurementPreconfigurationRefuse,

MeasurementActivation,

QoEInformationTransfer,

PosSystemInformationDeliveryCommand,

MIABF1SetupTriggering,

MIABF1SetupOutcomeNotification

FROM F1AP-PDU-Contents

id-Reset,

id-F1Setup,

id-gNBDUConfigurationUpdate,

id-gNBCUConfigurationUpdate,

id-UEContextSetup,

id-UEContextRelease,

id-UEContextModification,

id-UEContextModificationRequired,

id-ErrorIndication,

id-UEContextReleaseRequest,

id-DLRRCMessageTransfer,

id-ULRRCMessageTransfer,

id-GNBDUResourceCoordination,

id-privateMessage,

id-UEInactivityNotification,

id-InitialULRRCMessageTransfer,

id-SystemInformationDeliveryCommand,

id-Paging,

id-Notify,

id-WriteReplaceWarning,

id-PWSCancel,

id-PWSRestartIndication,

id-PWSFailureIndication,

id-GNBDUStatusIndication,

id-RRCDeliveryReport,

id-F1Removal,

id-NetworkAccessRateReduction,

id-TraceStart,

id-DeactivateTrace,

id-DUCURadioInformationTransfer,

id-CUDURadioInformationTransfer,

id-BAPMappingConfiguration,

id-GNBDUResourceConfiguration,

id-IABTNLAddressAllocation,

id-IABUPConfigurationUpdate,

id-resourceStatusReportingInitiation,

id-resourceStatusReporting,

id-accessAndMobilityIndication,

id-ReferenceTimeInformationReportingControl,

id-ReferenceTimeInformationReport,

id-accessSuccess,

id-cellTrafficTrace,

id-PositioningMeasurementExchange,

id-PositioningAssistanceInformationControl,

id-PositioningAssistanceInformationFeedback,

id-PositioningMeasurementReport,

id-PositioningMeasurementAbort,

id-PositioningMeasurementFailureIndication,

id-PositioningMeasurementUpdate,

id-TRPInformationExchange,

id-PositioningInformationExchange,

id-PositioningActivation,

id-PositioningDeactivation,

id-PositioningInformationUpdate,

id-E-CIDMeasurementInitiation,

id-E-CIDMeasurementFailureIndication,

id-E-CIDMeasurementReport,

id-E-CIDMeasurementTermination,

id-BroadcastContextSetup,

id-BroadcastContextRelease,

id-BroadcastContextReleaseRequest,

id-BroadcastContextModification,

id-MulticastGroupPaging,

id-MulticastContextSetup,

id-MulticastContextRelease,

id-MulticastContextReleaseRequest,

id-MulticastContextModification,

id-MulticastDistributionSetup,

id-MulticastDistributionRelease,

id-PDCMeasurementInitiation,

id-PDCMeasurementInitiationRequest,

id-PDCMeasurementInitiationResponse,

id-PDCMeasurementInitiationFailure,

id-PDCMeasurementTerminationCommand,

id-PDCMeasurementFailureIndication,

id-PDCMeasurementReport,

id-pRSConfigurationExchange,

id-measurementPreconfiguration,

id-measurementActivation,

id-QoEInformationTransfer,

id-PosSystemInformationDeliveryCommand,

id-MIABF1SetupTriggering,

id-MIABF1SetupOutcomeNotification

FROM F1AP-Constants

ProtocolIE-SingleContainer{},

F1AP-PROTOCOL-IES

FROM F1AP-Containers;

-- **************************************************************

--

-- Interface Elementary Procedure Class

--

-- **************************************************************

F1AP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome							OPTIONAL,

&UnsuccessfulOutcome						OPTIONAL,

&procedureCode				ProcedureCode 	UNIQUE,

&criticality				Criticality 	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE			&InitiatingMessage

[SUCCESSFUL OUTCOME			&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE				&procedureCode

[CRITICALITY				&criticality]

}

-- **************************************************************

--

-- Interface PDU Definition

--

-- **************************************************************

F1AP-PDU ::= CHOICE {

initiatingMessage	InitiatingMessage,

successfulOutcome	SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

choice-extension	ProtocolIE-SingleContainer { { F1AP-PDU-ExtIEs} }

}

F1AP-PDU-ExtIEs F1AP-PROTOCOL-IES ::= { -- this extension is not used

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	F1AP-ELEMENTARY-PROCEDURE.&procedureCode		({F1AP-ELEMENTARY-PROCEDURES}),

criticality		F1AP-ELEMENTARY-PROCEDURE.&criticality			({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			F1AP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	F1AP-ELEMENTARY-PROCEDURE.&procedureCode		({F1AP-ELEMENTARY-PROCEDURES}),

criticality		F1AP-ELEMENTARY-PROCEDURE.&criticality			({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			F1AP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	F1AP-ELEMENTARY-PROCEDURE.&procedureCode		({F1AP-ELEMENTARY-PROCEDURES}),

criticality		F1AP-ELEMENTARY-PROCEDURE.&criticality			({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			F1AP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

F1AP-ELEMENTARY-PROCEDURES F1AP-ELEMENTARY-PROCEDURE ::= {

F1AP-ELEMENTARY-PROCEDURES-CLASS-1			|

F1AP-ELEMENTARY-PROCEDURES-CLASS-2,

...

}

F1AP-ELEMENTARY-PROCEDURES-CLASS-1 F1AP-ELEMENTARY-PROCEDURE ::= {

reset							|

f1Setup							|

gNBDUConfigurationUpdate		|

gNBCUConfigurationUpdate		|

uEContextSetup					|

uEContextRelease				|

uEContextModification			|

uEContextModificationRequired	|

writeReplaceWarning				|

pWSCancel					|

gNBDUResourceCoordination		|

f1Removal						|

bAPMappingConfiguration			|

gNBDUResourceConfiguration		|

iABTNLAddressAllocation			|

iABUPConfigurationUpdate		|

resourceStatusReportingInitiation	|

positioningMeasurementExchange	|

tRPInformationExchange			|

positioningInformationExchange	|

positioningActivation			|

e-CIDMeasurementInitiation		|

broadcastContextSetup			|

broadcastContextRelease			|

broadcastContextModification|

multicastContextSetup			|

multicastContextRelease		|

multicastContextModification	|

multicastDistributionSetup		|

multicastDistributionRelease	|

pDCMeasurementInitiation		|

pRSConfigurationExchange		|

measurementPreconfiguration	,

...

}

F1AP-ELEMENTARY-PROCEDURES-CLASS-2 F1AP-ELEMENTARY-PROCEDURE ::= {

errorIndication							|

uEContextReleaseRequest					|

dLRRCMessageTransfer					|

uLRRCMessageTransfer					|

uEInactivityNotification				|

privateMessage							|

initialULRRCMessageTransfer				|

systemInformationDelivery				|

paging									|

notify									|

pWSRestartIndication					|

pWSFailureIndication					|

gNBDUStatusIndication					|

rRCDeliveryReport						|

networkAccessRateReduction				|

traceStart								|

deactivateTrace							|

dUCURadioInformationTransfer			|

cUDURadioInformationTransfer			|

resourceStatusReporting					|

accessAndMobilityIndication				|

referenceTimeInformationReportingControl|

referenceTimeInformationReport			|

accessSuccess							|

cellTrafficTrace						|

positioningAssistanceInformationControl		|

positioningAssistanceInformationFeedback	|

positioningMeasurementReport				|

positioningMeasurementAbort					|

positioningMeasurementFailureIndication		|

positioningMeasurementUpdate				|

positioningDeactivation						|

e-CIDMeasurementFailureIndication			|

e-CIDMeasurementReport						|

e-CIDMeasurementTermination					|

positioningInformationUpdate				|

multicastGroupPaging						|

broadcastContextReleaseRequest				|

multicastContextReleaseRequest				|

pDCMeasurementReport						|

pDCMeasurementTerminationCommand			|

pDCMeasurementFailureIndication				|

measurementActivation						|

qoEInformationTransfer						|

posSystemInformationDelivery				|

mIABF1SetupTriggering						|

mIABF1SetupOutcomeNotification				,

...

}

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

reset F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Reset

SUCCESSFUL OUTCOME		ResetAcknowledge

PROCEDURE CODE			id-Reset

CRITICALITY				reject

}

f1Setup F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		F1SetupRequest

SUCCESSFUL OUTCOME		F1SetupResponse

UNSUCCESSFUL OUTCOME	F1SetupFailure

PROCEDURE CODE			id-F1Setup

CRITICALITY				reject

}

gNBDUConfigurationUpdate F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNBDUConfigurationUpdate

SUCCESSFUL OUTCOME		GNBDUConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	GNBDUConfigurationUpdateFailure

PROCEDURE CODE			id-gNBDUConfigurationUpdate

CRITICALITY				reject

}

gNBCUConfigurationUpdate F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNBCUConfigurationUpdate

SUCCESSFUL OUTCOME		GNBCUConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	GNBCUConfigurationUpdateFailure

PROCEDURE CODE			id-gNBCUConfigurationUpdate

CRITICALITY				reject

}

uEContextSetup F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextSetupRequest

SUCCESSFUL OUTCOME		UEContextSetupResponse

UNSUCCESSFUL OUTCOME	UEContextSetupFailure

PROCEDURE CODE			id-UEContextSetup

CRITICALITY				reject

}

uEContextRelease F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextReleaseCommand

SUCCESSFUL OUTCOME		UEContextReleaseComplete

PROCEDURE CODE			id-UEContextRelease

CRITICALITY				reject

}

uEContextModification F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextModificationRequest

SUCCESSFUL OUTCOME		UEContextModificationResponse

UNSUCCESSFUL OUTCOME	UEContextModificationFailure

PROCEDURE CODE			id-UEContextModification

CRITICALITY				reject

}

uEContextModificationRequired F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextModificationRequired

SUCCESSFUL OUTCOME		UEContextModificationConfirm

UNSUCCESSFUL OUTCOME	UEContextModificationRefuse

PROCEDURE CODE			id-UEContextModificationRequired

CRITICALITY				reject

}

writeReplaceWarning F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		WriteReplaceWarningRequest

SUCCESSFUL OUTCOME		WriteReplaceWarningResponse

PROCEDURE CODE			id-WriteReplaceWarning

CRITICALITY				reject

}

pWSCancel F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PWSCancelRequest

SUCCESSFUL OUTCOME		PWSCancelResponse

PROCEDURE CODE			id-PWSCancel

CRITICALITY				reject

}

errorIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-ErrorIndication

CRITICALITY				ignore

}

uEContextReleaseRequest F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextReleaseRequest

PROCEDURE CODE			id-UEContextReleaseRequest

CRITICALITY				ignore

}

initialULRRCMessageTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		InitialULRRCMessageTransfer

PROCEDURE CODE			id-InitialULRRCMessageTransfer

CRITICALITY				ignore

}

dLRRCMessageTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DLRRCMessageTransfer

PROCEDURE CODE			id-DLRRCMessageTransfer

CRITICALITY				ignore

}

uLRRCMessageTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ULRRCMessageTransfer

PROCEDURE CODE			id-ULRRCMessageTransfer

CRITICALITY				ignore

}

uEInactivityNotification  F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEInactivityNotification

PROCEDURE CODE			id-UEInactivityNotification

CRITICALITY				ignore

}

gNBDUResourceCoordination F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNBDUResourceCoordinationRequest

SUCCESSFUL OUTCOME		GNBDUResourceCoordinationResponse

PROCEDURE CODE			id-GNBDUResourceCoordination

CRITICALITY				reject

}

privateMessage F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY				ignore

}

systemInformationDelivery F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SystemInformationDeliveryCommand

PROCEDURE CODE			id-SystemInformationDeliveryCommand

CRITICALITY				ignore

}

paging F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Paging

PROCEDURE CODE			id-Paging

CRITICALITY				ignore

}

notify F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Notify

PROCEDURE CODE			id-Notify

CRITICALITY				ignore

}

networkAccessRateReduction F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NetworkAccessRateReduction

PROCEDURE CODE			id-NetworkAccessRateReduction

CRITICALITY				ignore

}

pWSRestartIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PWSRestartIndication

PROCEDURE CODE			id-PWSRestartIndication

CRITICALITY				ignore

}

pWSFailureIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PWSFailureIndication

PROCEDURE CODE			id-PWSFailureIndication

CRITICALITY				ignore

}

gNBDUStatusIndication 	F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNBDUStatusIndication

PROCEDURE CODE			id-GNBDUStatusIndication

CRITICALITY				ignore

}

rRCDeliveryReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RRCDeliveryReport

PROCEDURE CODE			id-RRCDeliveryReport

CRITICALITY				ignore

}

f1Removal F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		F1RemovalRequest

SUCCESSFUL OUTCOME		F1RemovalResponse

UNSUCCESSFUL OUTCOME	F1RemovalFailure

PROCEDURE CODE			id-F1Removal

CRITICALITY				reject

}

traceStart F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TraceStart

PROCEDURE CODE			id-TraceStart

CRITICALITY				ignore

}

deactivateTrace F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DeactivateTrace

PROCEDURE CODE			id-DeactivateTrace

CRITICALITY				ignore

}

dUCURadioInformationTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DUCURadioInformationTransfer

PROCEDURE CODE			id-DUCURadioInformationTransfer

CRITICALITY				ignore

}

cUDURadioInformationTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CUDURadioInformationTransfer

PROCEDURE CODE			id-CUDURadioInformationTransfer

CRITICALITY				ignore

}

bAPMappingConfiguration F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BAPMappingConfiguration

SUCCESSFUL OUTCOME		BAPMappingConfigurationAcknowledge

UNSUCCESSFUL OUTCOME	BAPMappingConfigurationFailure

PROCEDURE CODE			id-BAPMappingConfiguration

CRITICALITY				reject

}

gNBDUResourceConfiguration F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNBDUResourceConfiguration

SUCCESSFUL OUTCOME		GNBDUResourceConfigurationAcknowledge

UNSUCCESSFUL OUTCOME	GNBDUResourceConfigurationFailure

PROCEDURE CODE			id-GNBDUResourceConfiguration

CRITICALITY				reject

}

iABTNLAddressAllocation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		IABTNLAddressRequest

SUCCESSFUL OUTCOME		IABTNLAddressResponse

UNSUCCESSFUL OUTCOME	IABTNLAddressFailure

PROCEDURE CODE			id-IABTNLAddressAllocation

CRITICALITY				reject

}

iABUPConfigurationUpdate F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		IABUPConfigurationUpdateRequest

SUCCESSFUL OUTCOME		IABUPConfigurationUpdateResponse

UNSUCCESSFUL OUTCOME	IABUPConfigurationUpdateFailure

PROCEDURE CODE			id-IABUPConfigurationUpdate

CRITICALITY				reject

}

resourceStatusReportingInitiation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResourceStatusRequest

SUCCESSFUL OUTCOME		ResourceStatusResponse

UNSUCCESSFUL OUTCOME	ResourceStatusFailure

PROCEDURE CODE			id-resourceStatusReportingInitiation

CRITICALITY				reject

}

resourceStatusReporting F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResourceStatusUpdate

PROCEDURE CODE			id-resourceStatusReporting

CRITICALITY				ignore

}

accessAndMobilityIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AccessAndMobilityIndication

PROCEDURE CODE			id-accessAndMobilityIndication

CRITICALITY				ignore

}

referenceTimeInformationReportingControl F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ReferenceTimeInformationReportingControl

PROCEDURE CODE			id-ReferenceTimeInformationReportingControl

CRITICALITY				ignore

}

referenceTimeInformationReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ReferenceTimeInformationReport

PROCEDURE CODE			id-ReferenceTimeInformationReport

CRITICALITY				ignore

}

accessSuccess F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AccessSuccess

PROCEDURE CODE			id-accessSuccess

CRITICALITY				ignore

}

cellTrafficTrace F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CellTrafficTrace

PROCEDURE CODE			id-cellTrafficTrace

CRITICALITY				ignore

}

positioningAssistanceInformationControl F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningAssistanceInformationControl

PROCEDURE CODE			id-PositioningAssistanceInformationControl

CRITICALITY				ignore

}

positioningAssistanceInformationFeedback F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningAssistanceInformationFeedback

PROCEDURE CODE			id-PositioningAssistanceInformationFeedback

CRITICALITY				ignore

}

positioningMeasurementExchange F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningMeasurementRequest

SUCCESSFUL OUTCOME		PositioningMeasurementResponse

UNSUCCESSFUL OUTCOME	PositioningMeasurementFailure

PROCEDURE CODE			id-PositioningMeasurementExchange

CRITICALITY				reject

}

positioningMeasurementReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningMeasurementReport

PROCEDURE CODE			id-PositioningMeasurementReport

CRITICALITY				ignore

}

positioningMeasurementAbort F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningMeasurementAbort

PROCEDURE CODE			id-PositioningMeasurementAbort

CRITICALITY				ignore

}

positioningMeasurementFailureIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningMeasurementFailureIndication

PROCEDURE CODE			id-PositioningMeasurementFailureIndication

CRITICALITY				ignore

}

positioningMeasurementUpdate F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningMeasurementUpdate

PROCEDURE CODE			id-PositioningMeasurementUpdate

CRITICALITY				ignore

}

tRPInformationExchange F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TRPInformationRequest

SUCCESSFUL OUTCOME		TRPInformationResponse

UNSUCCESSFUL OUTCOME	TRPInformationFailure

PROCEDURE CODE			id-TRPInformationExchange

CRITICALITY				reject

}

positioningInformationExchange F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningInformationRequest

SUCCESSFUL OUTCOME		PositioningInformationResponse

UNSUCCESSFUL OUTCOME	PositioningInformationFailure

PROCEDURE CODE			id-PositioningInformationExchange

CRITICALITY				reject

}

positioningActivation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningActivationRequest

SUCCESSFUL OUTCOME		PositioningActivationResponse

UNSUCCESSFUL OUTCOME	PositioningActivationFailure

PROCEDURE CODE			id-PositioningActivation

CRITICALITY				reject

}

positioningDeactivation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningDeactivation

PROCEDURE CODE			id-PositioningDeactivation

CRITICALITY				ignore

}

e-CIDMeasurementInitiation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementInitiationRequest

SUCCESSFUL OUTCOME		E-CIDMeasurementInitiationResponse

UNSUCCESSFUL OUTCOME	E-CIDMeasurementInitiationFailure

PROCEDURE CODE			id-E-CIDMeasurementInitiation

CRITICALITY				reject

}

e-CIDMeasurementFailureIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementFailureIndication

PROCEDURE CODE			id-E-CIDMeasurementFailureIndication

CRITICALITY				ignore

}

e-CIDMeasurementReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementReport

PROCEDURE CODE			id-E-CIDMeasurementReport

CRITICALITY				ignore

}

e-CIDMeasurementTermination F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementTerminationCommand

PROCEDURE CODE			id-E-CIDMeasurementTermination

CRITICALITY				ignore

}

positioningInformationUpdate F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningInformationUpdate

PROCEDURE CODE			id-PositioningInformationUpdate

CRITICALITY				ignore

}

broadcastContextSetup F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastContextSetupRequest

SUCCESSFUL OUTCOME		BroadcastContextSetupResponse

UNSUCCESSFUL OUTCOME	BroadcastContextSetupFailure

PROCEDURE CODE			id-BroadcastContextSetup

CRITICALITY				reject

}

broadcastContextRelease F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastContextReleaseCommand

SUCCESSFUL OUTCOME		BroadcastContextReleaseComplete

PROCEDURE CODE			id-BroadcastContextRelease

CRITICALITY				reject

}

broadcastContextReleaseRequest F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastContextReleaseRequest

PROCEDURE CODE			id-BroadcastContextReleaseRequest

CRITICALITY				reject

}

broadcastContextModification F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastContextModificationRequest

SUCCESSFUL OUTCOME		BroadcastContextModificationResponse

UNSUCCESSFUL OUTCOME	BroadcastContextModificationFailure

PROCEDURE CODE			id-BroadcastContextModification

CRITICALITY				reject

}

multicastGroupPaging F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastGroupPaging

PROCEDURE CODE			id-MulticastGroupPaging

CRITICALITY				ignore

}

multicastContextSetup F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastContextSetupRequest

SUCCESSFUL OUTCOME		MulticastContextSetupResponse

UNSUCCESSFUL OUTCOME	MulticastContextSetupFailure

PROCEDURE CODE			id-MulticastContextSetup

CRITICALITY				reject

}

multicastContextRelease F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastContextReleaseCommand

SUCCESSFUL OUTCOME		MulticastContextReleaseComplete

PROCEDURE CODE			id-MulticastContextRelease

CRITICALITY				reject

}

multicastContextReleaseRequest F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastContextReleaseRequest

PROCEDURE CODE			id-MulticastContextReleaseRequest

CRITICALITY				reject

}

multicastContextModification F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastContextModificationRequest

SUCCESSFUL OUTCOME		MulticastContextModificationResponse

UNSUCCESSFUL OUTCOME	MulticastContextModificationFailure

PROCEDURE CODE			id-MulticastContextModification

CRITICALITY				reject

}

multicastDistributionSetup F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastDistributionSetupRequest

SUCCESSFUL OUTCOME		MulticastDistributionSetupResponse

UNSUCCESSFUL OUTCOME	MulticastDistributionSetupFailure

PROCEDURE CODE			id-MulticastDistributionSetup

CRITICALITY				reject

}

multicastDistributionRelease F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastDistributionReleaseCommand

SUCCESSFUL OUTCOME		MulticastDistributionReleaseComplete

PROCEDURE CODE			id-MulticastDistributionRelease

CRITICALITY				reject

}

pDCMeasurementInitiation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDCMeasurementInitiationRequest

SUCCESSFUL OUTCOME		PDCMeasurementInitiationResponse

UNSUCCESSFUL OUTCOME	PDCMeasurementInitiationFailure

PROCEDURE CODE			id-PDCMeasurementInitiation

CRITICALITY				reject

}

pDCMeasurementReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDCMeasurementReport

PROCEDURE CODE			id-PDCMeasurementReport

CRITICALITY				ignore

}

pDCMeasurementTerminationCommand F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDCMeasurementTerminationCommand

PROCEDURE CODE			id-PDCMeasurementTerminationCommand

CRITICALITY				ignore

}

pDCMeasurementFailureIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDCMeasurementFailureIndication

PROCEDURE CODE			id-PDCMeasurementFailureIndication

CRITICALITY				ignore

}

pRSConfigurationExchange F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PRSConfigurationRequest

SUCCESSFUL OUTCOME		PRSConfigurationResponse

UNSUCCESSFUL OUTCOME	PRSConfigurationFailure

PROCEDURE CODE			id-pRSConfigurationExchange

CRITICALITY				reject

}

measurementPreconfiguration F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementPreconfigurationRequired

SUCCESSFUL OUTCOME		MeasurementPreconfigurationConfirm

UNSUCCESSFUL OUTCOME	MeasurementPreconfigurationRefuse

PROCEDURE CODE			id-measurementPreconfiguration

CRITICALITY				reject

}

measurementActivation		F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementActivation

PROCEDURE CODE			id-measurementActivation

CRITICALITY				ignore

}

qoEInformationTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		QoEInformationTransfer

PROCEDURE CODE			id-QoEInformationTransfer

CRITICALITY				ignore

}

posSystemInformationDelivery F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PosSystemInformationDeliveryCommand

PROCEDURE CODE			id-PosSystemInformationDeliveryCommand

CRITICALITY				ignore

}

mIABF1SetupTriggering F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MIABF1SetupTriggering

PROCEDURE CODE			id-MIABF1SetupTriggering

CRITICALITY				ignore

}

mIABF1SetupOutcomeNotification F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MIABF1SetupOutcomeNotification

PROCEDURE CODE			id-MIABF1SetupOutcomeNotification

CRITICALITY				ignore

}

END

-- ASN1STOP

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

BroadcastMRBs-FailedToBeModified-Item,

BroadcastMRBs-FailedToBeSetup-Item,

BroadcastMRBs-FailedToBeSetupMod-Item,

BroadcastMRBs-Modified-Item,

BroadcastMRBs-Setup-Item,

BroadcastMRBs-SetupMod-Item,

BroadcastMRBs-ToBeModified-Item,

BroadcastMRBs-ToBeReleased-Item,

BroadcastMRBs-ToBeSetup-Item,

BroadcastMRBs-ToBeSetupMod-Item,

Candidate-SpCell-Item,

Cause,

Cells-Failed-to-be-Activated-List-Item,

Cells-Status-Item,

Cells-to-be-Activated-List-Item,

Cells-to-be-Deactivated-List-Item,

CellULConfigured,

CriticalityDiagnostics,

C-RNTI,

CUtoDURRCInformation,

DRB-Activity-Item,

DRBID,

DRBs-FailedToBeModified-Item,

DRBs-FailedToBeSetup-Item,

DRBs-FailedToBeSetupMod-Item,

DRB-Notify-Item,

DRBs-ModifiedConf-Item,

DRBs-Modified-Item,

DRBs-Required-ToBeModified-Item,

DRBs-Required-ToBeReleased-Item,

DRBs-Setup-Item,

DRBs-SetupMod-Item,

DRBs-ToBeModified-Item,

DRBs-ToBeReleased-Item,

DRBs-ToBeSetup-Item,

DRBs-ToBeSetupMod-Item,

DRXCycle,

DRXConfigurationIndicator,

DUtoCURRCInformation,

EUTRANQoS,

ExecuteDuplication,

FullConfiguration,

GNB-CU-MBS-F1AP-ID,

GNB-CU-UE-F1AP-ID,

GNB-DU-MBS-F1AP-ID,

GNB-DU-UE-F1AP-ID,

GNB-DU-ID,

GNB-DU-Served-Cells-Item,

GNB-DU-System-Information,

GNB-CU-Name,

GNB-DU-Name,

InactivityMonitoringRequest,

InactivityMonitoringResponse,

LowerLayerPresenceStatusChange,

MBS-Area-Session-ID,

MBS-CUtoDURRCInformation,

MBSMulticastF1UContextDescriptor,

MBS-Session-ID,

MBS-ServiceArea,

MulticastF1UContextReferenceCU,

MulticastF1UContext-ToBeSetup-Item,

MulticastF1UContext-Setup-Item,

MulticastF1UContext-FailedToBeSetup-Item,

MulticastMBSSessionList,

MulticastMRBs-ToBeSetup-Item,

MulticastMRBs-Setup-Item,

MulticastMRBs-FailedToBeSetup-Item,

MulticastMRBs-ToBeSetupMod-Item,

MulticastMRBs-ToBeModified-Item,

MulticastMRBs-ToBeReleased-Item,

MulticastMRBs-SetupMod-Item,

MulticastMRBs-FailedToBeSetupMod-Item,

MulticastMRBs-Modified-Item,

MulticastMRBs-FailedToBeModified-Item,

BroadcastAreaScope,

NotificationControl,

NRCGI,

NRPCI,

UEContextNotRetrievable,

Potential-SpCell-Item,

RAT-FrequencyPriorityInformation,

RequestedSRSTransmissionCharacteristics,

ResourceCoordinationTransferContainer,

RRCContainer,

RRCContainer-RRCSetupComplete,

RRCReconfigurationCompleteIndicator,

SCellIndex,

SCell-ToBeRemoved-Item,

SCell-ToBeSetup-Item,

SCell-ToBeSetupMod-Item,

SCell-FailedtoSetup-Item,

SCell-FailedtoSetupMod-Item,

ServCellIndex,

Served-Cell-Information,

Served-Cells-To-Add-Item,

Served-Cells-To-Delete-Item,

Served-Cells-To-Modify-Item,

ServingCellMO,

SNSSAI,

SRBID,

SRBs-FailedToBeSetup-Item,

SRBs-FailedToBeSetupMod-Item,

SRBs-Required-ToBeReleased-Item,

SRBs-ToBeReleased-Item,

SRBs-ToBeSetup-Item,

SRBs-ToBeSetupMod-Item,

SRBs-Modified-Item,

SRBs-Setup-Item,

SRBs-SetupMod-Item,

TimeToWait,

TransactionID,

TransmissionActionIndicator,

UE-associatedLogicalF1-ConnectionItem,

UEIdentity-List-For-Paging-Item,

DUtoCURRCContainer,

PagingCell-Item,

SItype-List,

UEIdentityIndexValue,

GNB-CU-TNL-Association-Setup-Item,

GNB-CU-TNL-Association-Failed-To-Setup-Item,

GNB-CU-TNL-Association-To-Add-Item,

GNB-CU-TNL-Association-To-Remove-Item,

GNB-CU-TNL-Association-To-Update-Item,

MaskedIMEISV,

PagingDRX,

PagingPriority,

PagingIdentity,

Cells-to-be-Barred-Item,

PWSSystemInformation,

Broadcast-To-Be-Cancelled-Item,

Cells-Broadcast-Cancelled-Item,

NR-CGI-List-For-Restart-Item,

PWS-Failed-NR-CGI-Item,

RepetitionPeriod,

NumberofBroadcastRequest,

Cells-To-Be-Broadcast-Item,

Cells-Broadcast-Completed-Item,

Cancel-all-Warning-Messages-Indicator,

EUTRA-NR-CellResourceCoordinationReq-Container,

EUTRA-NR-CellResourceCoordinationReqAck-Container,

RequestType,

PLMN-Identity,

RLCFailureIndication,

UplinkTxDirectCurrentListInformation,

SULAccessIndication,

Protected-EUTRA-Resources-Item,

GNB-DUConfigurationQuery,

BitRate,

RRC-Version,

GNBDUOverloadInformation,

RRCDeliveryStatusRequest,

NeedforGap,

RRCDeliveryStatus,

ResourceCoordinationTransferInformation,

Dedicated-SIDelivery-NeededUE-Item,

Associated-SCell-Item,

IgnoreResourceCoordinationContainer,

PagingOrigin,

UAC-Assistance-Info,

RANUEID,

GNB-DU-TNL-Association-To-Remove-Item,

NotificationInformation,

TraceActivation,

TraceID,

Neighbour-Cell-Information-Item,

SymbolAllocInSlot,

NumDLULSymbols,

AdditionalRRMPriorityIndex,

DUCURadioInformationType,

CUDURadioInformationType,

Transport-Layer-Address-Info,

BHChannels-ToBeSetup-Item,

BHChannels-Setup-Item,

BHChannels-FailedToBeSetup-Item,

BHChannels-ToBeModified-Item,

BHChannels-ToBeReleased-Item,

BHChannels-ToBeSetupMod-Item,

BHChannels-FailedToBeModified-Item,

BHChannels-FailedToBeSetupMod-Item,

BHChannels-Modified-Item,

BHChannels-SetupMod-Item,

BHChannels-Required-ToBeReleased-Item,

BAPAddress,

BAPPathID,

BAPRoutingID,

BH-Routing-Information-Added-List-Item,

BH-Routing-Information-Removed-List-Item,

Child-Nodes-List,

Child-Nodes-List-Item,

Child-Node-Cells-List,

Child-Node-Cells-List-Item,

Activated-Cells-to-be-Updated-List,

Activated-Cells-to-be-Updated-List-Item,

UL-BH-Non-UP-Traffic-Mapping,

IABTNLAddressesRequested,

IABIPv6RequestType,

IAB-TNL-Addresses-To-Remove-Item,

IABTNLAddress,

IAB-Allocated-TNL-Address-Item,

IABv4AddressesRequested,

TrafficMappingInfo,

UL-UP-TNL-Information-to-Update-List-Item,

UL-UP-TNL-Address-to-Update-List-Item,

DL-UP-TNL-Address-to-Update-List-Item,

NRV2XServicesAuthorized,

LTEV2XServicesAuthorized,

NRUESidelinkAggregateMaximumBitrate,

LTEUESidelinkAggregateMaximumBitrate,

SLDRBs-SetupMod-Item,

SLDRBs-ModifiedConf-Item,

SLDRBID,

SLDRBs-FailedToBeModified-Item,

SLDRBs-FailedToBeSetup-Item,

SLDRBs-FailedToBeSetupMod-Item,

SLDRBs-Modified-Item,

SLDRBs-Required-ToBeModified-Item,

SLDRBs-Required-ToBeReleased-Item,

SLDRBs-Setup-Item,

SLDRBs-ToBeModified-Item,

SLDRBs-ToBeReleased-Item,

SLDRBs-ToBeSetup-Item,

SLDRBs-ToBeSetupMod-Item,

GNBCUMeasurementID,

GNBDUMeasurementID,

RegistrationRequest,

ReportCharacteristics,

CellToReportList,

HardwareLoadIndicator,

CellMeasurementResultList,

ReportingPeriodicity,

TNLCapacityIndicator,

RACHReportInformationList,

RLFReportInformationList,

ReportingRequestType,

TimeReferenceInformation,

ConditionalInterDUMobilityInformation,

ConditionalIntraDUMobilityInformation,

TargetCellList,

MDTPLMNList,

PrivacyIndicator,

TransportLayerAddress,

URI-address,

NID,

PosAssistance-Information,

PosBroadcast,

PositioningBroadcastCells,

RoutingID,

PosAssistanceInformationFailureList,

PosMeasurementQuantities,

PosMeasurementResultList,

PosReportCharacteristics,

TRPInformationTypeItem,

TRPInformationItem,

LMF-MeasurementID,

RAN-MeasurementID,

SDT-Termination-Request,

SRSResourceSetID,

SpatialRelationInfo,

SRSResourceTrigger,

SRSConfiguration,

TRPList,

E-CID-MeasurementQuantities,

MeasurementPeriodicity,

E-CID-MeasurementResult,

Cell-Portion-ID,

LMF-UE-MeasurementID,

RAN-UE-MeasurementID,

RelativeTime1900,

SystemFrameNumber,

SlotNumber,

AbortTransmission,

TRP-MeasurementRequestList,

MeasurementBeamInfoRequest,

E-CID-ReportCharacteristics,

Extended-GNB-CU-Name,

Extended-GNB-DU-Name,

F1CTransferPath,

SCGIndicator,

SpatialRelationPerSRSResource,

MeasurementPeriodicityExtended,

SuccessfulHOReportInformationList,

Coverage-Modification-Notification,

CCO-Assistance-Information,

CellsForSON-List,

IABCongestionIndication,

IABConditionalRRCMessageDeliveryIndication,

F1CTransferPathNRDC,

BufferSizeThresh,

IAB-TNL-Addresses-Exception,

BAP-Header-Rewriting-Added-List-Item,

Re-routingEnableIndicator,

NonF1terminatingTopologyIndicator,

EgressNonF1terminatingTopologyIndicator,

IngressNonF1terminatingTopologyIndicator,

Neighbour-Node-Cells-List,

Neighbour-Node-Cells-List-Item,

NA-Resource-Configuration-List,

NA-Resource-Configuration-Item,

Serving-Cells-List,

Serving-Cells-List-Item,

RBSetConfiguration,

PDCMeasurementPeriodicity,

PDCMeasurementQuantities,

PDCMeasurementResult,

PDCReportType,

RAN-UE-PDC-MeasID,

SCGActivationRequest,

SCGActivationStatus,

TRP-MeasurementUpdateList,

PRSTRPList,

PRSTransmissionTRPList,

ResponseTime,

TRP-PRS-Info-List,

PRS-Measurement-Info-List,

PRSConfigRequestType,

MeasurementCharacteristicsRequestIndicator,

MeasurementTimeOccasion,

UEReportingInformation,

PosConextRevIndication,

NRRedCapUEIndication,

NRPagingeDRXInformation,

NRPagingeDRXInformationforRRCINACTIVE,

QoEInformation,

CG-SDTQueryIndication,

CG-SDTKeptIndicator,

CG-SDTSessionInfo,

SDTInformation,

FiveG-ProSeAuthorized,

UuRLCChannelToBeSetupList,

UuRLCChannelToBeModifiedList,

UuRLCChannelToBeReleasedList,

UuRLCChannelSetupList,

UuRLCChannelFailedToBeSetupList,

UuRLCChannelModifiedList,

UuRLCChannelFailedToBeModifiedList,

UuRLCChannelRequiredToBeModifiedList,

UuRLCChannelRequiredToBeReleasedList,

PC5RLCChannelToBeSetupList,

PC5RLCChannelToBeModifiedList,

PC5RLCChannelToBeReleasedList,

PC5RLCChannelSetupList,

PC5RLCChannelFailedToBeSetupList,

PC5RLCChannelFailedToBeModifiedList,

PC5RLCChannelRequiredToBeModifiedList,

PC5RLCChannelRequiredToBeReleasedList,

PC5RLCChannelModifiedList,

RemoteUELocalID,

PathSwitchConfiguration,

SidelinkRelayConfiguration,

PagingCause,

PEIPSAssistanceInfo,

UEPagingCapability,

GNBDUUESliceMaximumBitRateList,

MDTPollutedMeasurementIndicator,

UE-MulticastMRBs-ConfirmedToBeModified-Item,

UE-MulticastMRBs-RequiredToBeModified-Item,

UE-MulticastMRBs-RequiredToBeReleased-Item,

UE-MulticastMRBs-Setup-Item,

UE-MulticastMRBs-Setupnew-Item,

UE-MulticastMRBs-ToBeReleased-Item,

UE-MulticastMRBs-ToBeSetup-Item,

UE-MulticastMRBs-ToBeSetup-atModify-Item,

PosMeasurementAmount,

BAP-Header-Rewriting-Removed-List-Item,

SLDRXCycleList,

MDTPLMNModificationList,

ActivationRequestType,

PosMeasGapPreConfigList,

PosMeasurementPeriodicityNR-AoA,

SRSPosRRCInactiveConfig,

SDTBearerConfigurationQueryIndication,

SDTBearerConfigurationInfo,

ServingCellMO-List-Item,

ServingCellMO-encoded-in-CGC-List,

PosSItypeList,

DAPS-HO-Status,

UuRLCChannelID,

UplinkTxDirectCurrentTwoCarrierListInfo,

SRSPosRRCInactiveQueryIndication,

MC-PagingCell-Item,

UlTxDirectCurrentMoreCarrierInformation,

CPACMCGInformation,

ExtendedUEIdentityIndexValue,

HashedUEIdentityIndexValue,

GlobalGNB-ID,

Activated-Cells-Mapping-List-Item

FROM F1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

F1AP-PRIVATE-IES,

F1AP-PROTOCOL-EXTENSION,

F1AP-PROTOCOL-IES,

F1AP-PROTOCOL-IES-PAIR

FROM F1AP-Containers

id-BroadcastMRBs-FailedToBeModified-List,

id-BroadcastMRBs-FailedToBeModified-Item,

id-BroadcastMRBs-FailedToBeSetup-List,

id-BroadcastMRBs-FailedToBeSetup-Item,

id-BroadcastMRBs-FailedToBeSetupMod-List,

id-BroadcastMRBs-FailedToBeSetupMod-Item,

id-BroadcastMRBs-Modified-List,

id-BroadcastMRBs-Modified-Item,

id-BroadcastMRBs-Setup-List,

id-BroadcastMRBs-Setup-Item,

id-BroadcastMRBs-SetupMod-List,

id-BroadcastMRBs-SetupMod-Item,

id-BroadcastMRBs-ToBeModified-List,

id-BroadcastMRBs-ToBeModified-Item,

id-BroadcastMRBs-ToBeReleased-List,

id-BroadcastMRBs-ToBeReleased-Item,

id-BroadcastMRBs-ToBeSetup-List,

id-BroadcastMRBs-ToBeSetup-Item,

id-BroadcastMRBs-ToBeSetupMod-List,

id-BroadcastMRBs-ToBeSetupMod-Item,

id-Candidate-SpCell-Item,

id-Candidate-SpCell-List,

id-Cause,

id-Cancel-all-Warning-Messages-Indicator,

id-Cells-Failed-to-be-Activated-List,

id-Cells-Failed-to-be-Activated-List-Item,

id-Cells-Status-Item,

id-Cells-Status-List,

id-Cells-to-be-Activated-List,

id-Cells-to-be-Activated-List-Item,

id-Cells-to-be-Deactivated-List,

id-Cells-to-be-Deactivated-List-Item,

id-ConfirmedUEID,

id-CriticalityDiagnostics,

id-C-RNTI,

id-CUtoDURRCInformation,

id-DRB-Activity-Item,

id-DRB-Activity-List,

id-DRBs-FailedToBeModified-Item,

id-DRBs-FailedToBeModified-List,

id-DRBs-FailedToBeSetup-Item,

id-DRBs-FailedToBeSetup-List,

id-DRBs-FailedToBeSetupMod-Item,

id-DRBs-FailedToBeSetupMod-List,

id-DRBs-ModifiedConf-Item,

id-DRBs-ModifiedConf-List,

id-DRBs-Modified-Item,

id-DRBs-Modified-List,

id-DRB-Notify-Item,

id-DRB-Notify-List,

id-DRBs-Required-ToBeModified-Item,

id-DRBs-Required-ToBeModified-List,

id-DRBs-Required-ToBeReleased-Item,

id-DRBs-Required-ToBeReleased-List,

id-DRBs-Setup-Item,

id-DRBs-Setup-List,

id-DRBs-SetupMod-Item,

id-DRBs-SetupMod-List,

id-DRBs-ToBeModified-Item,

id-DRBs-ToBeModified-List,

id-DRBs-ToBeReleased-Item,

id-DRBs-ToBeReleased-List,

id-DRBs-ToBeSetup-Item,

id-DRBs-ToBeSetup-List,

id-DRBs-ToBeSetupMod-Item,

id-DRBs-ToBeSetupMod-List,

id-DRXCycle,

id-DUtoCURRCInformation,

id-ExecuteDuplication,

id-FullConfiguration,

id-gNB-CU-MBS-F1AP-ID,

id-gNB-CU-UE-F1AP-ID,

id-gNB-DU-MBS-F1AP-ID,

id-gNB-DU-UE-F1AP-ID,

id-gNB-DU-ID,

id-GNB-DU-Served-Cells-Item,

id-gNB-DU-Served-Cells-List,

id-gNB-CU-Name,

id-gNB-DU-Name,

id-Extended-GNB-CU-Name,

id-Extended-GNB-DU-Name,

id-InactivityMonitoringRequest,

id-InactivityMonitoringResponse,

id-MBS-Area-Session-ID,

id-MBS-CUtoDURRCInformation,

id-MBS-Session-ID,

id-MBS-ServiceArea,

id-MBSMulticastF1UContextDescriptor,

id-MC-PagingCell-Item,

id-MC-PagingCell-List,

id-MulticastF1UContextReferenceCU,

id-MulticastMBSSessionSetupList,

id-MulticastMBSSessionRemoveList,

id-MulticastMRBs-FailedToBeModified-List,

id-MulticastMRBs-FailedToBeModified-Item,

id-MulticastMRBs-FailedToBeSetup-List,

id-MulticastMRBs-FailedToBeSetup-Item,

id-MulticastMRBs-FailedToBeSetupMod-List,

id-MulticastMRBs-FailedToBeSetupMod-Item,

id-MulticastMRBs-Modified-List,

id-MulticastMRBs-Modified-Item,

id-MulticastMRBs-Setup-List,

id-MulticastMRBs-Setup-Item,

id-MulticastMRBs-SetupMod-List,

id-MulticastMRBs-SetupMod-Item,

id-MulticastMRBs-ToBeModified-List,

id-MulticastMRBs-ToBeModified-Item,

id-MulticastMRBs-ToBeReleased-List,

id-MulticastMRBs-ToBeReleased-Item,

id-MulticastMRBs-ToBeSetup-List,

id-MulticastMRBs-ToBeSetup-Item,

id-MulticastMRBs-ToBeSetupMod-List,

id-MulticastMRBs-ToBeSetupMod-Item,

id-MulticastF1UContext-ToBeSetup-List,

id-MulticastF1UContext-ToBeSetup-Item,

id-MulticastF1UContext-Setup-List,

id-MulticastF1UContext-Setup-Item,

id-MulticastF1UContext-FailedToBeSetup-List,

id-MulticastF1UContext-FailedToBeSetup-Item,

id-BroadcastAreaScope,

id-new-gNB-CU-UE-F1AP-ID,

id-new-gNB-DU-UE-F1AP-ID,

id-oldgNB-DU-UE-F1AP-ID,

id-PLMNAssistanceInfoForNetShar,

id-Potential-SpCell-Item,

id-Potential-SpCell-List,

id-RAT-FrequencyPriorityInformation,

id-RedirectedRRCmessage,

id-ResetType,

id-RequestedSRSTransmissionCharacteristics,

id-ResourceCoordinationTransferContainer,

id-RRCContainer,

id-RRCContainer-RRCSetupComplete,

id-RRCReconfigurationCompleteIndicator,

id-SCell-FailedtoSetup-List,

id-SCell-FailedtoSetup-Item,

id-SCell-FailedtoSetupMod-List,

id-SCell-FailedtoSetupMod-Item,

id-SCell-ToBeRemoved-Item,

id-SCell-ToBeRemoved-List,

id-SCell-ToBeSetup-Item,

id-SCell-ToBeSetup-List,

id-SCell-ToBeSetupMod-Item,

id-SCell-ToBeSetupMod-List,

id-SDT-Termination-Request,

id-SelectedPLMNID,

id-Served-Cells-To-Add-Item,

id-Served-Cells-To-Add-List,

id-Served-Cells-To-Delete-Item,

id-Served-Cells-To-Delete-List,

id-Served-Cells-To-Modify-Item,

id-Served-Cells-To-Modify-List,

id-ServCellIndex,

id-ServingCellMO,

id-SNSSAI,

id-SpCell-ID,

id-SpCellULConfigured,

id-SRBID,

id-SRBs-FailedToBeSetup-Item,

id-SRBs-FailedToBeSetup-List,

id-SRBs-FailedToBeSetupMod-Item,

id-SRBs-FailedToBeSetupMod-List,

id-SRBs-Required-ToBeReleased-Item,

id-SRBs-Required-ToBeReleased-List,

id-SRBs-ToBeReleased-Item,

id-SRBs-ToBeReleased-List,

id-SRBs-ToBeSetup-Item,

id-SRBs-ToBeSetup-List,

id-SRBs-ToBeSetupMod-Item,

id-SRBs-ToBeSetupMod-List,

id-SRBs-Modified-Item,

id-SRBs-Modified-List,

id-SRBs-Setup-Item,

id-SRBs-Setup-List,

id-SRBs-SetupMod-Item,

id-SRBs-SetupMod-List,

id-TimeToWait,

id-TransactionID,

id-TransmissionActionIndicator,

id-UEContextNotRetrievable,

id-UE-associatedLogicalF1-ConnectionItem,

id-UE-associatedLogicalF1-ConnectionListResAck,

id-UEIdentity-List-For-Paging-List,

id-UEIdentity-List-For-Paging-Item,

id-UE-MulticastMRBs-ConfirmedToBeModified-List,

id-UE-MulticastMRBs-ConfirmedToBeModified-Item,

id-UE-MulticastMRBs-RequiredToBeModified-List,

id-UE-MulticastMRBs-RequiredToBeModified-Item,

id-UE-MulticastMRBs-RequiredToBeReleased-List,

id-UE-MulticastMRBs-RequiredToBeReleased-Item,

id-UE-MulticastMRBs-Setup-List,

id-UE-MulticastMRBs-Setup-Item,

id-UE-MulticastMRBs-Setupnew-List,

id-UE-MulticastMRBs-Setupnew-Item,

id-UE-MulticastMRBs-ToBeReleased-List,

id-UE-MulticastMRBs-ToBeReleased-Item,

id-UE-MulticastMRBs-ToBeSetup-atModify-List,

id-UE-MulticastMRBs-ToBeSetup-atModify-Item,

id-UE-MulticastMRBs-ToBeSetup-List,

id-UE-MulticastMRBs-ToBeSetup-Item,

id-DUtoCURRCContainer,

id-NRCGI,

id-PagingCell-Item,

id-PagingCell-List,

id-PagingDRX,

id-PagingPriority,

id-SItype-List,

id-UEIdentityIndexValue,

id-GNB-CU-TNL-Association-Setup-List,

id-GNB-CU-TNL-Association-Setup-Item,

id-GNB-CU-TNL-Association-Failed-To-Setup-List,

id-GNB-CU-TNL-Association-Failed-To-Setup-Item,

id-GNB-CU-TNL-Association-To-Add-Item,

id-GNB-CU-TNL-Association-To-Add-List,

id-GNB-CU-TNL-Association-To-Remove-Item,

id-GNB-CU-TNL-Association-To-Remove-List,

id-GNB-CU-TNL-Association-To-Update-Item,

id-GNB-CU-TNL-Association-To-Update-List,

id-MaskedIMEISV,

id-PagingIdentity,

id-Cells-to-be-Barred-List,

id-Cells-to-be-Barred-Item,

id-PWSSystemInformation,

id-RepetitionPeriod,

id-NumberofBroadcastRequest,

id-Cells-To-Be-Broadcast-List,

id-Cells-To-Be-Broadcast-Item,

id-Cells-Broadcast-Completed-List,

id-Cells-Broadcast-Completed-Item,

id-Broadcast-To-Be-Cancelled-List,

id-Broadcast-To-Be-Cancelled-Item,

id-Cells-Broadcast-Cancelled-List,

id-Cells-Broadcast-Cancelled-Item,

id-NR-CGI-List-For-Restart-List,

id-NR-CGI-List-For-Restart-Item,

id-PWS-Failed-NR-CGI-List,

id-PWS-Failed-NR-CGI-Item,

id-EUTRA-NR-CellResourceCoordinationReq-Container,

id-EUTRA-NR-CellResourceCoordinationReqAck-Container,

id-Protected-EUTRA-Resources-List,

id-RequestType,

id-ServingPLMN,

id-DRXConfigurationIndicator,

id-RLCFailureIndication,

id-UplinkTxDirectCurrentListInformation,

id-SULAccessIndication,

id-Protected-EUTRA-Resources-Item,

id-GNB-DUConfigurationQuery,

id-GNB-DU-UE-AMBR-UL,

id-GNB-CU-RRC-Version,

id-GNB-DU-RRC-Version,

id-GNBDUOverloadInformation,

id-NeedforGap,

id-RRCDeliveryStatusRequest,

id-RRCDeliveryStatus,

id-Dedicated-SIDelivery-NeededUE-List,

id-Dedicated-SIDelivery-NeededUE-Item,

id-ResourceCoordinationTransferInformation,

id-Associated-SCell-List,

id-Associated-SCell-Item,

id-IgnoreResourceCoordinationContainer,

id-UAC-Assistance-Info,

id-RANUEID,

id-PagingOrigin,

id-GNB-DU-TNL-Association-To-Remove-Item,

id-GNB-DU-TNL-Association-To-Remove-List,

id-NotificationInformation,

id-TraceActivation,

id-TraceID,

id-Neighbour-Cell-Information-List,

id-Neighbour-Cell-Information-Item,

id-SymbolAllocInSlot,

id-NumDLULSymbols,

id-AdditionalRRMPriorityIndex,

id-DUCURadioInformationType,

id-CUDURadioInformationType,

id-LowerLayerPresenceStatusChange,

id-Transport-Layer-Address-Info,

id-BHChannels-ToBeSetup-List,

id-BHChannels-ToBeSetup-Item,

id-BHChannels-Setup-List,

id-BHChannels-Setup-Item,

id-BHChannels-ToBeModified-Item,

id-BHChannels-ToBeModified-List,

id-BHChannels-ToBeReleased-Item,

id-BHChannels-ToBeReleased-List,

id-BHChannels-ToBeSetupMod-Item,

id-BHChannels-ToBeSetupMod-List,

id-BHChannels-FailedToBeSetup-Item,

id-BHChannels-FailedToBeSetup-List,

id-BHChannels-FailedToBeModified-Item,

id-BHChannels-FailedToBeModified-List,

id-BHChannels-FailedToBeSetupMod-Item,

id-BHChannels-FailedToBeSetupMod-List,

id-BHChannels-Modified-Item,

id-BHChannels-Modified-List,

id-BHChannels-SetupMod-Item,

id-BHChannels-SetupMod-List,

id-BHChannels-Required-ToBeReleased-Item,

id-BHChannels-Required-ToBeReleased-List,

id-BAPAddress,

id-ConfiguredBAPAddress,

id-BH-Routing-Information-Added-List,

id-BH-Routing-Information-Added-List-Item,

id-BH-Routing-Information-Removed-List,

id-BH-Routing-Information-Removed-List-Item,

id-UL-BH-Non-UP-Traffic-Mapping,

id-Child-Nodes-List,

id-Activated-Cells-to-be-Updated-List,

id-IABIPv6RequestType,

id-IAB-TNL-Addresses-To-Remove-List,

id-IAB-TNL-Addresses-To-Remove-Item,

id-IAB-Allocated-TNL-Address-List,

id-IAB-Allocated-TNL-Address-Item,

id-IABv4AddressesRequested,

id-TrafficMappingInformation,

id-UL-UP-TNL-Information-to-Update-List,

id-UL-UP-TNL-Information-to-Update-List-Item,

id-UL-UP-TNL-Address-to-Update-List,

id-UL-UP-TNL-Address-to-Update-List-Item,

id-DL-UP-TNL-Address-to-Update-List,

id-DL-UP-TNL-Address-to-Update-List-Item,

id-NRV2XServicesAuthorized,

id-LTEV2XServicesAuthorized,

id-NRUESidelinkAggregateMaximumBitrate,

id-LTEUESidelinkAggregateMaximumBitrate,

id-PC5LinkAMBR,

id-SLDRBs-FailedToBeModified-Item,

id-SLDRBs-FailedToBeModified-List,

id-SLDRBs-FailedToBeSetup-Item,

id-SLDRBs-FailedToBeSetup-List,

id-SLDRBs-Modified-Item,

id-SLDRBs-Modified-List,

id-SLDRBs-Required-ToBeModified-Item,

id-SLDRBs-Required-ToBeModified-List,

id-SLDRBs-Required-ToBeReleased-Item,

id-SLDRBs-Required-ToBeReleased-List,

id-SLDRBs-Setup-Item,

id-SLDRBs-Setup-List,

id-SLDRBs-ToBeModified-Item,

id-SLDRBs-ToBeModified-List,

id-SLDRBs-ToBeReleased-Item,

id-SLDRBs-ToBeReleased-List,

id-SLDRBs-ToBeSetup-Item,

id-SLDRBs-ToBeSetup-List,

id-SLDRBs-ToBeSetupMod-Item,

id-SLDRBs-ToBeSetupMod-List,

id-SLDRBs-SetupMod-List,

id-SLDRBs-FailedToBeSetupMod-List,

id-SLDRBs-SetupMod-Item,

id-SLDRBs-FailedToBeSetupMod-Item,

id-SLDRBs-ModifiedConf-List,

id-SLDRBs-ModifiedConf-Item,

id-gNBCUMeasurementID,

id-gNBDUMeasurementID,

id-RegistrationRequest,

id-ReportCharacteristics,

id-CellToReportList,

id-CellMeasurementResultList,

id-HardwareLoadIndicator,

id-ReportingPeriodicity,

id-TNLCapacityIndicator,

id-RACHReportInformationList,

id-RLFReportInformationList,

id-ReportingRequestType,

id-TimeReferenceInformation,

id-ConditionalInterDUMobilityInformation,

id-ConditionalIntraDUMobilityInformation,

id-targetCellsToCancel,

id-requestedTargetCellGlobalID,

id-TraceCollectionEntityIPAddress,

id-ManagementBasedMDTPLMNList,

id-PrivacyIndicator,

id-TraceCollectionEntityURI,

id-ServingNID,

id-PosAssistance-Information,

id-PosBroadcast,

id-PositioningBroadcastCells,

id-RoutingID,

id-PosAssistanceInformationFailureList,

id-PosMeasurementQuantities,

id-PosMeasurementResultList,

id-PosMeasurementPeriodicity,

id-PosReportCharacteristics,

id-TRPInformationTypeListTRPReq,

id-TRPInformationTypeItem,

id-TRPInformationListTRPResp,

id-TRPInformationItem,

id-LMF-MeasurementID,

id-RAN-MeasurementID,

id-SRSType,

id-ActivationTime,

id-AbortTransmission,

id-SRSConfiguration,

id-TRPList,

id-E-CID-MeasurementQuantities,

id-E-CID-MeasurementPeriodicity,

id-E-CID-MeasurementResult,

id-Cell-Portion-ID,

id-LMF-UE-MeasurementID,

id-RAN-UE-MeasurementID,

id-SFNInitialisationTime,

id-SystemFrameNumber,

id-SlotNumber,

id-TRP-MeasurementRequestList,

id-MeasurementBeamInfoRequest,

id-E-CID-ReportCharacteristics,

id-F1CTransferPath,

id-SCGIndicator,

id-SRSSpatialRelationPerSRSResource,

id-PosMeasurementPeriodicityExtended,

id-SuccessfulHOReportInformationList,

id-Coverage-Modification-Notification,

id-CCO-Assistance-Information,

id-CellsForSON-List,

id-IABCongestionIndication,

id-IABConditionalRRCMessageDeliveryIndication,

id-F1CTransferPathNRDC,

id-BufferSizeThresh,

id-IAB-TNL-Addresses-Exception,

id-BAP-Header-Rewriting-Added-List,

id-BAP-Header-Rewriting-Added-List-Item,

id-Re-routingEnableIndicator,

id-NonF1terminatingTopologyIndicator,

id-EgressNonF1terminatingTopologyIndicator,

id-IngressNonF1terminatingTopologyIndicator,

id-Neighbour-Node-Cells-List,

id-Serving-Cells-List,

id-MDTPollutedMeasurementIndicator,

id-PDCMeasurementPeriodicity,

id-PDCMeasurementQuantities,

id-PDCMeasurementResult,

id-PDCReportType,

id-RAN-UE-PDC-MeasID,

id-SCGActivationRequest,

id-SCGActivationStatus,

id-TRP-MeasurementUpdateList,

id-PRSTRPList,

id-PRSTransmissionTRPList,

id-ResponseTime,

id-TRP-PRS-Info-List,

id-PRS-Measurement-Info-List,

id-PRSConfigRequestType,

id-MeasurementCharacteristicsRequestIndicator,

id-MeasurementTimeOccasion,

id-UEReportingInformation,

id-PosConextRevIndication,

id-NRRedCapUEIndication,

id-RANUEPagingDRX,

id-CNUEPagingDRX,

id-NRPagingeDRXInformation,

id-NRPagingeDRXInformationforRRCINACTIVE,

id-QoEInformation,

id-CG-SDTQueryIndication,

id-CG-SDTKeptIndicator,

id-CG-SDTSessionInfoOld,

id-SDTInformation,

id-FiveG-ProSeAuthorized,

id-FiveG-ProSePC5LinkAMBR,

id-FiveG-ProSeUEPC5AggregateMaximumBitrate,

id-UuRLCChannelToBeSetupList,

id-UuRLCChannelToBeModifiedList,

id-UuRLCChannelToBeReleasedList,

id-UuRLCChannelSetupList,

id-UuRLCChannelFailedToBeSetupList,

id-UuRLCChannelModifiedList,

id-UuRLCChannelFailedToBeModifiedList,

id-UuRLCChannelRequiredToBeModifiedList,

id-UuRLCChannelRequiredToBeReleasedList,

id-PC5RLCChannelToBeSetupList,

id-PC5RLCChannelToBeModifiedList,

id-PC5RLCChannelToBeReleasedList,

id-PC5RLCChannelSetupList,

id-PC5RLCChannelFailedToBeSetupList,

id-PC5RLCChannelModifiedList,

id-PC5RLCChannelFailedToBeModifiedList,

id-PC5RLCChannelRequiredToBeModifiedList,

id-PC5RLCChannelRequiredToBeReleasedList,

id-SidelinkRelayConfiguration,

id-UpdatedRemoteUELocalID,

id-PathSwitchConfiguration,

id-PagingCause,

id-PEIPSAssistanceInfo,

id-UEPagingCapability,

id-GNBDUUESliceMaximumBitRateList,

id-PosMeasurementAmount,

id-BAP-Header-Rewriting-Removed-List,

id-BAP-Header-Rewriting-Removed-List-Item,

id-SLDRXCycleList,

id-ManagementBasedMDTPLMNModificationList,

id-ActivationRequestType,

id-PosMeasGapPreConfigList,

id-PosMeasurementPeriodicityNR-AoA,

id-SRSPosRRCInactiveConfig,

id-SDTBearerConfigurationQueryIndication,

id-SDTBearerConfigurationInfo,

id-ServingCellMO-List,

id-ServingCellMO-List-Item,

id-ServingCellMO-encoded-in-CGC-List,

id-PosSItypeList,

id-DAPS-HO-Status,

id-SRBMappingInfo,

id-UplinkTxDirectCurrentTwoCarrierListInfo,

id-SRSPosRRCInactiveQueryIndication,

id-UlTxDirectCurrentMoreCarrierInformation,

id-CPACMCGInformation,

id-ExtendedUEIdentityIndexValue,

id-HashedUEIdentityIndexValue,

id-Target-gNB-ID,

id-Target-gNB-IP-address,

id-Target-SeGW-IP-address,

id-Activated-Cells-Mapping-List,

id-Activated-Cells-Mapping-List-Item,

id-F1SetupOutcome,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofErrors,

maxnoofIndividualF1ConnectionsToReset,

maxnoofPotentialSpCells,

maxnoofSCells,

maxnoofSRBs,

maxnoofPagingCells,

maxnoofTNLAssociations,

maxCellineNB,

maxnoofUEIDs,

maxnoofBHRLCChannels,

maxnoofRoutingEntries,

maxnoofChildIABNodes,

maxnoofServedCellsIAB,

maxnoofTLAsIAB,

maxnoofULUPTNLInformationforIAB,

maxnoofUPTNLAddresses,

maxnoofSLDRBs,

maxnoofTRPInfoTypes,

maxnoofTRPs,

maxnoofMRBs,

maxnoofUEIDforPaging,

maxnoofNeighbourNodeCellsIAB,

maxnoofMRBsforUE,

maxnoofServingCellMOs

FROM F1AP-Constants;

UNCHANGED PART OMITTED

#### -- MIAB F1 SETUP TRIGGERING PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MIAN F1 SETUP TRIGGERING

--

-- **************************************************************

MIABF1SetupTriggering ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MIABF1SetupTriggeringIEs}},

...

}

MIABF1SetupTriggeringIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID							PRESENCE mandatory	}|

{ ID id-Target-gNB-ID						CRITICALITY reject	TYPE GlobalGNB-ID							PRESENCE mandatory	}|

{ ID id-Target-gNB-IP-address				CRITICALITY ignore	TYPE TransportLayerAddress					PRESENCE optional	}|

{ ID id-Target-SeGW-IP-address				CRITICALITY ignore	TYPE TransportLayerAddress					PRESENCE optional	},

...

}

#### -- MIAB F1 SETUP OUTCOME NOTIFICATION PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MIAB F1 SETUP OUTCOME NOTIFICATION

--

-- **************************************************************

MIABF1SetupOutcomeNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MIABF1SetupOutcomeNotificationIEs}},

...

}

MIABF1SetupOutcomeNotificationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID								PRESENCE mandatory	}|

{ ID id-F1SetupOutcome						CRITICALITY reject	TYPE F1SetupOutcome								PRESENCE mandatory	}|

{ ID id-Activated-Cells-Mapping-List		CRITICALITY ignore	TYPE Activated-Cells-Mapping-List			PRESENCE optional	},

...

}

F1SetupOutcome ::= ENUMERATED {success, failure,...}

Activated-Cells-Mapping-List ::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Activated-Cells-Mapping-List-ItemIEs } }

Activated-Cells-Mapping-List-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Activated-Cells-Mapping-List-Item	CRITICALITY ignore	TYPE Activated-Cells-Mapping-List-Item PRESENCE mandatory },

...

}

END

-- ASN1STOP

### 9.4.5	Information Element Definitions

UNCHANGED PART OMITTED

#### -- A

Activated-Cells-Mapping-List-Item	::= SEQUENCE {

nRCGIforTargetLogicalDU						NRCGI,

nRCGIforSourceLogicalDU						NRCGI,

iE-Extensions	ProtocolExtensionContainer { { Activated-Cells-Mapping-List-ItemExtIEs } }	OPTIONAL,

...

}

Activated-Cells-Mapping-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UNCHANGED PART OMITTED

#### -- G

GlobalGNB-ID ::= SEQUENCE {

pLMNIdentity		PLMN-Identity,

gNB-ID				GNB-ID,

iE-Extensions		ProtocolExtensionContainer { {GlobalGNB-ID-ExtIEs} } OPTIONAL,

...

}

GlobalGNB-ID-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-ID ::= CHOICE {

gNB-ID					BIT STRING (SIZE(22..32)),

choice-Extensions		ProtocolIE-SingleContainer { {GNB-ID-ExtIEs} }

}

GNB-ID-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

UNCHANGED PART OMITTED

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM F1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-Reset									ProcedureCode ::= 0

id-F1Setup									ProcedureCode ::= 1

id-ErrorIndication							ProcedureCode ::= 2

id-gNBDUConfigurationUpdate					ProcedureCode ::= 3

id-gNBCUConfigurationUpdate					ProcedureCode ::= 4

id-UEContextSetup							ProcedureCode ::= 5

id-UEContextRelease							ProcedureCode ::= 6

id-UEContextModification					ProcedureCode ::= 7

id-UEContextModificationRequired			ProcedureCode ::= 8

id-UEMobilityCommand						ProcedureCode ::= 9

id-UEContextReleaseRequest					ProcedureCode ::= 10

id-InitialULRRCMessageTransfer				ProcedureCode ::= 11

id-DLRRCMessageTransfer						ProcedureCode ::= 12

id-ULRRCMessageTransfer						ProcedureCode ::= 13

id-privateMessage							ProcedureCode ::= 14

id-UEInactivityNotification					ProcedureCode ::= 15

id-GNBDUResourceCoordination				ProcedureCode ::= 16

id-SystemInformationDeliveryCommand			ProcedureCode ::= 17

id-Paging									ProcedureCode ::= 18

id-Notify									ProcedureCode ::= 19

id-WriteReplaceWarning						ProcedureCode ::= 20

id-PWSCancel								ProcedureCode ::= 21

id-PWSRestartIndication						ProcedureCode ::= 22

id-PWSFailureIndication						ProcedureCode ::= 23

id-GNBDUStatusIndication 					ProcedureCode ::= 24

id-RRCDeliveryReport		 				ProcedureCode ::= 25

id-F1Removal								ProcedureCode ::= 26

id-NetworkAccessRateReduction				ProcedureCode ::= 27

id-TraceStart								ProcedureCode ::= 28

id-DeactivateTrace							ProcedureCode ::= 29

id-DUCURadioInformationTransfer				ProcedureCode ::= 30

id-CUDURadioInformationTransfer				ProcedureCode ::= 31

id-BAPMappingConfiguration					ProcedureCode ::= 32

id-GNBDUResourceConfiguration				ProcedureCode ::= 33

id-IABTNLAddressAllocation					ProcedureCode ::= 34

id-IABUPConfigurationUpdate					ProcedureCode ::= 35

id-resourceStatusReportingInitiation		ProcedureCode ::= 36

id-resourceStatusReporting					ProcedureCode ::= 37

id-accessAndMobilityIndication				ProcedureCode ::= 38

id-accessSuccess							ProcedureCode ::= 39

id-cellTrafficTrace 							ProcedureCode ::= 40

id-PositioningMeasurementExchange			ProcedureCode ::= 41

id-PositioningAssistanceInformationControl	ProcedureCode ::= 42

id-PositioningAssistanceInformationFeedback	ProcedureCode ::= 43

id-PositioningMeasurementReport				ProcedureCode ::= 44

id-PositioningMeasurementAbort				ProcedureCode ::= 45

id-PositioningMeasurementFailureIndication	ProcedureCode ::= 46

id-PositioningMeasurementUpdate				ProcedureCode ::= 47

id-TRPInformationExchange					ProcedureCode ::= 48

id-PositioningInformationExchange			ProcedureCode ::= 49

id-PositioningActivation					ProcedureCode ::= 50

id-PositioningDeactivation					ProcedureCode ::= 51

id-E-CIDMeasurementInitiation				ProcedureCode ::= 52

id-E-CIDMeasurementFailureIndication		ProcedureCode ::= 53

id-E-CIDMeasurementReport					ProcedureCode ::= 54

id-E-CIDMeasurementTermination				ProcedureCode ::= 55

id-PositioningInformationUpdate				ProcedureCode ::= 56

id-ReferenceTimeInformationReport			ProcedureCode ::= 57

id-ReferenceTimeInformationReportingControl	ProcedureCode ::= 58

id-BroadcastContextSetup					ProcedureCode ::= 59

id-BroadcastContextRelease					ProcedureCode ::= 60

id-BroadcastContextReleaseRequest			ProcedureCode ::= 61

id-BroadcastContextModification				ProcedureCode ::= 62

id-MulticastGroupPaging						ProcedureCode ::= 63

id-MulticastContextSetup					ProcedureCode ::= 64

id-MulticastContextRelease					ProcedureCode ::= 65

id-MulticastContextReleaseRequest			ProcedureCode ::= 66

id-MulticastContextModification				ProcedureCode ::= 67

id-MulticastDistributionSetup				ProcedureCode ::= 68

id-MulticastDistributionRelease				ProcedureCode ::= 69

id-PDCMeasurementInitiation					ProcedureCode ::= 70

id-PDCMeasurementReport						ProcedureCode ::= 71

id-PDCMeasurementInitiationRequest			ProcedureCode ::= 72

id-PDCMeasurementInitiationResponse			ProcedureCode ::= 73

id-PDCMeasurementInitiationFailure			ProcedureCode ::= 74

id-pRSConfigurationExchange					ProcedureCode ::= 75

id-measurementPreconfiguration				ProcedureCode ::= 76

id-measurementActivation					ProcedureCode ::= 77

id-QoEInformationTransfer					ProcedureCode ::= 78

id-PDCMeasurementTerminationCommand			ProcedureCode ::= 79

id-PDCMeasurementFailureIndication 			ProcedureCode ::= 80

id-PosSystemInformationDeliveryCommand		ProcedureCode ::= 81

id-MIABF1SetupTriggering					ProcedureCode ::= xx

id-MIABF1SetupNotification				ProcedureCode ::= yy

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs							INTEGER ::= 65535

maxProtocolExtensions					INTEGER ::= 65535

maxProtocolIEs							INTEGER ::= 65535

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxNRARFCN								INTEGER ::= 3279165

maxnoofErrors							INTEGER ::= 256

maxnoofIndividualF1ConnectionsToReset	INTEGER ::= 65536

maxCellingNBDU							INTEGER ::= 512

maxnoofSCells							INTEGER ::= 32

maxnoofSRBs								INTEGER ::= 8

maxnoofDRBs								INTEGER ::= 64

maxnoofULUPTNLInformation				INTEGER ::= 2

maxnoofDLUPTNLInformation				INTEGER ::= 2

maxnoofBPLMNs							INTEGER ::= 6

maxnoofCandidateSpCells					INTEGER ::= 64

maxnoofPotentialSpCells					INTEGER ::= 64

maxnoofNrCellBands						INTEGER ::= 32

maxnoofSIBTypes							INTEGER ::= 32

maxnoofSITypes							INTEGER ::= 32

maxnoofPagingCells						INTEGER ::= 512

maxnoofTNLAssociations					INTEGER ::= 32

maxnoofQoSFlows							INTEGER ::= 64

maxnoofSliceItems						INTEGER ::= 1024

maxCellineNB							INTEGER ::= 256

maxnoofExtendedBPLMNs					INTEGER ::= 6

maxnoofUEIDs							INTEGER ::= 65536

maxnoofBPLMNsNR							INTEGER ::= 12

maxnoofUACPLMNs							INTEGER ::= 12

maxnoofUACperPLMN						INTEGER ::= 64

maxnoofAdditionalSIBs					INTEGER ::= 63

maxnoofslots							INTEGER ::= 5120

maxnoofTLAs								INTEGER ::=	16

maxnoofGTPTLAs							INTEGER ::=	16

maxnoofBHRLCChannels					INTEGER ::= 65536

maxnoofRoutingEntries					INTEGER ::= 1024

maxnoofIABSTCInfo						INTEGER ::= 45

maxnoofSymbols							INTEGER ::= 14

maxnoofServingCells						INTEGER ::= 32

maxnoofDUFSlots							INTEGER ::= 320

maxnoofHSNASlots						INTEGER ::= 5120

maxnoofServedCellsIAB					INTEGER ::= 512

maxnoofChildIABNodes					INTEGER ::= 1024

maxnoofNonUPTrafficMappings				INTEGER ::= 32

maxnoofTLAsIAB							INTEGER ::= 1024

maxnoofMappingEntries					INTEGER ::= 67108864

maxnoofDSInfo							INTEGER ::= 64

maxnoofEgressLinks						INTEGER ::= 2

maxnoofULUPTNLInformationforIAB			INTEGER ::= 32678

maxnoofUPTNLAddresses					INTEGER ::= 8

maxnoofSLDRBs							INTEGER ::= 512

maxnoofQoSParaSets						INTEGER ::= 8

maxnoofPC5QoSFlows						INTEGER ::= 2048

maxnoofSSBAreas							INTEGER ::=	64

maxnoofPhysicalResourceBlocks			INTEGER ::= 275

maxnoofPhysicalResourceBlocks-1			INTEGER ::= 274

maxnoofPRACHconfigs						INTEGER ::= 16

maxnoofRACHReports						INTEGER ::= 64

maxnoofRLFReports						INTEGER ::= 64

maxnoofAdditionalPDCPDuplicationTNL		INTEGER ::=	2

maxnoofRLCDuplicationState				INTEGER ::=	3

maxnoofCHOcells							INTEGER ::= 8

maxnoofMDTPLMNs							INTEGER ::=	16

maxnoofCAGsupported						INTEGER ::= 12

maxnoofNIDsupported						INTEGER ::= 12

maxnoofNRSCSs							INTEGER ::= 5

maxnoofExtSliceItems					INTEGER ::= 65535

maxnoofPosMeas							INTEGER ::=	16384

maxnoofTRPInfoTypes						INTEGER ::=	64

maxnoofTRPs								INTEGER ::=	65535

maxnoofSRSTriggerStates					INTEGER ::= 3

maxnoofSpatialRelations					INTEGER ::= 64

maxnoBcastCell							INTEGER ::= 16384

maxnoofAngleInfo						INTEGER ::= 65535

maxnooflcs-gcs-translation				INTEGER ::= 3

maxnoofPath								INTEGER ::= 2

maxnoofMeasE-CID						INTEGER ::= 64

maxnoofSSBs								INTEGER ::= 255

maxnoSRS-ResourceSets					INTEGER ::= 16

maxnoSRS-ResourcePerSet					INTEGER ::= 16

maxnoSRS-Carriers						INTEGER ::= 32

maxnoSCSs								INTEGER ::= 5

maxnoSRS-Resources						INTEGER ::= 64

maxnoSRS-PosResources					INTEGER ::= 64

maxnoSRS-PosResourceSets				INTEGER ::= 16

maxnoSRS-PosResourcePerSet				INTEGER ::= 16

maxnoofPRS-ResourceSets					INTEGER ::= 2

maxnoofPRS-ResourcesPerSet				INTEGER ::= 64

maxNoOfMeasTRPs							INTEGER ::= 64

maxnoofPRSresourceSets					INTEGER ::= 8

maxnoofPRSresources						INTEGER ::= 64

maxnoofSuccessfulHOReports				INTEGER ::= 64

maxnoofNR-UChannelIDs					INTEGER ::= 16

maxServedCellforSON						INTEGER ::= 256

maxNeighbourCellforSON					INTEGER ::= 32

maxAffectedCells						INTEGER ::= 32

maxnoofMRBs								INTEGER ::= 32

maxnoofMBSQoSFlows						INTEGER ::= 64

maxnoofMBSFSAs 							INTEGER ::= 256

maxnoofUEIDforPaging 					INTEGER ::= 4096

maxnoofCellsforMBS						INTEGER ::= 512

maxnoofTAIforMBS						INTEGER ::= 512

maxnoofMBSAreaSessionIDs				INTEGER ::= 256

maxnoofMBSServiceAreaInformation		INTEGER ::= 256

maxnoofIABCongInd						INTEGER ::= 1024

maxnoofNeighbourNodeCellsIAB			INTEGER ::= 1024

maxnoofRBsetsPerCell					INTEGER ::= 8

maxnoofRBsetsPerCell-1					INTEGER ::= 7

maxnoofMeasPDC							INTEGER ::= 16

maxnoARPs								INTEGER ::=	16

maxnoofULAoAs							INTEGER ::= 8

maxNoPathExtended						INTEGER ::= 8

maxnoTRPTEGs							INTEGER ::= 8

maxFreqLayers							INTEGER ::= 4

maxNumResourcesPerAngle					INTEGER ::= 24

maxnoAzimuthAngles						INTEGER ::= 3600

maxnoElevationAngles					INTEGER ::= 1801

maxnoofPRSTRPs							INTEGER ::= 256

maxnoofQoEInformation					INTEGER ::= 16

maxnoofUuRLCChannels					INTEGER ::= 32

maxnoofPC5RLCChannels					INTEGER ::= 512

maxnoofSMBRValues						INTEGER ::= 8

maxnoofMRBsforUE						INTEGER ::= 64

maxnoofMBSSessionsofUE					INTEGER ::= 256

maxnoofSLdestinations					INTEGER ::= 32

maxnoofNSAGs							INTEGER ::= 256

maxnoofSDTBearers						INTEGER ::= 72

maxnoofServingCellMOs					INTEGER ::= 16

maxNrofBWPs								INTEGER ::= 8

maxnoofPosSITypes						INTEGER ::= 32

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause											ProtocolIE-ID ::= 0

id-Cells-Failed-to-be-Activated-List				ProtocolIE-ID ::= 1

id-Cells-Failed-to-be-Activated-List-Item			ProtocolIE-ID ::= 2

id-Cells-to-be-Activated-List						ProtocolIE-ID ::= 3

id-Cells-to-be-Activated-List-Item					ProtocolIE-ID ::= 4

id-Cells-to-be-Deactivated-List						ProtocolIE-ID ::= 5

id-Cells-to-be-Deactivated-List-Item				ProtocolIE-ID ::= 6

id-CriticalityDiagnostics							ProtocolIE-ID ::= 7

id-CUtoDURRCInformation								ProtocolIE-ID ::= 9

id-DRBs-FailedToBeModified-Item						ProtocolIE-ID ::= 12

id-DRBs-FailedToBeModified-List						ProtocolIE-ID ::= 13

id-DRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 14

id-DRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 15

id-DRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 16

id-DRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 17

id-DRBs-ModifiedConf-Item							ProtocolIE-ID ::= 18

id-DRBs-ModifiedConf-List							ProtocolIE-ID ::= 19

id-DRBs-Modified-Item								ProtocolIE-ID ::= 20

id-DRBs-Modified-List								ProtocolIE-ID ::= 21

id-DRBs-Required-ToBeModified-Item					ProtocolIE-ID ::= 22

id-DRBs-Required-ToBeModified-List					ProtocolIE-ID ::= 23

id-DRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 24

id-DRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 25

id-DRBs-Setup-Item									ProtocolIE-ID ::= 26

id-DRBs-Setup-List									ProtocolIE-ID ::= 27

id-DRBs-SetupMod-Item								ProtocolIE-ID ::= 28

id-DRBs-SetupMod-List								ProtocolIE-ID ::= 29

id-DRBs-ToBeModified-Item							ProtocolIE-ID ::= 30

id-DRBs-ToBeModified-List							ProtocolIE-ID ::= 31

id-DRBs-ToBeReleased-Item							ProtocolIE-ID ::= 32

id-DRBs-ToBeReleased-List							ProtocolIE-ID ::= 33

id-DRBs-ToBeSetup-Item								ProtocolIE-ID ::= 34

id-DRBs-ToBeSetup-List								ProtocolIE-ID ::= 35

id-DRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 36

id-DRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 37

id-DRXCycle											ProtocolIE-ID ::= 38

id-DUtoCURRCInformation								ProtocolIE-ID ::= 39

id-gNB-CU-UE-F1AP-ID								ProtocolIE-ID ::= 40

id-gNB-DU-UE-F1AP-ID								ProtocolIE-ID ::= 41

id-gNB-DU-ID										ProtocolIE-ID ::= 42

id-GNB-DU-Served-Cells-Item							ProtocolIE-ID ::= 43

id-gNB-DU-Served-Cells-List							ProtocolIE-ID ::= 44

id-gNB-DU-Name										ProtocolIE-ID ::= 45

id-NRCellID											ProtocolIE-ID ::= 46

id-oldgNB-DU-UE-F1AP-ID								ProtocolIE-ID ::= 47

id-ResetType										ProtocolIE-ID ::= 48

id-ResourceCoordinationTransferContainer			ProtocolIE-ID ::= 49

id-RRCContainer										ProtocolIE-ID ::= 50

id-SCell-ToBeRemoved-Item							ProtocolIE-ID ::= 51

id-SCell-ToBeRemoved-List							ProtocolIE-ID ::= 52

id-SCell-ToBeSetup-Item								ProtocolIE-ID ::= 53

id-SCell-ToBeSetup-List								ProtocolIE-ID ::= 54

id-SCell-ToBeSetupMod-Item							ProtocolIE-ID ::= 55

id-SCell-ToBeSetupMod-List							ProtocolIE-ID ::= 56

id-Served-Cells-To-Add-Item							ProtocolIE-ID ::= 57

id-Served-Cells-To-Add-List							ProtocolIE-ID ::= 58

id-Served-Cells-To-Delete-Item						ProtocolIE-ID ::= 59

id-Served-Cells-To-Delete-List						ProtocolIE-ID ::= 60

id-Served-Cells-To-Modify-Item						ProtocolIE-ID ::= 61

id-Served-Cells-To-Modify-List						ProtocolIE-ID ::= 62

id-SpCell-ID										ProtocolIE-ID ::= 63

id-SRBID											ProtocolIE-ID ::= 64

id-SRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 65

id-SRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 66

id-SRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 67

id-SRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 68

id-SRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 69

id-SRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 70

id-SRBs-ToBeReleased-Item							ProtocolIE-ID ::= 71

id-SRBs-ToBeReleased-List							ProtocolIE-ID ::= 72

id-SRBs-ToBeSetup-Item								ProtocolIE-ID ::= 73

id-SRBs-ToBeSetup-List								ProtocolIE-ID ::= 74

id-SRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 75

id-SRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 76

id-TimeToWait										ProtocolIE-ID ::= 77

id-TransactionID									ProtocolIE-ID ::= 78

id-TransmissionActionIndicator						ProtocolIE-ID ::= 79

id-UE-associatedLogicalF1-ConnectionItem 			ProtocolIE-ID ::= 80

id-UE-associatedLogicalF1-ConnectionListResAck		ProtocolIE-ID ::= 81

id-gNB-CU-Name										ProtocolIE-ID ::= 82

id-SCell-FailedtoSetup-List							ProtocolIE-ID ::= 83

id-SCell-FailedtoSetup-Item							ProtocolIE-ID ::= 84

id-SCell-FailedtoSetupMod-List						ProtocolIE-ID ::= 85

id-SCell-FailedtoSetupMod-Item						ProtocolIE-ID ::= 86

id-RRCReconfigurationCompleteIndicator 				ProtocolIE-ID ::= 87

id-Cells-Status-Item								ProtocolIE-ID ::= 88

id-Cells-Status-List								ProtocolIE-ID ::= 89

id-Candidate-SpCell-List							ProtocolIE-ID ::= 90

id-Candidate-SpCell-Item							ProtocolIE-ID ::= 91

id-Potential-SpCell-List							ProtocolIE-ID ::= 92

id-Potential-SpCell-Item							ProtocolIE-ID ::= 93

id-FullConfiguration								ProtocolIE-ID ::= 94

id-C-RNTI											ProtocolIE-ID ::= 95

id-SpCellULConfigured								ProtocolIE-ID ::= 96

id-InactivityMonitoringRequest						ProtocolIE-ID ::= 97

id-InactivityMonitoringResponse						ProtocolIE-ID ::= 98

id-DRB-Activity-Item								ProtocolIE-ID ::= 99

id-DRB-Activity-List								ProtocolIE-ID ::= 100

id-EUTRA-NR-CellResourceCoordinationReq-Container	ProtocolIE-ID ::= 101

id-EUTRA-NR-CellResourceCoordinationReqAck-Container	ProtocolIE-ID ::= 102

id-Protected-EUTRA-Resources-List					ProtocolIE-ID ::= 105

id-RequestType 										ProtocolIE-ID ::= 106

id-ServCellIndex									ProtocolIE-ID ::= 107

id-RAT-FrequencyPriorityInformation					ProtocolIE-ID ::= 108

id-ExecuteDuplication								ProtocolIE-ID ::= 109

id-NRCGI											ProtocolIE-ID ::= 111

id-PagingCell-Item									ProtocolIE-ID ::= 112

id-PagingCell-List									ProtocolIE-ID ::= 113

id-PagingDRX										ProtocolIE-ID ::= 114

id-PagingPriority 									ProtocolIE-ID ::= 115

id-SItype-List										ProtocolIE-ID ::= 116

id-UEIdentityIndexValue								ProtocolIE-ID ::= 117

id-gNB-CUSystemInformation							ProtocolIE-ID ::= 118

id-HandoverPreparationInformation					ProtocolIE-ID ::= 119

id-GNB-CU-TNL-Association-To-Add-Item				ProtocolIE-ID ::= 120

id-GNB-CU-TNL-Association-To-Add-List				ProtocolIE-ID ::= 121

id-GNB-CU-TNL-Association-To-Remove-Item			ProtocolIE-ID ::= 122

id-GNB-CU-TNL-Association-To-Remove-List			ProtocolIE-ID ::= 123

id-GNB-CU-TNL-Association-To-Update-Item			ProtocolIE-ID ::= 124

id-GNB-CU-TNL-Association-To-Update-List			ProtocolIE-ID ::= 125

id-MaskedIMEISV										ProtocolIE-ID ::= 126

id-PagingIdentity									ProtocolIE-ID ::= 127

id-DUtoCURRCContainer								ProtocolIE-ID ::= 128

id-Cells-to-be-Barred-List							ProtocolIE-ID ::= 129

id-Cells-to-be-Barred-Item							ProtocolIE-ID ::= 130

id-TAISliceSupportList								ProtocolIE-ID ::= 131

id-GNB-CU-TNL-Association-Setup-List				ProtocolIE-ID ::= 132

id-GNB-CU-TNL-Association-Setup-Item				ProtocolIE-ID ::= 133

id-GNB-CU-TNL-Association-Failed-To-Setup-List		ProtocolIE-ID ::= 134

id-GNB-CU-TNL-Association-Failed-To-Setup-Item		ProtocolIE-ID ::= 135

id-DRB-Notify-Item									ProtocolIE-ID ::= 136

id-DRB-Notify-List									ProtocolIE-ID ::= 137

id-NotficationControl								ProtocolIE-ID ::= 138

id-RANAC											ProtocolIE-ID ::= 139

id-PWSSystemInformation								ProtocolIE-ID ::= 140

id-RepetitionPeriod									ProtocolIE-ID ::= 141

id-NumberofBroadcastRequest							ProtocolIE-ID ::= 142

id-Cells-To-Be-Broadcast-List						ProtocolIE-ID ::= 144

id-Cells-To-Be-Broadcast-Item						ProtocolIE-ID ::= 145

id-Cells-Broadcast-Completed-List 					ProtocolIE-ID ::= 146

id-Cells-Broadcast-Completed-Item 					ProtocolIE-ID ::= 147

id-Broadcast-To-Be-Cancelled-List 					ProtocolIE-ID ::= 148

id-Broadcast-To-Be-Cancelled-Item 					ProtocolIE-ID ::= 149

id-Cells-Broadcast-Cancelled-List 					ProtocolIE-ID ::= 150

id-Cells-Broadcast-Cancelled-Item 					ProtocolIE-ID ::= 151

id-NR-CGI-List-For-Restart-List 						ProtocolIE-ID ::= 152

id-NR-CGI-List-For-Restart-Item 						ProtocolIE-ID ::= 153

id-PWS-Failed-NR-CGI-List 							ProtocolIE-ID ::= 154

id-PWS-Failed-NR-CGI-Item 							ProtocolIE-ID ::= 155

id-ConfirmedUEID									ProtocolIE-ID ::= 156

id-Cancel-all-Warning-Messages-Indicator			ProtocolIE-ID ::= 157

id-GNB-DU-UE-AMBR-UL								ProtocolIE-ID ::= 158

id-DRXConfigurationIndicator						ProtocolIE-ID ::= 159

id-RLC-Status										ProtocolIE-ID ::= 160

id-DLPDCPSNLength									ProtocolIE-ID ::= 161

id-GNB-DUConfigurationQuery							ProtocolIE-ID ::= 162

id-MeasurementTimingConfiguration					ProtocolIE-ID ::= 163

id-DRB-Information									ProtocolIE-ID ::= 164

id-ServingPLMN										ProtocolIE-ID ::= 165

id-Protected-EUTRA-Resources-Item					ProtocolIE-ID ::= 168

id-GNB-CU-RRC-Version								ProtocolIE-ID ::= 170

id-GNB-DU-RRC-Version								ProtocolIE-ID ::= 171

id-GNBDUOverloadInformation							ProtocolIE-ID ::= 172

id-CellGroupConfig									ProtocolIE-ID ::= 173

id-RLCFailureIndication								ProtocolIE-ID ::= 174

id-UplinkTxDirectCurrentListInformation				ProtocolIE-ID ::= 175

id-DC-Based-Duplication-Configured					ProtocolIE-ID ::= 176

id-DC-Based-Duplication-Activation					ProtocolIE-ID ::= 177

id-SULAccessIndication								ProtocolIE-ID ::= 178

id-AvailablePLMNList								ProtocolIE-ID ::= 179

id-PDUSessionID										ProtocolIE-ID ::= 180

id-ULPDUSessionAggregateMaximumBitRate				ProtocolIE-ID ::= 181

id-ServingCellMO									ProtocolIE-ID ::= 182

id-QoSFlowMappingIndication							ProtocolIE-ID ::= 183

id-RRCDeliveryStatusRequest							ProtocolIE-ID ::= 184

id-RRCDeliveryStatus								ProtocolIE-ID ::= 185

id-BearerTypeChange									ProtocolIE-ID ::= 186

id-RLCMode											ProtocolIE-ID ::= 187

id-Duplication-Activation							ProtocolIE-ID ::= 188

id-Dedicated-SIDelivery-NeededUE-List				ProtocolIE-ID ::= 189

id-Dedicated-SIDelivery-NeededUE-Item				ProtocolIE-ID ::= 190

id-DRX-LongCycleStartOffset							ProtocolIE-ID ::= 191

id-ULPDCPSNLength									ProtocolIE-ID ::= 192

id-SelectedBandCombinationIndex						ProtocolIE-ID ::= 193

id-SelectedFeatureSetEntryIndex						ProtocolIE-ID ::= 194

id-ResourceCoordinationTransferInformation			ProtocolIE-ID ::= 195

id-ExtendedServedPLMNs-List							ProtocolIE-ID ::= 196

id-ExtendedAvailablePLMN-List						ProtocolIE-ID ::= 197

id-Associated-SCell-List							ProtocolIE-ID ::= 198

id-latest-RRC-Version-Enhanced						ProtocolIE-ID ::= 199

id-Associated-SCell-Item							ProtocolIE-ID ::= 200

id-Cell-Direction									ProtocolIE-ID ::= 201

id-SRBs-Setup-List									ProtocolIE-ID ::= 202

id-SRBs-Setup-Item									ProtocolIE-ID ::= 203

id-SRBs-SetupMod-List								ProtocolIE-ID ::= 204

id-SRBs-SetupMod-Item								ProtocolIE-ID ::= 205

id-SRBs-Modified-List								ProtocolIE-ID ::= 206

id-SRBs-Modified-Item								ProtocolIE-ID ::= 207

id-Ph-InfoSCG										ProtocolIE-ID ::= 208

id-RequestedBandCombinationIndex					ProtocolIE-ID ::= 209

id-RequestedFeatureSetEntryIndex					ProtocolIE-ID ::= 210

id-RequestedP-MaxFR2								ProtocolIE-ID ::= 211

id-DRX-Config										ProtocolIE-ID ::= 212

id-IgnoreResourceCoordinationContainer				ProtocolIE-ID ::= 213

id-UEAssistanceInformation							ProtocolIE-ID ::= 214

id-NeedforGap										ProtocolIE-ID ::= 215

id-PagingOrigin										ProtocolIE-ID ::= 216

id-new-gNB-CU-UE-F1AP-ID							ProtocolIE-ID ::= 217

id-RedirectedRRCmessage								ProtocolIE-ID ::= 218

id-new-gNB-DU-UE-F1AP-ID							ProtocolIE-ID ::= 219

id-NotificationInformation							ProtocolIE-ID ::= 220

id-PLMNAssistanceInfoForNetShar						ProtocolIE-ID ::= 221

id-UEContextNotRetrievable							ProtocolIE-ID ::= 222

id-BPLMN-ID-Info-List								ProtocolIE-ID ::= 223

id-SelectedPLMNID									ProtocolIE-ID ::= 224

id-UAC-Assistance-Info								ProtocolIE-ID ::= 225

id-RANUEID											ProtocolIE-ID ::= 226

id-GNB-DU-TNL-Association-To-Remove-Item			ProtocolIE-ID ::= 227

id-GNB-DU-TNL-Association-To-Remove-List			ProtocolIE-ID ::= 228

id-TNLAssociationTransportLayerAddressgNBDU			ProtocolIE-ID ::= 229

id-portNumber										ProtocolIE-ID ::= 230

id-AdditionalSIBMessageList							ProtocolIE-ID ::= 231

id-Cell-Type										ProtocolIE-ID ::= 232

id-IgnorePRACHConfiguration							ProtocolIE-ID ::= 233

id-CG-Config										ProtocolIE-ID ::= 234

id-PDCCH-BlindDetectionSCG							ProtocolIE-ID ::= 235

id-Requested-PDCCH-BlindDetectionSCG				ProtocolIE-ID ::= 236

id-Ph-InfoMCG										ProtocolIE-ID ::= 237

id-MeasGapSharingConfig								ProtocolIE-ID ::= 238

id-systemInformationAreaID							ProtocolIE-ID ::= 239

id-areaScope										ProtocolIE-ID ::= 240

id-RRCContainer-RRCSetupComplete					ProtocolIE-ID ::= 241

id-TraceActivation									ProtocolIE-ID ::= 242

id-TraceID											ProtocolIE-ID ::= 243

id-Neighbour-Cell-Information-List					ProtocolIE-ID ::= 244

id-SymbolAllocInSlot								ProtocolIE-ID ::= 246

id-NumDLULSymbols									ProtocolIE-ID ::= 247

id-AdditionalRRMPriorityIndex						ProtocolIE-ID ::= 248

id-DUCURadioInformationType							ProtocolIE-ID ::= 249

id-CUDURadioInformationType 						ProtocolIE-ID ::= 250

id-AggressorgNBSetID								ProtocolIE-ID ::= 251

id-VictimgNBSetID									ProtocolIE-ID ::= 252

id-LowerLayerPresenceStatusChange					ProtocolIE-ID ::= 253

id-Transport-Layer-Address-Info						ProtocolIE-ID ::= 254

id-Neighbour-Cell-Information-Item					ProtocolIE-ID ::= 255

id-IntendedTDD-DL-ULConfig							ProtocolIE-ID ::= 256

id-QosMonitoringRequest								ProtocolIE-ID ::= 257

id-BHChannels-ToBeSetup-List						ProtocolIE-ID ::= 258

id-BHChannels-ToBeSetup-Item						ProtocolIE-ID ::= 259

id-BHChannels-Setup-List							ProtocolIE-ID ::= 260

id-BHChannels-Setup-Item							ProtocolIE-ID ::= 261

id-BHChannels-ToBeModified-Item						ProtocolIE-ID ::= 262

id-BHChannels-ToBeModified-List						ProtocolIE-ID ::= 263

id-BHChannels-ToBeReleased-Item						ProtocolIE-ID ::= 264

id-BHChannels-ToBeReleased-List						ProtocolIE-ID ::= 265

id-BHChannels-ToBeSetupMod-Item						ProtocolIE-ID ::= 266

id-BHChannels-ToBeSetupMod-List						ProtocolIE-ID ::= 267

id-BHChannels-FailedToBeModified-Item				ProtocolIE-ID ::= 268

id-BHChannels-FailedToBeModified-List				ProtocolIE-ID ::= 269

id-BHChannels-FailedToBeSetupMod-Item				ProtocolIE-ID ::= 270

id-BHChannels-FailedToBeSetupMod-List				ProtocolIE-ID ::= 271

id-BHChannels-Modified-Item							ProtocolIE-ID ::= 272

id-BHChannels-Modified-List							ProtocolIE-ID ::= 273

id-BHChannels-SetupMod-Item							ProtocolIE-ID ::= 274

id-BHChannels-SetupMod-List							ProtocolIE-ID ::= 275

id-BHChannels-Required-ToBeReleased-Item			ProtocolIE-ID ::= 276

id-BHChannels-Required-ToBeReleased-List			ProtocolIE-ID ::= 277

id-BHChannels-FailedToBeSetup-Item					ProtocolIE-ID ::= 278

id-BHChannels-FailedToBeSetup-List					ProtocolIE-ID ::= 279

id-BHInfo											ProtocolIE-ID ::= 280

id-BAPAddress										ProtocolIE-ID ::= 281

id-ConfiguredBAPAddress								ProtocolIE-ID ::= 282

id-BH-Routing-Information-Added-List				ProtocolIE-ID ::= 283

id-BH-Routing-Information-Added-List-Item			ProtocolIE-ID ::= 284

id-BH-Routing-Information-Removed-List				ProtocolIE-ID ::= 285

id-BH-Routing-Information-Removed-List-Item			ProtocolIE-ID ::= 286

id-UL-BH-Non-UP-Traffic-Mapping						ProtocolIE-ID ::= 287

id-Activated-Cells-to-be-Updated-List				ProtocolIE-ID ::= 288

id-Child-Nodes-List									ProtocolIE-ID ::= 289

id-IAB-Info-IAB-DU									ProtocolIE-ID ::= 290

id-IAB-Info-IAB-donor-CU							ProtocolIE-ID ::= 291

id-IAB-TNL-Addresses-To-Remove-List					ProtocolIE-ID ::= 292

id-IAB-TNL-Addresses-To-Remove-Item					ProtocolIE-ID ::= 293

id-IAB-Allocated-TNL-Address-List					ProtocolIE-ID ::= 294

id-IAB-Allocated-TNL-Address-Item					ProtocolIE-ID ::= 295

id-IABIPv6RequestType								ProtocolIE-ID ::= 296

id-IABv4AddressesRequested							ProtocolIE-ID ::= 297

id-IAB-Barred										ProtocolIE-ID ::= 298

id-TrafficMappingInformation						ProtocolIE-ID ::= 299

id-UL-UP-TNL-Information-to-Update-List				ProtocolIE-ID ::= 300

id-UL-UP-TNL-Information-to-Update-List-Item		ProtocolIE-ID ::= 301

id-UL-UP-TNL-Address-to-Update-List					ProtocolIE-ID ::= 302

id-UL-UP-TNL-Address-to-Update-List-Item			ProtocolIE-ID ::= 303

id-DL-UP-TNL-Address-to-Update-List					ProtocolIE-ID ::= 304

id-DL-UP-TNL-Address-to-Update-List-Item			ProtocolIE-ID ::= 305

id-NRV2XServicesAuthorized							ProtocolIE-ID ::= 306

id-LTEV2XServicesAuthorized							ProtocolIE-ID ::= 307

id-NRUESidelinkAggregateMaximumBitrate				ProtocolIE-ID ::= 308

id-LTEUESidelinkAggregateMaximumBitrate				ProtocolIE-ID ::= 309

id-SIB12-message									ProtocolIE-ID ::= 310

id-SIB13-message									ProtocolIE-ID ::= 311

id-SIB14-message									ProtocolIE-ID ::= 312

id-SLDRBs-FailedToBeModified-Item					ProtocolIE-ID ::= 313

id-SLDRBs-FailedToBeModified-List					ProtocolIE-ID ::= 314

id-SLDRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 315

id-SLDRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 316

id-SLDRBs-Modified-Item								ProtocolIE-ID ::= 317

id-SLDRBs-Modified-List								ProtocolIE-ID ::= 318

id-SLDRBs-Required-ToBeModified-Item				ProtocolIE-ID ::= 319

id-SLDRBs-Required-ToBeModified-List				ProtocolIE-ID ::= 320

id-SLDRBs-Required-ToBeReleased-Item				ProtocolIE-ID ::= 321

id-SLDRBs-Required-ToBeReleased-List				ProtocolIE-ID ::= 322

id-SLDRBs-Setup-Item								ProtocolIE-ID ::= 323

id-SLDRBs-Setup-List								ProtocolIE-ID ::= 324

id-SLDRBs-ToBeModified-Item							ProtocolIE-ID ::= 325

id-SLDRBs-ToBeModified-List							ProtocolIE-ID ::= 326

id-SLDRBs-ToBeReleased-Item							ProtocolIE-ID ::= 327

id-SLDRBs-ToBeReleased-List							ProtocolIE-ID ::= 328

id-SLDRBs-ToBeSetup-Item							ProtocolIE-ID ::= 329

id-SLDRBs-ToBeSetup-List							ProtocolIE-ID ::= 330

id-SLDRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 331

id-SLDRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 332

id-SLDRBs-SetupMod-List								ProtocolIE-ID ::= 333

id-SLDRBs-FailedToBeSetupMod-List					ProtocolIE-ID ::= 334

id-SLDRBs-SetupMod-Item								ProtocolIE-ID ::= 335

id-SLDRBs-FailedToBeSetupMod-Item					ProtocolIE-ID ::= 336

id-SLDRBs-ModifiedConf-List							ProtocolIE-ID ::= 337

id-SLDRBs-ModifiedConf-Item							ProtocolIE-ID ::= 338

id-UEAssistanceInformationEUTRA						ProtocolIE-ID ::= 339

id-PC5LinkAMBR										ProtocolIE-ID ::= 340

id-SL-PHY-MAC-RLC-Config							ProtocolIE-ID ::= 341

id-SL-ConfigDedicatedEUTRA-Info						ProtocolIE-ID ::= 342

id-AlternativeQoSParaSetList						ProtocolIE-ID ::= 343

id-CurrentQoSParaSetIndex							ProtocolIE-ID ::= 344

id-gNBCUMeasurementID								ProtocolIE-ID ::= 345

id-gNBDUMeasurementID								ProtocolIE-ID ::= 346

id-RegistrationRequest								ProtocolIE-ID ::= 347

id-ReportCharacteristics							ProtocolIE-ID ::= 348

id-CellToReportList									ProtocolIE-ID ::= 349

id-CellMeasurementResultList						ProtocolIE-ID ::= 350

id-HardwareLoadIndicator							ProtocolIE-ID ::= 351

id-ReportingPeriodicity								ProtocolIE-ID ::= 352

id-TNLCapacityIndicator								ProtocolIE-ID ::= 353

id-CarrierList										ProtocolIE-ID ::= 354

id-ULCarrierList									ProtocolIE-ID ::= 355

id-FrequencyShift7p5khz								ProtocolIE-ID ::= 356

id-SSB-PositionsInBurst								ProtocolIE-ID ::= 357

id-NRPRACHConfig									ProtocolIE-ID ::= 358

id-RACHReportInformationList						ProtocolIE-ID ::= 359

id-RLFReportInformationList							ProtocolIE-ID ::= 360

id-TDD-UL-DLConfigCommonNR							ProtocolIE-ID ::= 361

id-CNPacketDelayBudgetDownlink						ProtocolIE-ID ::= 362

id-ExtendedPacketDelayBudget						ProtocolIE-ID ::= 363

id-TSCTrafficCharacteristics						ProtocolIE-ID ::= 364

id-ReportingRequestType								ProtocolIE-ID ::= 365

id-TimeReferenceInformation							ProtocolIE-ID ::= 366

id-CNPacketDelayBudgetUplink						ProtocolIE-ID ::= 369

id-AdditionalPDCPDuplicationTNL-List				ProtocolIE-ID ::= 370

id-RLCDuplicationInformation						ProtocolIE-ID ::= 371

id-AdditionalDuplicationIndication					ProtocolIE-ID ::= 372

id-ConditionalInterDUMobilityInformation			ProtocolIE-ID ::= 373

id-ConditionalIntraDUMobilityInformation			ProtocolIE-ID ::= 374

id-targetCellsToCancel								ProtocolIE-ID ::= 375

id-requestedTargetCellGlobalID						ProtocolIE-ID ::= 376

id-ManagementBasedMDTPLMNList						ProtocolIE-ID ::= 377

id-TraceCollectionEntityIPAddress 					ProtocolIE-ID ::= 378

id-PrivacyIndicator									ProtocolIE-ID ::= 379

id-TraceCollectionEntityURI							ProtocolIE-ID ::= 380

id-mdtConfiguration									ProtocolIE-ID ::= 381

id-ServingNID										ProtocolIE-ID ::= 382

id-NPNBroadcastInformation							ProtocolIE-ID ::= 383

id-NPNSupportInfo									ProtocolIE-ID ::= 384

id-NID												ProtocolIE-ID ::= 385

id-AvailableSNPN-ID-List							ProtocolIE-ID ::= 386

id-SIB10-message									ProtocolIE-ID ::= 387

id-DLCarrierList									ProtocolIE-ID ::= 389

id-ExtendedTAISliceSupportList					ProtocolIE-ID ::= 390

id-RequestedSRSTransmissionCharacteristics			ProtocolIE-ID ::= 391

id-PosAssistance-Information						ProtocolIE-ID ::= 392

id-PosBroadcast										ProtocolIE-ID ::= 393

id-RoutingID										ProtocolIE-ID ::= 394

id-PosAssistanceInformationFailureList				ProtocolIE-ID ::= 395

id-PosMeasurementQuantities							ProtocolIE-ID ::= 396

id-PosMeasurementResultList							ProtocolIE-ID ::= 397

id-TRPInformationTypeListTRPReq						ProtocolIE-ID ::= 398

id-TRPInformationTypeItem							ProtocolIE-ID ::= 399

id-TRPInformationListTRPResp						ProtocolIE-ID ::= 400

id-TRPInformationItem								ProtocolIE-ID ::= 401

id-LMF-MeasurementID								ProtocolIE-ID ::= 402

id-SRSType											ProtocolIE-ID ::= 403

id-ActivationTime									ProtocolIE-ID ::= 404

id-AbortTransmission								ProtocolIE-ID ::= 405

id-PositioningBroadcastCells						ProtocolIE-ID ::= 406

id-SRSConfiguration									ProtocolIE-ID ::= 407

id-PosReportCharacteristics							ProtocolIE-ID ::= 408

id-PosMeasurementPeriodicity						ProtocolIE-ID ::= 409

id-TRPList											ProtocolIE-ID ::= 410

id-RAN-MeasurementID								ProtocolIE-ID ::= 411

id-LMF-UE-MeasurementID								ProtocolIE-ID ::= 412

id-RAN-UE-MeasurementID								ProtocolIE-ID ::= 413

id-E-CID-MeasurementQuantities						ProtocolIE-ID ::= 414

id-E-CID-MeasurementQuantities-Item					ProtocolIE-ID ::= 415

id-E-CID-MeasurementPeriodicity						ProtocolIE-ID ::= 416

id-E-CID-MeasurementResult							ProtocolIE-ID ::= 417

id-Cell-Portion-ID									ProtocolIE-ID ::= 418

id-SFNInitialisationTime							ProtocolIE-ID ::= 419

id-SystemFrameNumber								ProtocolIE-ID ::= 420

id-SlotNumber										ProtocolIE-ID ::= 421

id-TRP-MeasurementRequestList						ProtocolIE-ID ::= 422

id-MeasurementBeamInfoRequest						ProtocolIE-ID ::= 423

id-E-CID-ReportCharacteristics						ProtocolIE-ID ::= 424

id-ConfiguredTACIndication							ProtocolIE-ID ::= 425

id-Extended-GNB-CU-Name								ProtocolIE-ID ::= 426

id-Extended-GNB-DU-Name								ProtocolIE-ID ::= 427

id-F1CTransferPath									ProtocolIE-ID ::= 428

id-SFN-Offset										ProtocolIE-ID ::= 429

id-TransmissionStopIndicator						ProtocolIE-ID ::= 430

id-SrsFrequency										ProtocolIE-ID ::= 431

id-SCGIndicator										ProtocolIE-ID ::= 432

id-EstimatedArrivalProbability						ProtocolIE-ID ::= 433

id-TRPType											ProtocolIE-ID ::= 434

id-SRSSpatialRelationPerSRSResource					ProtocolIE-ID ::= 435

id-PDCPTerminatingNodeDLTNLAddrInfo					ProtocolIE-ID ::= 436

id-ENBDLTNLAddress									ProtocolIE-ID ::= 437

id-PosMeasurementPeriodicityExtended				ProtocolIE-ID ::= 438

id-PRS-Resource-ID									ProtocolIE-ID ::= 439

id-LocationMeasurementInformation					ProtocolIE-ID ::= 440

id-SliceRadioResourceStatus							ProtocolIE-ID ::= 441

id-CompositeAvailableCapacity-SUL					ProtocolIE-ID ::= 442

id-SuccessfulHOReportInformationList				ProtocolIE-ID ::= 443

id-NR-U-Channel-List								ProtocolIE-ID ::= 444

id-NR-U												ProtocolIE-ID ::= 445

id-Coverage-Modification-Notification				ProtocolIE-ID ::= 446

id-CCO-Assistance-Information						ProtocolIE-ID ::= 447

id-Neighbor-node-CCO-Assistance-Information-List	ProtocolIE-ID ::= 448

id-CellsForSON-List									ProtocolIE-ID ::= 449

id-MIMOPRBusageInformation							ProtocolIE-ID ::= 450

id-gNB-CU-MBS-F1AP-ID								ProtocolIE-ID ::= 451

id-gNB-DU-MBS-F1AP-ID								ProtocolIE-ID ::= 452

id-MBS-Area-Session-ID								ProtocolIE-ID ::= 453

id-MBS-CUtoDURRCInformation							ProtocolIE-ID ::= 454

id-MBS-Session-ID									ProtocolIE-ID ::= 455

id-SNSSAI											ProtocolIE-ID ::= 456

id-MBS-Broadcast-NeighbourCellList					ProtocolIE-ID ::= 457

id-BroadcastMRBs-FailedToBeModified-List			ProtocolIE-ID ::= 458

id-BroadcastMRBs-FailedToBeModified-Item			ProtocolIE-ID ::= 459

id-BroadcastMRBs-FailedToBeSetup-List				ProtocolIE-ID ::= 460

id-BroadcastMRBs-FailedToBeSetup-Item				ProtocolIE-ID ::= 461

id-BroadcastMRBs-FailedToBeSetupMod-List			ProtocolIE-ID ::= 462

id-BroadcastMRBs-FailedToBeSetupMod-Item			ProtocolIE-ID ::= 463

id-BroadcastMRBs-Modified-List						ProtocolIE-ID ::= 464

id-BroadcastMRBs-Modified-Item						ProtocolIE-ID ::= 465

id-BroadcastMRBs-Setup-List							ProtocolIE-ID ::= 466

id-BroadcastMRBs-Setup-Item							ProtocolIE-ID ::= 467

id-BroadcastMRBs-SetupMod-List						ProtocolIE-ID ::= 468

id-BroadcastMRBs-SetupMod-Item						ProtocolIE-ID ::= 469

id-BroadcastMRBs-ToBeModified-List					ProtocolIE-ID ::= 470

id-BroadcastMRBs-ToBeModified-Item					ProtocolIE-ID ::= 471

id-BroadcastMRBs-ToBeReleased-List					ProtocolIE-ID ::= 472

id-BroadcastMRBs-ToBeReleased-Item					ProtocolIE-ID ::= 473

id-BroadcastMRBs-ToBeSetup-List						ProtocolIE-ID ::= 474

id-BroadcastMRBs-ToBeSetup-Item						ProtocolIE-ID ::= 475

id-BroadcastMRBs-ToBeSetupMod-List					ProtocolIE-ID ::= 476

id-BroadcastMRBs-ToBeSetupMod-Item					ProtocolIE-ID ::= 477

id-Supported-MBS-FSA-ID-List						ProtocolIE-ID ::= 478

id-UEIdentity-List-For-Paging-List 					ProtocolIE-ID ::= 479

id-UEIdentity-List-For-Paging-Item 					ProtocolIE-ID ::= 480

id-MBS-ServiceArea									ProtocolIE-ID ::= 481

id-MulticastMRBs-FailedToBeModified-List			ProtocolIE-ID ::= 482

id-MulticastMRBs-FailedToBeModified-Item			ProtocolIE-ID ::= 483

id-MulticastMRBs-FailedToBeSetup-List				ProtocolIE-ID ::= 484

id-MulticastMRBs-FailedToBeSetup-Item				ProtocolIE-ID ::= 485

id-MulticastMRBs-FailedToBeSetupMod-List			ProtocolIE-ID ::= 486

id-MulticastMRBs-FailedToBeSetupMod-Item			ProtocolIE-ID ::= 487

id-MulticastMRBs-Modified-List						ProtocolIE-ID ::= 488

id-MulticastMRBs-Modified-Item						ProtocolIE-ID ::= 489

id-MulticastMRBs-Setup-List							ProtocolIE-ID ::= 490

id-MulticastMRBs-Setup-Item							ProtocolIE-ID ::= 491

id-MulticastMRBs-SetupMod-List						ProtocolIE-ID ::= 492

id-MulticastMRBs-SetupMod-Item						ProtocolIE-ID ::= 493

id-MulticastMRBs-ToBeModified-List					ProtocolIE-ID ::= 494

id-MulticastMRBs-ToBeModified-Item					ProtocolIE-ID ::= 495

id-MulticastMRBs-ToBeReleased-List					ProtocolIE-ID ::= 496

id-MulticastMRBs-ToBeReleased-Item					ProtocolIE-ID ::= 497

id-MulticastMRBs-ToBeSetup-List						ProtocolIE-ID ::= 498

id-MulticastMRBs-ToBeSetup-Item						ProtocolIE-ID ::= 499

id-MulticastMRBs-ToBeSetupMod-List					ProtocolIE-ID ::= 500

id-MulticastMRBs-ToBeSetupMod-Item					ProtocolIE-ID ::= 501

id-MBSMulticastF1UContextDescriptor					ProtocolIE-ID ::= 502

id-MulticastF1UContext-ToBeSetup-List				ProtocolIE-ID ::= 503

id-MulticastF1UContext-ToBeSetup-Item				ProtocolIE-ID ::= 504

id-MulticastF1UContext-Setup-List					ProtocolIE-ID ::= 505

id-MulticastF1UContext-Setup-Item					ProtocolIE-ID ::= 506

id-MulticastF1UContext-FailedToBeSetup-List			ProtocolIE-ID ::= 507

id-MulticastF1UContext-FailedToBeSetup-Item			ProtocolIE-ID ::= 508

id-IABCongestionIndication							ProtocolIE-ID ::= 509

id-IABConditionalRRCMessageDeliveryIndication		ProtocolIE-ID ::= 510

id-F1CTransferPathNRDC								ProtocolIE-ID ::= 511

id-BufferSizeThresh 								ProtocolIE-ID ::= 512

id-IAB-TNL-Addresses-Exception						ProtocolIE-ID ::= 513

id-BAP-Header-Rewriting-Added-List					ProtocolIE-ID ::= 514

id-BAP-Header-Rewriting-Added-List-Item				ProtocolIE-ID ::= 515

id-Re-routingEnableIndicator						ProtocolIE-ID ::= 516

id-NonF1terminatingTopologyIndicator				ProtocolIE-ID ::= 517

id-EgressNonF1terminatingTopologyIndicator			ProtocolIE-ID ::= 518

id-IngressNonF1terminatingTopologyIndicator			ProtocolIE-ID ::= 519

id-rBSetConfiguration 								ProtocolIE-ID ::= 520

id-frequency-Domain-HSNA-Configuration-List			ProtocolIE-ID ::= 521

id-child-IAB-Nodes-NA-Resource-List					ProtocolIE-ID ::= 522

id-Parent-IAB-Nodes-NA-Resource-Configuration-List	ProtocolIE-ID ::= 523

id-uL-FreqInfo										ProtocolIE-ID ::= 524

id-uL-Transmission-Bandwidth						ProtocolIE-ID ::= 525

id-dL-FreqInfo										ProtocolIE-ID ::= 526

id-dL-Transmission-Bandwidth						ProtocolIE-ID ::= 527

id-uL-NR-Carrier-List								ProtocolIE-ID ::= 528

id-dL-NR-Carrier-List								ProtocolIE-ID ::= 529

id-nRFreqInfo										ProtocolIE-ID ::= 530

id-transmission-Bandwidth							ProtocolIE-ID ::= 531

id-nR-Carrier-List									ProtocolIE-ID ::= 532

id-Neighbour-Node-Cells-List						ProtocolIE-ID ::= 533

id-Serving-Cells-List								ProtocolIE-ID ::= 534

id-permutation										ProtocolIE-ID ::= 535

id-MDTPollutedMeasurementIndicator					ProtocolIE-ID ::= 536

id-M5ReportAmount 									ProtocolIE-ID ::= 537

id-M6ReportAmount 									ProtocolIE-ID ::= 538

id-M7ReportAmount 									ProtocolIE-ID ::= 539

id-SurvivalTime										ProtocolIE-ID ::= 540

id-PDCMeasurementPeriodicity						ProtocolIE-ID ::= 541

id-PDCMeasurementQuantities							ProtocolIE-ID ::= 542

id-PDCMeasurementQuantities-Item					ProtocolIE-ID ::= 543

id-PDCMeasurementResult								ProtocolIE-ID ::= 544

id-PDCReportType									ProtocolIE-ID ::= 545

id-RAN-UE-PDC-MeasID								ProtocolIE-ID ::= 546

id-SCGActivationRequest								ProtocolIE-ID ::= 547

id-SCGActivationStatus								ProtocolIE-ID ::= 548

id-PRSTRPList										ProtocolIE-ID ::= 549

id-PRSTransmissionTRPList							ProtocolIE-ID ::= 550

id-OnDemandPRS										ProtocolIE-ID ::= 551

id-AoA-SearchWindow									ProtocolIE-ID ::= 552

id-TRP-MeasurementUpdateList						ProtocolIE-ID ::= 553

id-ZoAInformation									ProtocolIE-ID ::= 554

id-ResponseTime										ProtocolIE-ID ::= 555

id-ARPLocationInfo									ProtocolIE-ID ::= 556

id-ARP-ID											ProtocolIE-ID ::= 557

id-MultipleULAoA									ProtocolIE-ID ::= 558

id-UL-SRS-RSRPP										ProtocolIE-ID ::= 559

id-SRSResourcetype									ProtocolIE-ID ::= 560

id-ExtendedAdditionalPathList						ProtocolIE-ID ::= 561

id-LoS-NLoSInformation								ProtocolIE-ID ::= 562

id-NumberOfTRPRxTEG									ProtocolIE-ID ::= 564

id-NumberOfTRPRxTxTEG								ProtocolIE-ID ::= 565

id-TRPTxTEGAssociation								ProtocolIE-ID ::= 566

id-TRPTEGInformation								ProtocolIE-ID ::= 567

id-TRPRx-TEGInformation								ProtocolIE-ID ::= 568

id-TRP-PRS-Info-List								ProtocolIE-ID ::= 569

id-PRS-Measurement-Info-List						ProtocolIE-ID ::= 570

id-PRSConfigRequestType								ProtocolIE-ID ::= 571

id-MeasurementTimeOccasion							ProtocolIE-ID ::= 573

id-MeasurementCharacteristicsRequestIndicator		ProtocolIE-ID ::= 574

id-UEReportingInformation							ProtocolIE-ID ::= 575

id-PosConextRevIndication							ProtocolIE-ID ::= 576

id-TRPBeamAntennaInformation						ProtocolIE-ID ::= 577

id-NRRedCapUEIndication 							ProtocolIE-ID ::= 578

id-Redcap-Bcast-Information							ProtocolIE-ID ::= 579

id-RANUEPagingDRX									ProtocolIE-ID ::= 580

id-CNUEPagingDRX									ProtocolIE-ID ::= 581

id-NRPagingeDRXInformation							ProtocolIE-ID ::= 582

id-NRPagingeDRXInformationforRRCINACTIVE			ProtocolIE-ID ::= 583

id-NR-TADV											ProtocolIE-ID ::= 584

id-QoEInformation									ProtocolIE-ID ::= 585

id-CG-SDTQueryIndication							ProtocolIE-ID ::= 586

id-SDT-MAC-PHY-CG-Config							ProtocolIE-ID ::= 587

id-CG-SDTKeptIndicator								ProtocolIE-ID ::= 588

id-CG-SDTindicatorSetup								ProtocolIE-ID ::= 589

id-CG-SDTindicatorMod								ProtocolIE-ID ::= 590

id-CG-SDTSessionInfoOld								ProtocolIE-ID ::= 591

id-SDTInformation									ProtocolIE-ID ::= 592

id-SDTRLCBearerConfiguration						ProtocolIE-ID ::= 593

id-FiveG-ProSeAuthorized							ProtocolIE-ID ::= 594

id-FiveG-ProSeUEPC5AggregateMaximumBitrate			ProtocolIE-ID ::= 595

id-FiveG-ProSePC5LinkAMBR	 						ProtocolIE-ID ::= 596

id-SRBMappingInfo									ProtocolIE-ID ::= 597

id-DRBMappingInfo									ProtocolIE-ID ::= 598

id-UuRLCChannelToBeSetupList						ProtocolIE-ID ::= 599

id-UuRLCChannelToBeModifiedList						ProtocolIE-ID ::= 600

id-UuRLCChannelToBeReleasedList						ProtocolIE-ID ::= 601

id-UuRLCChannelSetupList							ProtocolIE-ID ::= 602

id-UuRLCChannelFailedToBeSetupList					ProtocolIE-ID ::= 603

id-UuRLCChannelModifiedList							ProtocolIE-ID ::= 604

id-UuRLCChannelFailedToBeModifiedList				ProtocolIE-ID ::= 605

id-UuRLCChannelRequiredToBeModifiedList				ProtocolIE-ID ::= 606

id-UuRLCChannelRequiredToBeReleasedList				ProtocolIE-ID ::= 607

id-PC5RLCChannelToBeSetupList						ProtocolIE-ID ::= 608

id-PC5RLCChannelToBeModifiedList					ProtocolIE-ID ::= 609

id-PC5RLCChannelToBeReleasedList					ProtocolIE-ID ::= 610

id-PC5RLCChannelSetupList							ProtocolIE-ID ::= 611

id-PC5RLCChannelFailedToBeSetupList					ProtocolIE-ID ::= 612

id-PC5RLCChannelFailedToBeModifiedList				ProtocolIE-ID ::= 613

id-PC5RLCChannelRequiredToBeModifiedList			ProtocolIE-ID ::= 614

id-PC5RLCChannelRequiredToBeReleasedList			ProtocolIE-ID ::= 615

id-PC5RLCChannelModifiedList						ProtocolIE-ID ::= 616

id-SidelinkRelayConfiguration						ProtocolIE-ID ::= 617

id-UpdatedRemoteUELocalID							ProtocolIE-ID ::= 618

id-PathSwitchConfiguration							ProtocolIE-ID ::= 619

id-PagingCause										ProtocolIE-ID ::= 620

id-MUSIM-GapConfig									ProtocolIE-ID ::= 621

id-PEIPSAssistanceInfo								ProtocolIE-ID ::= 622

id-UEPagingCapability								ProtocolIE-ID ::= 623

id-LastUsedCellIndication							ProtocolIE-ID ::= 624

id-SIB17-message									ProtocolIE-ID ::= 625

id-GNBDUUESliceMaximumBitRateList					ProtocolIE-ID ::= 626

id-SIB20-message									ProtocolIE-ID ::= 627

id-UE-MulticastMRBs-ToBeReleased-List				ProtocolIE-ID ::= 628

id-UE-MulticastMRBs-ToBeReleased-Item				ProtocolIE-ID ::= 629

id-UE-MulticastMRBs-ToBeSetup-List					ProtocolIE-ID ::= 630

id-UE-MulticastMRBs-ToBeSetup-Item					ProtocolIE-ID ::= 631

id-MulticastMBSSessionSetupList						ProtocolIE-ID ::= 632

id-MulticastMBSSessionRemoveList					ProtocolIE-ID ::= 633

id-PosMeasurementAmount								ProtocolIE-ID ::= 634

id-SDT-Termination-Request							ProtocolIE-ID ::= 635

id-pathPower										ProtocolIE-ID ::= 636

id-DU-RX-MT-RX-Extend								ProtocolIE-ID ::= 637

id-DU-TX-MT-TX-Extend								ProtocolIE-ID ::= 638

id-DU-RX-MT-TX-Extend								ProtocolIE-ID ::= 639

id-DU-TX-MT-RX-Extend								ProtocolIE-ID ::= 640

id-BAP-Header-Rewriting-Removed-List				ProtocolIE-ID ::= 641

id-BAP-Header-Rewriting-Removed-List-Item			ProtocolIE-ID ::= 642

id-SLDRXCycleList									ProtocolIE-ID ::= 643

id-TAINSAGSupportList								ProtocolIE-ID ::= 644

id-SL-RLC-ChannelToAddModList						ProtocolIE-ID ::= 645

id-BroadcastAreaScope								ProtocolIE-ID ::= 646

id-ManagementBasedMDTPLMNModificationList 			ProtocolIE-ID ::= 647

id-SIB15-message									ProtocolIE-ID ::= 648

id-ActivationRequestType							ProtocolIE-ID ::= 649

id-PosMeasGapPreConfigList							ProtocolIE-ID ::= 650

id-InterFrequencyConfig-NoGap						ProtocolIE-ID ::= 651

id-MBSInterestIndication							ProtocolIE-ID ::= 652

id-UE-MulticastMRBs-ConfirmedToBeModified-List		ProtocolIE-ID ::= 653

id-UE-MulticastMRBs-ConfirmedToBeModified-Item		ProtocolIE-ID ::= 654

id-UE-MulticastMRBs-RequiredToBeModified-List		ProtocolIE-ID ::= 655

id-UE-MulticastMRBs-RequiredToBeModified-Item		ProtocolIE-ID ::= 656

id-UE-MulticastMRBs-RequiredToBeReleased-List		ProtocolIE-ID ::= 657

id-UE-MulticastMRBs-RequiredToBeReleased-Item		ProtocolIE-ID ::= 658

id-L571Info											ProtocolIE-ID ::= 659

id-L1151Info										ProtocolIE-ID ::= 660

id-SCS-480											ProtocolIE-ID ::= 661

id-SCS-960											ProtocolIE-ID ::= 662

id-SRSPortIndex										ProtocolIE-ID ::= 663

id-PEISubgroupingSupportIndication					ProtocolIE-ID ::= 664

id-NeedForGapsInfoNR								ProtocolIE-ID ::= 665

id-NeedForGapNCSGInfoNR								ProtocolIE-ID ::= 666

id-NeedForGapNCSGInfoEUTRA							ProtocolIE-ID ::= 667

id-procedure-code-668-not-to-be-used				ProtocolIE-ID ::= 668

id-procedure-code-669-not-to-be-used				ProtocolIE-ID ::= 669

id-procedure-code-670-not-to-be-used				ProtocolIE-ID ::= 670

id-Source-MRB-ID									ProtocolIE-ID ::= 671

id-PosMeasurementPeriodicityNR-AoA					ProtocolIE-ID ::= 672

id-RedCapIndication									ProtocolIE-ID ::= 673

id-SRSPosRRCInactiveConfig							ProtocolIE-ID ::= 674

id-SDTBearerConfigurationQueryIndication			ProtocolIE-ID ::= 675

id-SDTBearerConfigurationInfo						ProtocolIE-ID ::= 676

id-UL-GapFR2-Config									ProtocolIE-ID ::= 677

id-ConfigRestrictInfoDAPS							ProtocolIE-ID ::= 678

id-UE-MulticastMRBs-Setup-List						ProtocolIE-ID ::= 679

id-UE-MulticastMRBs-Setup-Item						ProtocolIE-ID ::= 680

id-MulticastF1UContextReferenceCU					ProtocolIE-ID ::= 681

id-PosSItypeList									ProtocolIE-ID ::= 682

id-DAPS-HO-Status									ProtocolIE-ID ::= 683

id-UplinkTxDirectCurrentTwoCarrierListInfo			ProtocolIE-ID ::= 684

id-UE-MulticastMRBs-ToBeSetup-atModify-List			ProtocolIE-ID ::= 685

id-UE-MulticastMRBs-ToBeSetup-atModify-Item			ProtocolIE-ID ::= 686

id-MC-PagingCell-List								ProtocolIE-ID ::= 687

id-MC-PagingCell-Item								ProtocolIE-ID ::= 688

id-SRSPosRRCInactiveQueryIndication					ProtocolIE-ID ::= 689

id-UlTxDirectCurrentMoreCarrierInformation	        ProtocolIE-ID ::= 690

id-CPACMCGInformation								ProtocolIE-ID ::= 691

id-TwoPHRModeMCG									ProtocolIE-ID ::= 692

id-TwoPHRModeSCG									ProtocolIE-ID ::= 693

id-ExtendedUEIdentityIndexValue						ProtocolIE-ID ::= 694

id-ServingCellMO-List								ProtocolIE-ID ::= 695

id-ServingCellMO-List-Item							ProtocolIE-ID ::= 696

id-ServingCellMO-encoded-in-CGC-List				ProtocolIE-ID ::= 697

id-HashedUEIdentityIndexValue						ProtocolIE-ID ::= 698

id-UE-MulticastMRBs-Setupnew-List					ProtocolIE-ID ::= 699

id-UE-MulticastMRBs-Setupnew-Item					ProtocolIE-ID ::= 700

id-ncd-SSB-RedCapInitialBWP-SDT						ProtocolIE-ID ::= 701

id-nrofSymbolsExtended								ProtocolIE-ID ::= 702

id-repetitionFactorExtended							ProtocolIE-ID ::= 703

id-startRBHopping									ProtocolIE-ID ::= 704

id-startRBIndex										ProtocolIE-ID ::= 705

id-transmissionCombn8								ProtocolIE-ID ::= 706

id-Target-gNB-ID									ProtocolIE-ID ::= XX1

id-Target-gNB-IP-address							ProtocolIE-ID ::= XX2

id-Target-SeGW-IP-address							ProtocolIE-ID ::= XX3

id-Activated-Cells-Mapping-List						ProtocolIE-ID ::= XX4

id-Activated-Cells-Mapping-List-Item				ProtocolIE-ID ::= XX5

id-F1SetupOutcome									ProtocolIE-ID ::= XX6

END

-- ASN1STOP

END OF CHANGES