3GPP TSG-RAN WG3 #121						                             R3-23xxxx

21th – 25th Aug 2023 Toulouse, France

Title:	[draft] Reply LS on area scope for QoE measurements

Response to:	R2-2306569 LS on area scope for QoE measurements

Release:	Rel-18

Work Item:	NR_QoE_enh-Core

Source:	Samsung [to be RAN3]

To:	RAN2

Cc:	SA4, SA5

Contact person:

Name:	Xingyu HAN

E-mail Address:	xingyu.han@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# Overall description

RAN3 thanks RAN2 for the LS on area scope for QoE measurements.

Regarding the following Question asked by RAN2,

Q2) RAN2 would also like to ask SA4/SA5/RAN3 whether there is a problem if for UEs in RRC CONNECTED the network performs area scope checking (with Area Scope of QMC) and UE application also performs area scope checking (with LocationFilter) at the same time. It should be noted that area scope management for UEs in RRC CONNECTED in Rel-17 relies on the gNB releasing the QoE configuration when the UE moves out of the applicable area scope.

Answer to Q2:

RAN3 thinks that, at any given time, area scope checking is performed either by the RAN or by the UE, but not by both at the same time. In particular:

- When a UE is in the RRC_CONNECTED state, the area scope checking is done by the RAN, based on the Area Scope of QMC IE, in line with the current network behavior.

- When the UE is in RRC_IDLE or RRC_INACTIVE state, LocationFilter can be used, provided that it is extended to fully mimic the Area Scope of QMC IE. The extension would require introducing TAC, list of TAIs and PLMNs into the LocationFilter.

In addition, RAN3 has agreed to correct the presence of QoE Area Scope IE from mandatory to optional in NGAP, in order to align with SA5 spec, i.e. TS 28.405.

# Actions

To RAN2

ACTION: 	RAN3 kindly asks RAN2 to take above information into account.

# Dates of next RAN3 meetings

RAN3#121bis         09-13 Oct 2023             Xiamen, CN

RAN3#122		  13-17 Nov 2023			Chicago, USA