3GPP TSG-RAN WG3 Meeting #121	R3-234647

Toulouse, France, 21 – 25 Aug, 2023

Agenda Item:	10.2.3

Source:	Huawei

Title:	(TP for SON BLCR for TS 36.423): Remaining issues for RACH optimisation

Document for:	other

- Introduction

This document contains a TP for SON BLCR for TS36.423.

TP for SON BLCR for TS 36.423

Start of the change

### 8.3.16	Access and Mobility Indication

#### 8.3.16.1	General

The purpose of the Access and Mobility Indication procedure is to transfer Access and Mobility related information between E-UTRAN nodes.

#### 8.3.16.2	Successful Operation

Figure 8.3.16.2-1: Access and Mobility Indication. Successful operation – eNB-initiated

Figure 8.3.16.2-2: Access and Mobility Indication. Successful operation – eNB-initiated

The eNB initiates the procedure by sending the ACCESS AND MOBILITY INDICATION message sent to the en-gNB.

The eNB1 initiates the procedure by sending the ACCESS AND MOBILITY INDICATION message sent to the eNB2.

If the PSCell List Container IE is included in the ACCESS AND MOBILITY INDICATION message, the eNB2 may use it to identify the en-gNB to which the RA Report Container IE should be forwarded.

#### 8.3.16.3	Abnormal Conditions

Not applicable.

Start of the next change

9.1.2.50	ACCESS AND MOBILITY INDICATION

This message is sent by the eNB to the eNB/en-gNB to transfer access and mobility related information.

Direction: eNB  en-gNB, eNB1  eNB2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NR RA Report List |  | 0..1 |  |  | YES | ignore |
| >NR RA Report List Item |  | 1 .. <maxnoofRAReports> |  |  | EACH | ignore |
| >>NR RA Report Container | M |  | OCTET STRING | RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [10]. | – |  |
| >>UE Assistant Identifier | O |  | en-gNB UE X2AP ID 9.2.100 |  | – |  |
| >>PSCell List Container | O |  | OCTET STRING | [FFS – Name to be decided by RAN2] IE as defined in subclause [FFS – to be decided by RAN2] in TS 36.331 [9]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |

next change

9.1.2.x	RACH INDICATION

This message is sent by the en-gNB to inform the MeNB node that one or more RA reports are available at the UE.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| RA Report Indication List |  | 1 |  |  | YES | reject |
| >RA Report Indication List Item |  | 1 .. <maxnoofRAReportIndications> |  |  | - |  |
| >>RA Report Retrieval UE ID | M |  | eNB UE X2AP ID 9.2.24 | Indicate the UE ID need to retrieve RA Report. | - |  |
| >>MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB eNB | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReportIndications | Maximum number of RA Report indications in the message. Value is FFS. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

RaReportIndicationList	 ::= SEQUENCE (SIZE(1..maxnoofRAReportIndications)) OF RaReportIndicationList-Item

RaReportIndicationList-Item	::= SEQUENCE {

RA-Report-Retrieval-UE-ID				UE-X2AP-ID,

iE-Extensions				ProtocolExtensionContainer { {RaReportIndicationList-Item-ExtIEs} }	OPTIONAL,

...

}

RaReportIndicationList-Item-ExtIEs X2AP-PROTOCOL-EXTENSION::= {

...

}

End of the change