3GPP TSG-RAN WG3 Meeting #122	R3-237092

Chicago, USA, Nov 13th – 17th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | <CR#> |  | rev |  | - |  |  | Current version: |  |  | 17.6.0 |  |  |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  | UICC apps |  |  |  | ME |  |  | Radio Access Network | X |  | Core Network | X |  |

| Title: | (BL CR to 38.300) for XR Enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, ZTE, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) S (adding to the sourcing companies’ CR statistics) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add stage-2 changes to support NR XR enhancement |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes are: - Add Stage-2 changes based on RAN3#121bis NR XR enhancement related agreements. . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NR XR enhancement cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.x.1 (new), 16.x.2 (new), 16.x.3 (new), 16.x.4 (new), 16.x.5 (new), 16.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.413 CR 6002 TS 38.423 CR 6003 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 38.415 CR 6006 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS 38.425 CR 6007 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R3-237092 : rev2 – BL CR prepared based on R3-236001. R3-236001: rev1 - integrates agreed TP R3-235891 into BL CR R3-235125 |  |  |  |  |  |  |  |  |

Next Modified Subclause (new)

## 16.X	eXtended Reality Services

### 16.X.1	General

The following functions will be supported by NG-RAN for handling eXtended Reality (XR) services.

-	PDU Set QoS handling.

- 	Power saving enhancements.

-	Capacity enhancements

### 16.X.2	Awareness

XR-Awareness relies on QoS flows, PDU Sets, Data Bursts and traffic assistance information (see TS 23.501 [3]). Optional PDU Set QoS Parameters may be provided by the SMF to the gNB as part of the QoS profile of the QoS flow.

The SMF provides the following PDU Set QoS Parameters to NG-RAN as part of the NG-AP PDU Session Resource Management procedure The NG-RAN node shall, if supported, store the PDU Set QoS Parameters and take into account the received PDU Set QoS Parametersas specified in TS 23.501 [3]:

- PDU Set Delay Budget (PSDB): as defined in TS 23.501 [3], upper bound for the duration between the reception time of the first PDU (at the UPF for DL, at the UE for UL) and the time when all PDUs of a PDU Set have been successfully received (at the UE in DL, at the UPF in UL). A QoS Flow is associated with only one PSDB, and when available, it applies to both DL and UL and supersedes the PDB of the QoS flow.

- PDU Set Error Rate (PSER): as defined in TS 23.501 [3], upper bound for a rate of non-congestion related PDU Set losses between RAN and the UE. A QoS Flow is associated with only one PSER, and when available, it applies to both DL and UL and supersedes the PER of the QoS flow.

NOTE 1:	In this release, a PDU set is considered as successfully delivered only when all PDUs of a PDU Set are delivered successfully.

- PDU Set Integrated Handling Information (PSIHI): indicates whether all PDUs of the PDU Set are needed for the usage of PDU Set by application layer, as defined in TS 23.501 [3].

NOTE 2:	The PDU Set QoS parameters are common for all PDU Sets within a QoS flow.

Editor’s note: whether 5GC provides separate PDU Set QoS Parameters for DL and UL Traffic and pending reply LS from SA2.

During the Xn-handover procedure, the source NG-RAN node  sends the stored PDU Set QoS Parameters as part of the QoS profile to the target NG-RAN node.

In addition, the UPF can identify PDUs that belong to PDU Sets, and may determine the following PDU Set Information which it sends to the gNB in the GTP-U header:

-	PDU Set Sequence Number;

-	Indication of End PDU of the PDU Set;

-	PDU Sequence Number within a PDU Set;

-	PDU Set Size in bytes;

-	PDU Set Importance (PSI), which identifies the relative importance of a PDU Set compared to other PDU Sets within the same QoS Flow.

### 16.X.3	Power Saving

5GC may provide XR traffic assistance information to NG-RAN through NG-AP TSC Assistance Information (TSCAI) as specified in clause 5.37.8 of TS 23.501[3]. This assistance information will be used by NG-RAN node to configure connected mode DRX scheme to enable UE power saving for efficient handling of XR traffic.

-	UL and/or DL Periodicity

-	N6 Jitter Information associated with the DL Periodicity

In addition, 5GC may provide the following information through NG-U as specified in clause 5.37.5.2 of TS 23.501[3]. The NG-RAN node can indicate UE to go back to sleep mode if the transmission of the Data Burst ends if the UE is configured with C-DRX.

-	Indication of End of Data Burst in the GTP-U header of the last PDU in downlink.

### 16.X.4	Capacity

.

NG-RAN node may perform downlink PDU Set discarding based on implementation by taking at least PSDB, PSI, PSIHI parameters into account.

### 16.X. 5	Non-Homogeneous support of PDU set handling in NG-RAN

During an XR session, at mobility from a non-supporting NG-RAN node to a supporting NG-RAN node, the target NG-RAN node may provide the “PDU set handling support” indication (FFS on the name) to SMF and  SMF will act as specified in TS 23.501[3].

Editor’s note: The design of signalling PDU Set handling support indication is FFS

## 16.Y 	ECN marking for L4S and congestion information exposure

In order to support ECN marking for L4S at NG-RAN as specified in TS 23.501[3], SMF provides ECN marking request per QoS flow level to NG-RAN as part of PDU Session Resource Management procedure and if the NG-RAN supports ECN marking, it provides an indication back to SMF which is used by SMF as specified in TS 23.501[3].  During Xn Handover procedure, source NG-RAN node provides ECN marking request to target NG-RAN.

For ECN marking for L4S at UPF and congestion reporting from NG-RAN to UPF, SMF requests the NG-RAN to report assistance information per QoS flow level as part of PDU Session Resource Management procedure and if the NG-RAN supports assistance information reporting, it provides an indication back to SMF which is used by SMF as specified in TS 23.501[3]. During Xn Handover procedure, source NG-RAN node provides assistance information indication to target NG-RAN.

Editor’s Notes: Additional details will be updated based on further discussions.

End of Changes