3GPP TSG-RAN WG3#122	R3-237240

Chicago, USA, 13th – 17th Nov 2023

Agenda Item: 		12.2.2.2

Source:		NEC

Title:	AI/ML Configuration of Measured UE Trajectory information

Document for:		Discussion and decision

- Introduction

This contribution further discusses some remaining issues after last RAN3#121bis meeting.

12.2.2.2. ME and Xn procedures ….. RAN3#121: The measurement object to request the report of the UE Trajectory IE should be introduced in the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message. The AI/ML Measurement ID in HANDOVER REQUEST from source gNB is reused to trigger the target node feedback the UE trajectory information to the source gNB. AI/ML INFORMATION UPDATE from target node is used to convey the UE trajectory information. The Measured UE Trajectory reporting is one-time report. The conditions triggering the one-time reporting are at least: - Condition A: Switch of UE state (e.g. UE goes to RRC_IDLE/RRC_INACTIVE); - Condition B: UE leaves the target node; - Condition C: Expiration of the report time duration; - Condition D: The configured number of hand-overs within the same target node is reached. Report time duration and configured number of intra-NG-RAN node inter-cell handovers are included in DATA COLLECTION REQUEST message. The list of Cell IDs included in the Measured UE Trajectory is mandatory. RAN3#121bis: Encode the Measured UE Trajectory with new IE with Global NG-RAN Cell Identity and Time UE Stays in Cell as mandatory. Define a new Time Duration IE for the collection of measured UE trajectory in the DATA COLLECTION REQUEST message. FFS on the details of the Time Duration IE. Define a new Number of Visited Cell IE for the collection of measured UE trajectory in the DATA COLLECTION REQUEST message. FFS on the details of the Number of Visited Cell IE. The maximum number of cells for predicted UE trajectory is 16. The maximum number of cells for measured UE trajectory is 16. Work on stage3 details

- Discussion

Related with the UE trajectory reporting condition, current baseline CR to 38.423 has captured related information elements and IE i.e. the Report Time Duration IE and Number of Handovers IE is marked as FFS, which need to design its detail.

9.2.3.P Additional UE Trajectory Reporting Conditions (FFS on the IE name) This IE contains additional conditions triggering the target node to report UE Trajectory after successful handover. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Report Time Duration (FFS); O; INTEGER (1..4095) (FFS); The maximum time duration, in seconds, starting from successful handover execution, after which the target node will report the UE Trajectory.. Number of handovers (FFS); O; INTEGER (1..128) (FFS); Maximum number of intra-node handovers after which the target node will report the UE Trajectory.. Editor´s Note: Details for this IE are FFS

These FFS are all correlate with the FFS that are captured in the chair note i.e.

Define a new Time Duration IE for the collection of measured UE trajectory in the DATA COLLECTION REQUEST message. FFS on the details of the Time Duration IE. Define a new Number of Visited Cell IE for the collection of measured UE trajectory in the DATA COLLECTION REQUEST message. FFS on the details of the Number of Visited Cell IE.

The detail that needs to check are the name and their values.

Report Time Duration

There may be two alternatives to define:

- Alternative 1: define as INTEGER

ReportTimeDuration ::= INTEGER {1..4095} -- granularity in second

- Alternative 2: define as ENUMERATED

ReportingPeriodicity ::= ENUMERATED {S5,s10,s15,s20,s30, s40, s60,...} --s means second

Either way is Ok but only need to consider its granularity. While INTEGER {1..4095} way would be more flexible, on the other hand, need to consider whether the requested side can handle. However since this Report Time Duration IE is only for the reporting of the UE Trajectory which need to collect the UE movement occur within the time duration, it is not considered to be so much impact from the granularity, therefore the flexible way would be OK.

Proposal 1: for the Report Time Duration IE of the configuration of the UE Trajectory Reporting, it is defined as INTEGER {1..4095}.

Number of handovers

This may be rather straightforward as it is only to configure the target node to report the UE trajectory when the number of UE’s handover has reached the configured Number of Handovers, the value should not be too small, therefore the INTEGER {1..128} has not significant issue.

Proposal 2: for the Number of Handovers IE of the configuration of the UE Trajectory Reporting, it is defined as  INTEGER {1..128}.

The Additional UE Trajectory Reporting Conditions IE (FFS on the IE name)

This Additional UE Trajectory Reporting Conditions IE is rather to configure the target node for the reporting of measured UE Trajectory, simply just to name as its meaning, e.g. UE Trajectory Reporting Configuration IE

Proposal 3: the IE name is to be Additional UE Trajectory Reporting Conditions UE Trajectory Reporting Configuration IE

- Proposals

In this paper we discussed the stage 3 detail for UE Trajectory information.

Proposal 1: for the Report Time Duration IE of the configuration of the UE Trajectory Reporting, it is defined as INTEGER {1..4095}.

Proposal 2: for the Number of Handovers IE of the configuration of the UE Trajectory Reporting, it is defined as  INTEGER {1..128}.

Proposal 3: the IE name is to be Additional UE Trajectory Reporting Conditions UE Trajectory Reporting Configuration IE

Proposal 4: agree the TP.

- Reference

[1] RP-231159_New WI Artificial Intelligence (AI) Machine Learning (ML) for NG-RAN

[2] TR 37.817 Study on enhancement for Data Collection for NR and EN-DC

[3] R3-235739 SoD of CB:# AIRAN3_ME

| R3-235953 | (BLCR to 38.300) Draft CR to 38.300 on AI/ML for NG-RAN | Rel-18 | 38.300 | NR_AIML_NGRAN-Core |
| --- | --- | --- | --- | --- |
| R3-235954 | (BL CR to 38.420) Support of AI/ML for NG-RAN | Rel-18 | 38.420 | NR_AIML_NGRAN-Core |
| R3-235955 | (BLCR to 38.423) for AI/ML for NG-RAN | Rel-18 | 38.423 | NR_AIML_NGRAN-Core |
| R3-235050 | CR to TS 38.401 for addition of AI/ML-RAN feature in the case of split architecture | Rel-18 | 38.401 | NR_AIML_NGRAN-Core |