3GPP TSG-RAN WG3 Meeting #122	R2-237245

Chicago, USA, Nov. 13th – 17th, 2023

Agenda Item:	10.2.2

Source:	NEC

Title:	Discussion on MRO on fast MCG recovery

Document for:	Decision

1	Introduction

During the previous meeting, it has been agreed that

MRO for the fast MCG recovery: Case a: SCG fails or is deactivated when the UE attempts MCG recovery (i.e. a SCG failure/deactivation while T316 is running after MCG failure) Case b: the signalling delay is longer than the time the UE waits for the response (T316 expired); other problems are not precluded if legacy MRO mechanism cannot cope with it. RAN3 #119 Sub-Case b1/Sub-Case b2 would not be considered for MRO for fast MCG recovery failure. Case c/Case d/Case e/Case f would not be considered for MRO for fast MCG recovery failure? Additional information to be reported by UE? Agreements: Definitions of MRO events for CPAC will be introduced in TS 37.340 in a new chapter (based on [1226], CPA needs to be added). Agreements: During CPAC configuration, the value of the Time Stay IE for the source PSCell UHI, sent in S-NODE ADDITION REQUEST message, does not reflect the exact time the UE stayed in the source PSCell. In Rel.18, no extra signalling will be enabled to support CPAC UHI, stage2 text to be checked? A scenario when MCG fails before CPA/CPC is executed is a subject to regular MCG MRO correction to avoid possible resource waste. Agreements: Case f1, where the SCG fails or is deactivated yet before the UE sends the MCGFailureInformation is to be addressed. Case (c), i.e. the “near-failure scenario” will be addressed. WA: In order to enable support for pre-Rel.18 UEs (and help avoid wasting Uu resources), the T316 will be delivered from the MN to the SN. Agreements: Stage-2 description of the detection mechanism will be introduced in TS 38.300 (based on R3-231585). Agreements: Scenario ‘a’ is redefined: SCG fails when the UE is undergoing fast MCG recovery (i.e. SCG failure happens while T316 is running). The document containing list of scenarios (last noted at #118 as R3-226422) is recommended to be updated to cover the scenario ‘f1’. To be continued: Transfer of the T316 from the MN to the SN to support the MRO for the fast MCG recovery Rel.18 scenarios for pre-Rel.18 UEs (or Uu resource usage optimisation). Agreements:Agree to define case c in R18. Solution can be further discussed. How to support pre-R18 UE? Whether the network based solution or UE based solution should be used for Case c? Whether the MRO for fast MCG recovery needs to support pre-R18 UEs? MRO for CPAC: Whether CPA/CPC candidate PSCell list can be sent from MN to the source/last serving SN? UHI for CPAC? RAN3 assumes the indication is needed and draft the LS to RAN2 to check the feasibility of the solution for MRO for voice fallback. MRO for CPAC: For CPA or MN initiated CPC, agree Option 1 as way forward? For SN initiated CPC, agree Option 1 as way forward? Is the MN aware about intra-SN mobility occurring? MRO for inter-system handover for voice fallback: Stage2 and stage3 updates, e.g., name issue MRO for the fast MCG recovery: It is beneficial for the UE to report the time between MCG failure and SCG failure for fast MCG failure recovery. How to forward the information in RAN3? LS to RAN2 on additional information for fast MCG failure recovery?

It could be found that some remaining issues should be resolved related to the newly introduced case C. So in this contribution, we will try to resolve the remaining issues of fast MCG recovery.

2	Discussion

According to the latest progress, RAN2 has agreed to use RLF report for the near failure case and agreed to include elapsed T316 for this scenario. During the offline discussion in the last meeting, some additional information has been proposed, which are listed in the following:

- Indication that it is a near failure recovery case(e.g. will the presence of T316 be enough to indicate the recovery is successful?)

- PSCell used for fast MCG recovery

- Cell where the UE successfully connects after recovery

From our understanding, since we have already sent the LS towards RAN2 and check the necessity of the first alt, so there is no need to be hurry to make the decision of the first alt. Instead, we can wait for RAN2’s further instruction on the necessity. For the PSCell used for fast MCG recovery, it can be included in the RLF report since the spec impact would be very small. For the third, so far we cannot see any particular benefit to include it. Therefore, it is suggested to include PSCell used for fast MCG recovery into the RLF report used for near failure case. The corresponding LS can be sent to RAN2.

It is suggested to include PSCell used for fast MCG recovery into the RLF report used for near failure case and the corresponding LS can be sent to RAN2.

During last meeting, one of the remaining issues left, which is whether to transfer T316 from the MN to the SN to support the MRO for the fast MCG recovery Rel-18 scenarios for pre-Rel 18 UEs on Uu resource usage optimization.

MRO for the fast MCG recovery: Support of pre-R18 UEs?

From our view, at current stage, the motivation of such optimization is not clear due to it is not a must for Pre-Rel 18 UE to support the Rel-18 fast MCG recovery scenarios. Thus, such optimization may just involve unnecessary signalling overhead.

Therefore, it is not suggested to transfer T316 from the MN to the SN to support the MRO for the fast MCG recovery Rel-18 scenarios for pre-Rel-18 UE or Uu resource usage optimization.

Pre-18 UE should not be supported for fast MCG recovery.

3	Conclusion and Proposals

In this contribution, we have analysed other potential cases which may be included in the MRO for fast MCG recovery, as well as some necessary information should be reported for fast MCG recovery, and the following proposals has been given out:

Proposal1.	It is suggested to include PSCell used for fast MCG recovery into the RLF report used for near failure case and the corresponding LS can be sent to RAN2.

Proposal2.	Pre-18 UE should not be supported for fast MCG recovery.

4	References