3GPP TSG-RAN WG3 Meeting #122	R3-237336

Chicago, USA, 13th – 17th November 2023

Agenda Item:	10.2.2

Source:	ZTE

Title:	(TPs for SON BL CR 37.340 and 38.423) MRO for CPAC

Document for:	Discussions & Approval

1	Introduction

In RAN3#121bis meeting [1], the MRO for CPAC and fast MCG recovery have been discussed, and some progress have been reached as below.

MRO for CPAC: Agree to use option 1. No further stage3 impact foreseen at this stage.

Since there are still some open issues, in this contribution, we provide the further discussion on the MRO enhancements for CPAC in Rel-18.

2	Discussion

2.1   MRO for CPAC

In last meeting, the stage 2 description of the CPAC procedure for Option 1 has been introduced in [2], which is given as below.

In last meeting, one open issue is whether the verification of the intra-SN PSCell change is needed by MN at the beginning of the whole procedure. From our point of view, the verification of the intra-SN PSCell change i s needed. While, since the SCGFailureInformation shall be sent to MN from the UE, the MN is able to get the previous PSCell ID and the failure PSCell ID. With these info, the MN is able to decide whether the PSCell change is intra-SN or not. Therefore, there is no need for MN to trigger the SCG Failure Information Report procedure to perform the verification as legacy.

Proposal 1: There is no need for MN to trigger the SCG Failure Information Report procedure to perform the verification of intra-SN PSCell change as legacy.

Based on the agreed procedure given above, for the CPA or MN initiated CPC, the suitable PSCell ID can be obtained by MN from the UE directly. If MN decides to send the SCGFailureInformation to the target SN via the SCG FAILURE INFORMATION REPORT message, it implicitly indicates that the wrong target PSCell selection is performed by the target SN, and there is no need to include the suitable PSCell ID in the SCG FAILURE INFOMRATION REPORT message.

While, for the SN initiated CPC, since the suitable PSCell ID cannot be obtained by the source SN directly, this information should be sent by MN to the source SN via the SCG FAILURE INFORMATION REPORT message to help the source SN decide which node performs the wrong target PSCell selection.

Proposal 2: Add the suitable PSCell ID in the SCG FAILURE INFORMATION REPORT message for SN initiated CPC.

3	Conclusion

Proposal 1: There is no need for MN to trigger the SCG Failure Information Report procedure to perform the verification of intra-SN PSCell change as legacy.

Proposal 2: Add the suitable PSCell ID in the SCG FAILURE INFORMATION REPORT message for SN initiated CPC.

The corresponding TPs for SON BL CR 37.340 and 38.423 on MRO for CPAC are given in the Annex.

References

- R3-235728, Summary of Offline Discussion on MRO, Huawei

- R3-235809, (TP for SON BLCR for 37.340) SON enhancements for CPAC, Samsung

Annex

Text Proposal for SON BL CR 37.340 on MRO for CPAC

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 10.18	Self-optimisation for PSCell change

### 10.18.1	General

For analysis of PSCell change failure, the UE makes the SCG Failure Information available to the MN.

### 10.18.2	PSCell change failure

One of the functions of self-optimization for PSCell change is to detect PSCell change failures that occur due to Too late PSCell change or Too early PSCell change, or Triggering PSCell change to wrong PSCell. These problems are defined as follows:

-	Too late PSCell change: an SCG failure occurs after the UE has stayed for a long period of time in the PSCell; a suitable different PSCell is found based on the measurements reported from the UE.

-	Too early PSCell change: an SCG failure occurs shortly after a successful PSCell change from a source PSCell to a target PSCell or a PSCell change failure occurs during the PSCell change procedure; source PSCell is still the suitable PSCell based on the measurements reported from the UE.

-	Triggering PSCell change to wrong PSCell: an SCG failure occurs shortly after a successful PSCell change from a source PSCell to a target PSCell or a PSCell change failure occurs during the PSCell change procedure; a suitable PSCell different with source PSCell or target PSCell is found based on the measurements reported from the UE.

In the definition above, the "successful PSCell change" refers to the UE state, namely the successful completion of the RA procedure.

MN performs initial analysis to identify the node that caused the failure. The MN may use the SCG Failure Information Report procedure to verify whether intra-SN PSCell change has been triggered in the last serving SN and stores the SCG Failure Information for the time needed to receive possible response from the last serving SN. If the failure is caused by a source SN, the MN forwards then the SCG Failure Information to the source SN. The node responsible for the last PSCell change (the source SN, the last serving SN or the MN) performs the final root cause analysis.

10.18.A	Conditional PSCell addition or change failure

One of the functions of self-optimization for CPAC is to detect CPAC failures that occur due to Too late CPC execution or Too early CPC/CPA execution, or CPC/CPA execution to wrong PSCell. These problems are defined as follows:

-	Too Late CPC Execution: UE receives CPC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE.

-	Too Early CPC/CPA Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE.

-	CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. There are two sub-cases:

-	if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC or by the MN initiating the CPA to a (candidate) target SN, but not one of the candidate PSCells selected by the (candidate) target SN, it is wrong target PSCell selection at the (candidate) target SN;

-	else, it is wrong candidate PSCell list selection at the node initiating the CPC or at the MN initiating the CPA.

In the definition above, the "successful CPC/CPA execution" refers to the UE state, namely the successful completion of the RA procedure.

The MN verifies whether intra-SN PSCell change has been triggered in the last serving SN and performs the initial analysis when SCGFailureInformation is received from the UE e.g. whether it is CPA or CPC, if CPC whether it is MN initiated or SN initiated.

For CPA or MN initiated CPC, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the target SN, MN sends the SCGFailureInformation to the target SN to make the target SN perform the final root cause analysis. Otherwise, MN performs the final root casue analysis.

For SN initiated CPC, the MN sends the SCGFailureInformation to source SN, and source SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the target SN, the source SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCGFailureInformation to the target SN to make the target SN perform the final root cause analysis. Otherwise, the source SN performs the final root cause analysis.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

Text Proposal for SON BL CR 38.423 on MRO for CPAC

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

9.1.2.29	SCG FAILURE INFORMATION REPORT

This message is sent by M-NG-RAN node to S-NG-RAN node to report a PSCell change failure event.

Direction: M-NG-RAN node  S-NG-RAN node .

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node. | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node. | YES | ignore |
| Source PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of source PSCell for PSCell change procedure | YES | ignore |
| Failed PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of PSCell where SCG failure occurs for PSCell change procedure | YES | ignore |
| SCG Failure Report Container | M |  | OCTET STRING | The SCGFailureInformation message or the SCGFailureInformationEUTRA message as defined in TS 38.331 [10] or the SCGFailureInformation message or the SCGFailureInformationNR message as defined in TS 36.331 [14] | YES | ignore |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the PSCell change. It’s provided by S-NG-RAN node in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |
| CPAC Configuration | O |  | 9.2.2.xy |  | YES | ignore |
| Suitable PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of suitable PSCell for conditional PSCell change procedure | YES | ignore |

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>