3GPP TSG-RAN WG3 Meeting #122	R3-237401

Chicago, US, 13th – 17th Nov 2023

Agenda item:	 10.2.1

Source:		Lenovo

Title:		Discussion on SON enhancements for inter-RAT SHR and SPR

Document for:		Discussion and Decision

1	Introduction

RAN3#121 meeting achieved following agreements:

In case of MN initiated PSCell change, MN will have the final say on the T310/T312 SPR thresholds.

Support correlation of SHR and LTE RLF Report in case both are generated during an inter-RAT HO from NR to LTE

In order to support correlation, C-RNTI (either source C-RNTI or Target C-RNTI) and time between HO execution and SHR retrieval can be used if reported by the UE. It is up to RAN2 to decide whether to use source C-RNTI or Target C-RNTI.

If RAN2 agrees to use Target C-RNTI in order to support correlation, Target C-RNTI should be included in the Xn HANDOVER REPORT.

RAN3#121bis meeting further agreed:

For SN initiated PSCell change or CPC, source SN should provide the T310/T312 SPR triggers to MN.

There is no need to report any additional information from UE in Rel-18 in order to support UE context retrieval for SHR and SPR.

The objective of T304 SPR trigger is to optimize RACH access issues in target SN.

In this paper, we further discuss left issues for inter-RAT SHR and SPR.

2	Discussion

  - 2.1 inter-RAT SHR from NR to LTE

For the issue about correlating SHR and RLF reports for the case where the failure happens shortly after a successful handover, RAN3#121 meeting agreed that it is beneficial that the UE reports a C-RNTI (from either source or target cell, leaving it up to RAN2 to decide whether to use source C-RNTI or target C-RNTI) and the time between the handover command and the reporting of this event in SHR. RAN2#123bis meeting discussed this issue and concluded that the C-RNTI from the target cell is included in inter-RAT SHR to enable the correlation of the SHR and RLF report [1].

Observation: RAN2 agreed that the C-RNTI from the target cell is included in inter-RAT SHR to enable the correlation of the SHR and RLF report.

Based on RAN2’s reply LS, we can confirm previous RAN3’s agreement that “If RAN2 agrees to use Target C-RNTI in order to support correlation, Target C-RNTI should be included in the Xn HANDOVER REPORT”.

Proposal 1: Confirm previous RAN3’s agreement that Target C-RNTI should be included in the Xn HANDOVER REPORT message to support correlation.

  - 2.2 SPR

RAN3#121 meeting agreed that in case of MN initiated PSCell change, MN will have the final say on the T310/T312 SPR thresholds, whether SN can propose its preferred T310/T312 SPR thresholds to MN and whether the T310/T312 timer values can be provided as assistance information from SN to MN (e.g., to assist in the root cause analysis) are FFS.

Due to MN has the final say on the T310/T312 SPR thresholds, if source SN provides its preferred T310/T312 SPR thresholds to MN, MN need to determine whether the source SN provied preference is good, furthermore, MN need to provide feedback to source SN, siganlling overhead and implementation complexity are caused, on the other hand, there is no strong benefit for source SN to provide its preferred T310/T312 SPR thresholds to MN because T310/T312 SPR trigger threshold is just a percentage value rather than an absolute value. Therefore, there is no need for source SN to provide its preferred T310/T312 SPR trigger threshold to MN.

Proposal 2: For MN initiated PSCell change or CPC, no need for source SN to provide its preferred T310/T312 SPR trigger threshold to MN.

If source SN needs to provide T310/T312 timer values to MN, whether source SN does it automously or based on MN’s request needs further discussion, additionally if T310/T312 timer value changes, whether/how to update T310/T312 SPR trigger threshold at MN side is another question.

Since T310/T312 SPR trigger threshold is a percentage value (e.g. 40%, 60% or 80%), there is no issue for MN to decide the percentage value by itself without knowing the exact timer value of T310/T312, which means that it is not mandatory of MN to achieve the configured timer value of T310/T312 from source SN, thus signalling coordination between MN and source SN can be avoided.

In general, the easy way is that MN configures T310/T312 SPR trigger threshold without source SN’s assistance information.

Proposal 3: For MN initiated PSCell change or CPC, no need for source SN to provide T310/T312 timer values to MN.

Another left FFS is whether objective of T304 SPR trigger is also to optimize the mobility configuration in the initiating node. We can find that T304 is also a trigger condition for SHR, if SHR is triggered due to T304 trigger condition is fulfilled, the SHR would be forwarded to target node directly due to the the objective of T304 SHR trigger is to optimize RACH access issues in target node, this principle can also be applied to SPR, i.e. the objective of T304 SPR trigger is mainly to optimize RACH access issues in target SN. If objective of T304 SPR trigger is also to optimize the mobility configuration in the initiating node, conflicting optimization is caused, e.g. unnecessary optimization in the initiating node may result in too late PSCell change/addition.

Moreover, we should follow previous agreements that when SPR is triggered due to T304 trigger condition, the SPR would always be firstly forwarded to the old MN which configures the SPR configuration to the UE, then the old MN would only forward the SPR to target SN directly for RACH optimization, but the old MN would not forward the SPR to the initiating node for MRO purpose.

Proposal 4: The objective of T304 SPR trigger is only to optimize RACH access issues in target SN.

3	Conclusion

This paper mainly discussed SON enhancements for inter-RAT SHR and SPR, the proposals were achieved as following:

Observation: RAN2 agreed that the C-RNTI from the target cell is included in inter-RAT SHR to enable the correlation of the SHR and RLF report.

Proposal 1: Confirm previous RAN3’s agreement that Target C-RNTI should be included in the Xn HANDOVER REPORT message to support correlation.

Proposal 2: For MN initiated PSCell change or CPC, no need for source SN to provide its preferred T310/T312 SPR trigger threshold to MN.

Proposal 3: For MN initiated PSCell change or CPC, no need for source SN to provide T310/T312 timer values to MN.

Proposal 4: The objective of T304 SPR trigger is only to optimize RACH access issues in target SN.

Reference

[1] R2-2311528, Reply LS on SHR and SPR, Nokia