3GPP TSG-RAN WG3 #122	R3-237410

Chicago, USA, 13th – 17th November 2023

Agenda Item:	12.2.2.2

Source:	Lenovo

Title: 		Left issues related to mobility enhancement use case

Document for:	Discussion and Approval

# Introduction

In this paper, we discuss the left issues related to mobility enhancement use case.

# 2	Discussion

Any UE Performance Feedback reporting should occur no later than the expiration of the Reporting Duration. The Measured UE Trajectory reporting is one-time report. The conditions triggering the one-time reporting are at least: - Condition A: Switch of UE state (e.g. UE goes to RRC_IDLE/RRC_INACTIVE); - Condition B: UE leaves the target node; - Condition C: Expiration of the report time duration; - Condition D: The configured number of hand-overs within the same target node is reached. Report time duration and configured number of intra-NG-RAN node inter-cell handovers are included in DATA COLLECTION REQUEST message. Define a new Time Duration IE for the collection of measured UE trajectory in the DATA COLLECTION REQUEST message. FFS on the details of the Time Duration IE. Define a new Number of Visited Cell IE for the collection of measured UE trajectory in the DATA COLLECTION REQUEST message. FFS on the details of the Number of Visited Cell IE.

For UE performance feedback, RAN3 agreed that the UE performance feedback reporting should occur no later than the expiration of the reporting duration. Similar as UE performance feedback reporting, any UE trajectory reporting after handover should occur no later than the expiration of reporting duration. Besides, if the number of handovers is configured, any UE trajectory reporting after handover should occur no later than the configured number of handovers is exceeded.

If Time Duration IE is configured for the measured UE trajectory report, it starts at successful handover execution, and any UE trajectory reporting after handover should occur no later than the expiration of reporting duration.

If the Number of Visited Cell IE is configured for the measured UE trajectory report, any UE trajectory reporting after handover should occur no later than the configured number of handovers is exceeded.

On the other hand, in our understanding there are two cases, measured UE trajectory could be reported before report duration expiry:

- Case#1: Due to interruption and UE trajectory can be no longer measured by the target gNB

- Case#2: Due to signalling efficiency reason, gNB can report the measured trajectory of many UEs in the same DATA COLLECTION UPDATE message even if the configured report duration for some UEs is not reached.

For Case#1, the interruption could be any of the following:

- Radio link failure happens

- The UE enters RRC inactive/idle state

- The UE is handed over to another gNB

In our understanding, it is beneficial to indicate the source gNB the cause of interruption of measured UE trajectory considering they are related to the UE trajectory prediction made by source gNB as well considering:

- If it is because of UE being handed over to another gNB, itself is useful UE trajectory information and can be used as training input data or feedback data.

- If it is because of RLF in short time after handover, it implies suboptimal handover decision.

- If it is because of RRC state change, then the interruption should not be interpreted as relevant to a UE trajectory prediction or suboptimal handover decision as above.

There are three existing cause values could be used with extended meaning:

- User Inactivity,

- Radio Connection With UE Lost,

- Failure in the Radio Interface Procedure

Besides, after receiving the indication from target gNB that the requested UE performance can no longer be provided, source gNB can delete the stored UE context immediately without waiting until the report duration expires.

Source gNB can delete the stored UE context immediately after knowing the UE performance feedback will be no longer provided.

If target gNB can no longer provide measured UE trajectory for a particular UE (e.g., due to RLF, RRC state, HO), the target gNB indicates to the source gNB via a cause value associated with the UE in the DATA COLLECTION UPDATE message.

RAN3 further discusses if legacy cause values can be reused with extended meaning or new cause values are needed.

In addition, if target gNB strictly follow the time configuration (Time Duration IE), and start counting report duration immediately after the handover, it may lead to a sub-optimal situation. Since handovers of different UEs cannot occur at exactly the same time, eventually the target gNB may trigger many DATA COLLECTION UPDATE messages each carries the measured UE trajectory for one particular UE. At the end, one DATA COLLECTION UPDATE message will carry measured trajectory for only one UE even if it is designed to carry multiple UEs’ at the same time.

To avoid above mentioned sub-optimal situation, and use DATA COLLECTION UPDATE message to carry measured trajectory for many UEs at the same time, RAN3 is suggested to support gNB to report the measured trajectory of one UE before the report duration expiry proactively.

gNB can report the measured UE trajectory of many UEs in the same DATA COLLECTION UPDATE message even if the configured report duration for some UEs is not reached.

RAN3 agrees the relevant TP in R3-237409.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	Source gNB can delete the stored UE context immediately after knowing the UE performance feedback will be no longer provided.

Based on the discussion above, we propose:

Proposal 1	If Time Duration IE is configured for the measured UE trajectory report, it starts at successful handover execution, and any UE trajectory reporting after handover should occur no later than the expiration of reporting duration.

Proposal 2	If the Number of Visited Cell IE is configured for the measured UE trajectory report, any UE trajectory reporting after handover should occur no later than the configured number of handovers is exceeded.

Proposal 3	If target gNB can no longer provide measured UE trajectory for a particular UE (e.g., due to RLF, RRC state, HO), the target gNB indicates to the source gNB via a cause value associated with the UE in the DATA COLLECTION UPDATE message.

Proposal 4	RAN3 further discusses if legacy cause values can be reused with extended meaning or new cause values are needed.

Proposal 5	gNB can report the measured UE trajectory of many UEs in the same DATA COLLECTION UPDATE message even if the configured report duration for some UEs is not reached.

Proposal 6	RAN3 agrees the relevant TP in R3-237409.