3GPP TSG-RAN WG3 Meeting #122	R3-237505

Chicago, IL, U.S.A., November 13th – 17th 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1038 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of QoE Measurement Status IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Lenovo, CATT, Xiaomi, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of the “not-started” codepoint to the existing QoE Measurement Status IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.224. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------Start of changes-------------------------------------------

#### 9.3.1.224	UE Application Layer Measurement Configuration Information

This IE defines configuration information for the QMC functionality.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [45]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are coming with the QMC activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3-bytes Octet String. | - |  |
| Service Type | M |  | ENUMERATED (QMC for DASH streaming, QMC for MTSI, QMC for VR, ...) | This IE indicates the service type of QoE measurements. | - |  |
| CHOICE Area Scope of QMC | O |  |  |  | - |  |
| >Cell based |  |  |  |  | - |  |
| >>Cell ID List for QMC |  | 1..<maxnoofCellIDforQMC> |  |  | - |  |
| >>>NG-RAN CGI | M |  | 9.3.1.73 | This IE can only indicate the NR CGI. | - |  |
| >TA based |  |  |  |  | - |  |
| >>TA List for QMC |  | 1..<maxnoofTAforQMC> |  |  | - |  |
| >>>TAC | M |  | 9.3.3.10 | The TAI is derived using the current serving PLMN. | - |  |
| >TAI based |  |  |  |  | - |  |
| >>TAI List for QMC |  | 1..<maxnoofTAforQMC> |  |  | - |  |
| >>>TAI | M |  | 9.3.3.11 |  | - |  |
| >PLMN area based |  |  |  |  | - |  |
| >>PLMN List for QMC |  | 1..<maxnoofPLMNforQMC> |  |  | - |  |
| >>>PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| Measurement Collection Entity IP Address | M |  | Transport Layer Address 9.3.2.4 | The IP address of the entity receiving the QoE measurement report. | - |  |
| QoE Measurement Status | O |  | ENUMERATED (ongoing, not-started,…) | Indicates whether the QoE measurement has been started or not. Present in case of NG-based handover. | - |  |
| Container for Application Layer Measurement Configuration | O |  | OCTET STRING (SIZE(1.. 8000)) | Contains application layer measurement configuration, see Annex L in 26.247 [46], clause 16.5 in TS 26.114 [51] and clause 9 in TS 26.118 [52]. Present in case of initial QoE configuration, and shall be included in Source to Target Transparent Container IE for signalling-based QMC during NG-based handover. | - |  |
| Measurement Configuration Application Layer ID | O |  | INTEGER (0..15, …) | This IE is present only when the message containing it is NG-based handover related. The IE indicates the identity of the application layer measurement configuration, as defined in TS 38.331 [18]. | - |  |
| Slice Support List for QMC |  | 0..1 |  |  | - |  |
| >Slice Support QMC Item |  | 1..<maxnoofSNSSAIforQMC> |  |  | - |  |
| >>S-NSSAI | M |  | 9.3.1.24 |  | - |  |
| CHOICE MDT Alignment Information | O |  |  | Indicates the MDT measurements with which alignment is required. | - |  |
| >S-based MDT |  |  |  |  | - |  |
| >>NG-RAN Trace ID | M |  | OCTET STRING (SIZE(8)) | This IE is composed of the following: Trace Reference defined in TS 32.422 [11] (leftmost 6 octets, with PLMN information encoded as in 9.3.3.5), and Trace Recording Session Reference defined in TS 32.422 [11] (last 2 octets). | - |  |
| Available RAN Visible QoE Metrics | O |  | 9.3.1.225 | Present in case of initial QoE configuration and in case of NG-based handover for signalling-based QoE measurement. | - |  |
| Assistance Information of QoE Measurement (FFS) | O |  | INTEGER (1..16, …) | This IE indicates the suggested priority of the application layer measurement configuration. Values are ordered in decreasing order of priority, i.e. with 1 as the highest priority and 16 as the lowest priority. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforQMC | Maximum no. of Cell ID subject for QMC scope. Value is 32. |
| maxnoofTAforQMC | Maximum no. of TA subject for QMC scope. Value is 8. |
| maxnoofPLMNforQMC | Maximum no. of PLMNs in the PLMN list for QMC scope. Value is 16. |
| maxnoofSNSSAIforQMC | Maximum no. of S-NSSAIs in the S-NSSAI list for QMC scope. Value is 16. |

-------------------------------------------Next Change-------------------------------------------

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

UEAppLayerMeasConfigInfo ::= SEQUENCE {

qoEReference								QoEReference,

serviceType								ServiceType,

areaScopeOfQMC							AreaScopeOfQMC									OPTIONAL,

measCollEntityIPAddress					TransportLayerAddress,

qoEMeasurementStatus						ENUMERATED {ongoing, ..., not-started}					OPTIONAL,

containerForAppLayerMeasConfig			OCTET STRING (SIZE(1..8000))				OPTIONAL,

measConfigAppLayerID						INTEGER (0..15, ...) 					OPTIONAL,

sliceSupportListQMC						SliceSupportListQMC 						OPTIONAL,

mDT-AlignmentInfo						MDT-AlignmentInfo						OPTIONAL,

availableRANVisibleQoEMetrics			AvailableRANVisibleQoEMetrics			OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { UEAppLayerMeasConfigInfo-ExtIEs} }		OPTIONAL,

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-------------------------------------------End of changes-------------------------------------------