3GPP TSG-RAN3 Meeting #122	R3-237539

# Chicago, USA, 13th November – 17th November 2023

Agenda Item:	25.2.1

Source:	Ericsson

Title:	(TP to F1-AP BL CR): introduction of UL UE XR Traffic assistance Information

Document for:	Other

# Introduction

In last RAN3 meeting, companies have discussed about the encoding of the jitter information for DL, and it was mentioned that RAN2 agreed that UE also reports Burst Arrival time and UL Jitter information associated with the UL periodicity using the UE Assistance Information (UAI) [1].

The following FFS points were therefore captured:

Jitter information:

- The encoding of the N6 Jitter Information needs further checking from RAN2 progress (should we align UL and DL periodicities?)

- FFS how gNB-CU-CP handles the received XR UE assistance information (e.g., UL jitter information, BAT). Whether signalling to gNB-DU is needed.

In this paper we discuss the impacts related to receiving UL XR UE assistance information.

# Discussion

## UL XR Information from UE

Below are the agreements made by RAN2 in RAN2#123 meeting on UE traffic information for XR:

Jitter, periodicity and arrival time - 1: UE reports Burst Arrival time and Jitter associated with the UL data burst periodicity in uplink using UAI. - UE reports UL data burst periodicity in uplink using UAI. - All UAI fields for XR are optional fields in RRC. FFS how to handle persistency of signalled information (e.g. UE reports BAT first, then jitter). - Consider exact jitter range later on (e.g. via email discussion) - UE can also report there is no jitter (e.g. for pose). - Reuse UAI framework, e.g. network can configure when UE is allowed to report UAI. Exact triggering upon being configured and change of UAI is up to UE implementation. Network can configure prohibit timer for the reporting.

We can understand based on the RAN2 agreements that gNB-CU receives new information in RRC about the XR traffic characteristics, via the UE Assistance Information (UAI). The information are:

- Burst Arrival Time (BAT),

- UL Jitter Information

- UL periodicity

The purpose of providing the UL XR-related UE information is to enable fast adaptation of network resources and capacity to the XR changing traffic pattern. Since the scheduling functions for C-DRX configuration are located in the gNB-DU, it is motivated that the gNB-CU signals this received information from UE over F1 to gNB-DU for scheduling purpose. In RAN3, we can add the new information listed above in the UE CONTEXT SETUP REQUEST message and UE CONTEXT MODIFICATION REQUEST messages in TS 38.473. The granularity of the information is detailed further below.

Observation 1: It is beneficial for gNB-DU to be aware of UE UL XR information for the purpose of C-DRX configuration and XR Power saving.

Proposal 1: Introduce the BAT, UL Jitter Information and UL Periodicity received from the UE in the UE CONTEXT SETUP REQUEST message and UE CONTEXT MODIFICATION REQUEST messages in TS 38.473

Looking at RAN2’ s running CR [2], the encoding of the IEs are as below:

UEAssistanceInformation-v1800-IEs ::= SEQUENCE { ul-TrafficInfo-r18 UL-TrafficInfo-r18 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } [...] UL-TrafficInfo-r18 ::= SEQUENCE (SIZE (1..maxNrofPDU-Sessions-r17)) OF PDU-SessionUL-TrafficInfo-r18 PDU-SessionUL-TrafficInfo-r18 ::= SEQUENCE { pdu-SessionID-r18 PDU-SessionID, qos-FlowUL-TrafficInfoList-r18 SEQUENCE (SIZE (1..maxNrofQFIs)) OF QOS-FlowUL-TrafficInfo-r18 } QOS-FlowUL-TrafficInfo-r18 ::= SEQUENCE { qfi-r18 INTEGER (0..maxQFI), jitterRange-r18 FFS_TYPE OPTIONAL, burstArrivalTime-r18 CHOICE { referenceTime ReferenceTime-r16, referenceSFN-AndSlot ReferenceSFN-AndSlot-r18 } OPTIONAL, trafficPeriodicity-r18 FFS_TYPE OPTIONAL, pduSetIdentification-r18 BOOLEAN OPTIONAL, ... } ReferenceSFN-AndSlot-r18 ::= SEQUENCE { referenceSFN-r18 INTEGER (0..1023), referenceSlot-r18 INTEGER (0..639) } -- Editor’s note: RAN2 needs to decide how the range of jitter and traffic periodicity are supposed to be expressed in ASN.1. -- TAG-UEASSISTANCEINFORMATION-STOP -- ASN1STOP UL-TrafficInfo field descriptions. burstArrivalTime Indicates the average value of the arrival time of the first packet of the Data Burst for the concerned QoS flow. When indicated together with jitterRange, burstArrivalTime is used as a reference time for the indicated jitter range. If burstArrivalTime is indicated as referenceTime, the indicated time in 10ns unit from the origin is refDays*86400*1000*100000 + refSeconds*1000*100000 + refMilliSeconds*100000 + refTenNanoSeconds. The refDays field specifies the sequential number of days (with day count starting at 0) from 00:00:00 on Gregorian calendar date 6 January, 1980 (start of GPS time). If burstArrivalTime is indicated as referenceSFN-AndSlot, it refers to the UL timing of the closest SFN and slot with the indicated number.. jitterRange Indicates the maximum deviation of the arrival time of the first packet of a Data Burst compared to the time indicated with burstArrivalTime and the periodicity of the Data Bursts. This field shall only be reported together with the burstArrivalTime or after the burstArrivalTime has been already reported.. pduSetIdentification Indicates whether the UE is able to identify PDU Set related information for the QoS flow. If set to true, the UE is able to identify PDU set related information, otherwise, the UE is not able to do so.. qfi Identity of the QoS flow to which this UL traffic information refers.. trafficPeriodicity Indicates the average time period between the start times of two data bursts..

The values of the UL jitter and UL periodicity are FFS in RAN2 CR, whereas the value of the UL BAT is defined and different from the Burst Arrival Time IE defined in F1AP with a CHOICE structure.

#### 9.3.1.144	Burst Arrival Time

This IE indicates the Burst Arrival Time as defined in TS 23.501 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Burst Arrival Time | M |  | OCTET STRING | Encoded in the same format as the ReferenceTime IE as defined in TS 38.331 [8]. The value is provided with 1 us accuracy. |

Considering the structure of the UL Traffic info in UAI, the XR UE traffic assistance information being defined per QFI, it is proposed to define a new IE on the same level as QFI in the F1AP message, as below:

#### 9.3.1.A	UE XR Traffic Assistance Information

This IE provides the UE’s XR traffic assistance information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE UL Periodicity | O |  | FFS |  |
| UE UL Jitter Information | O |  | FFS |  |
| CHOICE UE Burst Arrival Time | M |  |  |  |
| >reference Time |  |  |  |  |
| >>reference Time | M |  | Burst Arrival Time 9.3.1.144 |  |
| >reference SFN and Slot |  |  |  |  |
| >>reference SFN | M |  | INTEGER (0..1023) |  |
| >>reference Slot | M |  | INTEGER (0..639) |  |

Proposal 2 : RAN3 to add new UE XR Traffic Assistance Information with FFS on a per QoS Flow Identifier IE level in the F1 dedicated messages

RAN3 can also discuss whether other nodes in the network (e.g., gNB-CU-UP, CN) need to be aware of the UE UL XR information.

Proposal 3: RAN3 to discuss if the UE XR Traffic Assistance Information IE need to be signalled to gNB-CU-UP and/or CN

## DL jitter information

Since UL jitter information is FFS in RAN2 RRC running CR, it is better to decouple the N6 Jitter information received from the CN, from the UE UL Jitter, since the sources are different and the encoding may be not the same. It is proposed to introduce separate Jitter information in RAN3 specs.

Proposal 4: RAN3 separates N6 Jitter Information from UE’ UL Jitter information.

# Conclusion

Observation 1: It is beneficial for gNB-DU to be aware of UE UL XR information for the purpose of C-DRX configuration and XR Power saving.

Proposal 1: Introduce the BAT, UL Jitter Information and UL Periodicity received from the UE in the UE CONTEXT SETUP REQUEST message and UE CONTEXT MODIFICATION REQUEST messages in TS 38.473

Proposal 2 : RAN3 to add new UE XR Traffic Assistance Information with FFS on a per QoS Flow Identifier IE level in the F1 dedicated messages

Proposal 3: RAN3 to discuss if the UE XR Traffic Assistance Information IE need to be signalled to gNB-CU-UP and/or CN

Proposal 4: RAN3 separates N6 Jitter Information from UE’ UL Jitter information.

# References

- R3-235722 CB: # XR_PDUSethandling, section 4.4

- XR RRC running CR_v11_rapp.docx

# TP to F1AP BL CR

START OF CHANGES

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| CU to DU RRC Information | M |  | 9.3.1.25 |  | YES | reject |
| Candidate SpCell List |  | 0..1 |  |  | YES | ignore |
| >Candidate SpCell Item IEs |  | 1 .. <maxnoofCandidateSpCells> |  |  | EACH | ignore |
| >>Candidate SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16] | - |  |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1 .. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | If included, it should be set to true. This IE is ignored if the Additional Duplication Indication IE is present. | - |  |
| >>Additional Duplication Indication | O |  | ENUMERATED (three, four, …) |  | YES | ignore |
| >>SDT RLC Bearer Configuration | O |  | OCTET STRING | Includes the RLC-BearerConfig IE defined in subclause 6.3.2 of TS 38.331 [8] | YES | ignore |
| >>SRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID for the SRB | YES | ignore |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>>>>>UE XR Traffic Assistance Information | O |  | 9.3.1.A | UE XR Traffic pattern information associated with the QFI. | YES | ignore |
| >>>>Monitoring Request (FFS) | O |  | 9.3.1.a |  | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| ** Unchanged text skipped ** |  |  |  |  |  |  |

Next change

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| ServCellIndex | O |  | INTEGER (0..31, ...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| Transmission Action Indicator | O |  | 9.3.1.11 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| RRC Reconfiguration Complete Indicator | O |  | 9.3.1.30 |  | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message message as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | reject |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| SCell To Be Removed List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Removed Item IEs |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1..<maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | This IE is ignored if the Additional Duplication Indication IE is present. | - |  |
| >>Additional Duplication Indication | O |  | ENUMERATED (three, four, …) |  | YES | ignore |
| >>SRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID for the SRB | YES | ignore |
| >>SDT Indicator Setup | O |  | ENUMERATED (true, …) | Indicates SDT SRB. | YES | reject |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>>>>>UE XR Traffic Assistance Information | O |  | 9.3.1.A | UE XR Traffic pattern information associated with the QFI. | YES | ignore |
| >>>>Monitoring Request (FFS) | O |  | 9.3.1.a |  | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>>>DRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID of the DL tunnel corresponding to such UL tunnel | YES | ignore |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>UL Configuration | O |  | UL Configuration 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | - |  |
| >>DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>DL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. < maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| >>SDT Indicator Setup | O |  | ENUMERATED (true, …) | Indicates SDT DRB. | YES | reject |
| DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | O |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>>>>>UE XR Traffic Assistance Information | O |  | 9.3.1.A | UE XR Traffic pattern information associated with the QFI. | YES | ignore |
| >>>>Monitoring Request (FFS) | O |  | 9.3.1.a |  | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| ** Unchanged text skipped ** |  |  |  |  |  |  |

NEXT CHANGE

#### 9.3.1.A	UE XR Traffic Assistance Information

This IE provides the UE’s XR traffic assistance information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE UL Periodicity | O |  | FFS |  |
| UE UL Jitter Information | O |  | FFS |  |
| CHOICE UE Burst Arrival Time | M |  |  |  |
| >reference Time |  |  |  |  |
| >>reference Time | M |  | Burst Arrival Time 9.3.1.144 |  |
| >reference SFN and Slot |  |  |  |  |
| >>reference SFN | M |  | INTEGER (0..1023) |  |
| >>reference Slot | M |  | INTEGER (0..639) |  |

NEXT CHANGE

ASN.1 to be provided after convergence on tabular