3GPP TSG-RAN WG3 Meeting #122	R3-237659

Chicago, USA,13th – 17th November 2023

Agenda Item:	11.4

Source:	ZTE, China Telecom, China Unicom

Title:	(TPs for BL CR of TS38.401&38.473) Deactivation of RVQoE reporting over F1AP

Document for:	Discussions & Approval

1	Introduction

This paper provides the text proposas to BLCR of 38.401 and 38.473 on deactivation of RVQoE reporting over F1..

2	Text Proposal for 38.401

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

7.10	Support of RAN visible QoE measurement

The RAN visible QoE measurement function is specified in TS 38.300 [2].

In split gNB architecture, upon the reception of the RAN visible QoE measurement report from the UE, the gNB-CU may forward the corresponding QoE information to the gNB-DU. The QoE information transferred to the gNB-DU may include the RAN visible QoE measurement results received from the UE, along with the corresponding DRB ID(s).

The gNB-DU may deactivate the transmission of RAN visible QoE measurements from the gNB-CU, by initiating QoE Information Transfer Control procedure towards the gNB-CU. The QoE Information Transfer Control message may include an optional Deactivation Indication List IE, which contains a list of F1AP UE identifiers, in order to allow the deactivation of RVQoE reporting in UE level. Upon receiving the deactivation message, if there is a list of UE identifier, the gNB-CU stops forwarding the RAN visible QoE reports to the specific UEs as indicated by the UE id list. If there is no UE id list received in the deactivation message, the gNB-CU stops forwarding the RAN visible QoE reports to the gNB-DU.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

- Text Proposal for 38.473

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

8.16.x	QoE Information Transfer Control

8.16.x.1	General

The purpose of the QoE Information Transfer Control procedure is to control the RAN visible QoE information transfer from the gNB-CU to the gNB-DU. The procedure uses non-UE associated signalling.

8.16.x.2	Successful operation

Figure 8.16.x.2-1: QoE Information Transfer Control procedure.

The gNB-DU initiates the procedure by sending the QOE INFORMATION TRANSFER CONTROL message to the gNB-CU.

If the Deactivation Indication List IE is present in the message, the gNB-CU shall, if supported, deactivate the QoE information transfer from gNB-CU to gNB-DU for the UEs indicated therein, as specified in TS 38.401[4].

8.16.x.3	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>