# 3GPP TSG-WG3 Meeting #122   	                  R3-237671

# Chicago, USA, Nov.13th-17th, 2023

# Source: 		CMCC

# Title:         (TP to TS 38.300) SL relay service continuity

# Agenda item:	16.3

Document Type:	Other

Introduction

In SL relay service continuity, the SL-RSRP and SD-RSRP are used to evaluate the transmission conditions between the remote UE and serving relay UE. As RAN2’ clarification, the L2 U2N remote UE should indicate the type of measurement report result, whether it is SL-RSRP or SD-RSRP.

- For i2i and i2d path switch procedures, the U2N remote UE applies the SL-RSRP threshold when measuring SL-RSRP and the SD-RSRP threshold when measuring SD-RSRP.

- Both SL-RSRP and SD-RSRP thresholds are expected to be available to the UE.  FFS signalling details (e.g., if the second one defaults to be equal to the first).

- The L2 U2N Remote UE indicates whether it is reporting SL-RSRP and SD-RSRP.  ASN.1 details and procedural impact to be worked out in CR implementation.

In the current TS38.300 BLCR for service continuity, the above progress should be captured for i2x path switching. So, in this TP, we add corresponding description to remote UE measurement and report behaviors in 16.12.6.1 and 16.12.6.x.

TP to TS 38.300

----------------Start of the First Change---------------

16.12.6.1 Switching from indirect to direct

For service continuity of L2 U2N Relay, the following procedure is used, in case of L2 U2N Remote UE switching from indirect to direct path under the same gNB:

Figure 16.12.6.1-1: Procedure for L2 U2N Remote UE intra-gNB indirect to direct path switching

1.	The Uu measurement configuration and measurement report signalling procedures are performed to evaluate both relay link measurement and Uu link measurement. The measurement results from L2 U2N Remote UE are reported when configured measurement reporting criteria are met. The sidelink relay measurement report shall include at least L2 U2N Relay UE's source L2 ID, serving cell ID (i.e., NCGI/NCI), and sidelink measurement quantity result. The sidelink measurement quantity can be SL-RSRP of the serving L2 U2N Relay UE, and if SL-RSRP is not available, SD-RSRP is used. The type of sidelink measurement quantity result of serving L2 U2N Relay UE (i.e., SL-RSRP, SD-RSRP) should be indicated to the gNB.

2.	The gNB decides to switch the L2 U2N Remote UE onto direct Uu path.

3.	The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The L2 U2N Remote UE stops UP and CP transmission via the L2 U2N Relay UE after reception of the RRCReconfiguration message with the path switch configuration.

4.	The L2 U2N Remote UE synchronizes with the gNB and performs Random Access.

5.	The UE (i.e., L2 U2N Remote UE in previous steps) sends the RRCReconfigurationComplete message to the gNB via the direct path, using the configuration provided in the RRCReconfiguration message. From this step, the UE (i.e., L2 U2N Remote UE in previous steps) uses the RRC connection via the direct path to the gNB.

6.	The gNB sends the RRCReconfiguration message to the L2 U2N Relay UE to reconfigure the connection between the L2 U2N Relay UE and the gNB. The RRCReconfiguration message to the L2 U2N Relay UE can be sent any time after step 3 based on gNB implementation (e.g., to release Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE).

7.	Either L2 U2N Relay UE or L2 U2N Remote UE's AS layer can release PC5-RRC connection and indicates upper layers to release PC5 unicast link after receiving the RRCReconfiguration message from the gNB. The timing to execute link release is up to UE implementation.

8.	The data path is switched from indirect path to direct path between the UE (i.e., previous L2 U2N Remote UE) and the gNB. The PDCP re-establishment or PDCP data recovery in uplink is performed by the UE (i.e., previous L2 U2N Remote UE) for lossless delivery during path switch if gNB configures it.

NOTE:	Step 8 can be executed any time after step 4. Step 8 is independent of step 6 and step 7.

For service continuity of L2 U2N Relay, the following procedure is used, in case of L2 U2N Remote UE switching from indirect to direct path under another gNB:

Figure 16.12.6.1-2: Procedure for L2 U2N Remote UE inter-gNB indirect to direct path switching

1.	The Uu measurement configuration is configured by the source gNB, and measurement report signalling procedures are performed by the L2 U2N Remote UE to evaluate both relay link measurement and Uu link measurement. The measurement results from L2 U2N Remote UE are reported when configured measurement reporting criteria are met. The sidelink relay measurement report shall include at least L2 U2N Relay UE's source L2 ID, serving cell ID (i.e., NCGI/NCI), and sidelink measurement quantity result. The sidelink measurement quantity can be SL-RSRP of the serving L2 U2N Relay UE, and if SL-RSRP is not available, SD-RSRP is used. The type of sidelink measurement quantity result of serving L2 U2N Relay UE (i.e., SL-RSRP, SD-RSRP) should be indicated to the gNB.

2.	The source gNB decides to trigger path switch for the L2 U2N Remote UE, onto direct path.

3.	The source gNB sends the HANDOVER REQUEST message to the target gNB with necessary information to prepare the handover at the target side.

NOTE: In order to support the DL lossless handover for the L2 U2N Remote UE, the source gNB may not discard the DL data even though the delivery of the data have been acknowledged by the L2 U2N Relay UE based on the gNB implementation. Then, the source gNB forwards the buffered DL data to the target gNB during the data forwarding procedure.

4.	Admission Control may be performed by the target gNB.

5.	The target gNB prepares the path switch and sends the HANDOVER REQUEST ACKNOWLEDGE message to the source gNB, which contains new RRC configuration for the L2 U2N Remote UE.

6.	The source gNB triggers the path switch by sending an RRCReconfiguration message to the L2 U2N Remote UE, containing at least cell ID and the information required to access the target cell. The L2 U2N Remote UE stops UP and CP transmission via the L2 U2N Relay UE after reception of the RRCReconfiguration message.

7. 	The source gNB sends the SN STATUS TRANSFER message to the target gNB to convey the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status of the L2 U2N Remote UE's DRBs for which PDCP status preservation applies (i.e. for RLC AM).

8. 	The L2 U2N Remote UE synchronizes with the target gNB and performs Random Access.

9.	The L2 U2N Remote UE sends RRCReconfigurationComplete message to target gNB via the direct path.

10.	The target gNB sends the UE CONTEXT RELEASE to inform the source gNB about the success of the path switch.

11.	The source gNB sends RRCReconfiguration message to the L2 U2N Relay UE to reconfigure the connection between the L2 U2N Relay UE and the source gNB. The RRCReconfiguration message to the L2 U2N Relay UE can be sent any time after step 6 based on source gNB implementation (e.g., to release Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE).

12.	Either L2 U2N Relay UE or L2 U2N Remote UE’s AS layer can release PC5-RRC connection and indicates upper layer to release PC5 unicast link after receiving the RRCReconfiguration message from the source gNB. The timing to execute link release is up to UE implementation.

16.12.6.x Switching from indirect to indirect

Notes: RAN2 can put the description of intra-gNB i2i path-switching procedures in the 38.300 draft spec.

For service continuity of L2 U2N Remote UE, the following procedure is used, in case of the L2 U2N Remote UE switching from indirect to indirect path under another gNB via a L2 U2N Relay UE in RRC_CONNECTED:

Figure 16.12.6.x-2: Procedure for L2 U2N Remote UE inter-gNB indirect to indirect path switching

1.	The L2 U2N Remote UE reports one or multiple candidate L2 U2N Relay UE(s) and Uu measurements to the source gNB, after it measures/discovers the candidate L2 U2N Relay UE(s):

-     The L2 U2N Remote UE filters the appropriate L2 U2N Relay UE(s) according to relay selection criteria before reporting. The L2 U2N Remote UE shall report only the L2 U2N Relay UE candidate(s) that fulfil the higher layer criteria;

-     The reporting includes at least a L2 U2N Relay UE ID, a L2 U2N Relay UE' s serving cell ID, and a sidelink measurement quantity information. SD-RSRP is used as sidelink measurement quantity for the candidate L2 U2N Relay UE(s). The sidelink measurement quantity can be SL-RSRP of the serving L2 U2N Relay UE, and if SL-RSRP is not available, SD-RSRP is used. The type of sidelink measurement quantity result of serving L2 U2N Relay UE (i.e., SL-RSRP, SD-RSRP) should be indicated to the gNB.

2.	The source gNB decides to trigger the L2 U2N Remote UE to switch to an indirect path of another gNB.

3.	The source gNB sends a HANDOVER REQUEST message to the target gNB to prepare the path switch at the target side. The HANDOVER REQUEST message includes Remote UE L2 ID and a list of candidate target relay UE IDs belonging to one cell.

NOTE: In order to support the DL lossless handover for the L2 U2N Remote UE, the source gNB may not discard the DL data even though the delivery of the data have been acknowledged by the L2 U2N Relay UE based on the gNB implementation. Then, the source gNB forwards the buffered DL data to the target gNB during the data forwarding procedure.

4.	Admission Control may be performed by the target gNB.

5. 	The target gNB selects one target Relay UE from the list of candidate Relay UEs provided by the source gNB, sends the RRCReconfiguration message to the L2 U2N Relay UE for relaying configuration, which includes at least the L2 U2N Remote UE's local ID and L2 ID, Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration.

6.	The target gNB sends the HANDOVER REQUEST ACKNOWLEDGE to the source gNB, which contains new RRC configuration for L2 U2N Remote UE.

7. 	The source gNB sends the RRCReconfiguration message to the the L2 U2N Remote UE, which includes at least the target L2 U2N Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic and the associated end-to-end radio bearer(s). The L2 U2N Remote UE stops UP and CP transmission over the (source) indirect path after reception of the RRCReconfiguration message from the source gNB.

8. The source gNB sends the SN STATUS TRANSFER message to the target gNB to convey the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status of the L2 U2N Remote UE's DRBs for which PDCP status preservation applies (i.e. for RLC AM).

9.The L2 U2N Remote UE establishes PC5 connection to target the L2 U2N Relay UE.

10.	The L2 U2N Remote UE sends the RRCReconfigurationComplete message to the target gNB via the target L2 U2N Relay UE.

11.	The data path is switched from indirect path to indirect path between the L2 U2N Remote UE and the target gNB via the target L2 U2N Relay UE.

12.	The target gNB sends the UE CONTEXT RELEASE message to inform the source gNB about the success of the path switch.

13.	The source gNB sends the RRCReconfiguration message to the source L2 U2N Relay UE to reconfigure the connection between the source L2 U2N Relay UE and the source gNB. The RRCReconfiguration message to the L2 U2N Relay UE can be sent any time after step 6 based on source gNB implementation (e.g., to release Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE).

14. Either L2 U2N Relay UE or L2 U2N Remote UE’s AS layer can release PC5-RRC connection and indicates upper layer to release PC5 unicast link after receiving the RRCReconfiguration message from the source gNB. The timing to execute link release is up to UE implementation.

-----------------End of the first Change---------------