3GPP TSG-RAN WG3 Meeting #122	R3-237705

Chicago, USA, 13th – 17th November 2023

Agenda Item:	10.2.2

Source:	ZTE, China Telecom, China Unicom

Title:	(TP for SON BL CR 38.413) MRO for inter-system handover for voice fallback

Document for:	Approval

1	Introduction

In RAN3#121 meeting, the MRO for inter-system handover for voice fallback has been discussed, and the agreement has been reached as below.

Convert the WA into agreement “Define a new handover report type in the Inter-system HO Report over S1 and NG”.

While, in RAN3#121bis meeting, since the time is limited, there was no time for companies to further discuss the details of the IE design.

In this contribution, we provide the discussion on the details of the new handover report type and the corresponding stage 3 TP.

2	Discussion

In RAN3#121 meeting, the “Inter-system Mobility Failure for Voice Fallback” has been defined as one the handover report type in the Inter-system HO Report over NG[1]. While the details have not been completed.

According to the agreed use cases for inter-system handover for voice fallback as given below, after the RLF due to Voice Fallback, the UE shall attempt to re-connect a E-UTRAN cell. While, after the HOF due to Voice Fallback, the UE shall attempt to re-connect a E-UTRAN cell or a NG-RAN cell.

Consider Case 1-2 for MRO enhancements for inter-system inter-RAT handover for voice fallback: - Case 1: after failure (HOF/RLF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, a suitable E-UTRA cell is selected, and the UE tries RRC connection setup procedure for the voice service in the E-UTRA cell. - Case 2: after failure (HOF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, none suitable E-UTRAN cell can be selected, the UE reverts back to the configuration of the source PCell and initiates RRC re-establishment procedure in NR.

Furthermore, after the HOF due to Voice Fallback, the NR RLF Report shall be generated and it could be stored in the UE for 48 hours. When the UE moves back to NR, the NR RLF Report could be sent to the source NR cell by using the existing Xn or NG procedures. And the source NG-RAN node is able to decode the NR RLF Report. Therefore, there is no RAN3 impact for the HOF case.

While, after the RLF due to Voice Fallback, the LTE RLF Report is generated and it cannot be decoded directly by the NG-RAN node after the UE moves back to NR. Therefore, the LTE RLF Report should be sent to the new E-UTRAN cell for decoding. Then, the RLF related information should be sent from the E-UTRAN cell to the source NG-RAN cell. Thus, the S1 and NG interfaces should be enhanced to support the inter-system handover for voice fallback.

The above analysis is the intention of introducing the new handover report type over NG and S1. And the new handover report type “Inter-system Mobility Failure for Voice Fallback” was introduced only for the RLF case due to Voice Fallback.

Observation: The new inter-system report type “Inter-system Mobility Failure for Voice Fallback” was introduced only for the RLF case due to Voice Fallback

Furthermore, the name “Re-connected Cell ID” could be more appropriate, and the re-connected cell should be an E-UTRAN cell to follow the description of the agreed Case 1. Meanwhile, the LTE RLF Report should be optional, since it is not essential for the source NG-RAN node.

Proposal: In the Inter-system Mobility Failure for Voice Fallback, the re-connect cell should be an E-UTRAN cell and the UE RLF Report should be optional.

3	Conclusion

Observation: The new inter-system report type “Inter-system Mobility Failure for Voice Fallback” was introduced only for the RLF case due to Voice Fallback

Proposal: In the Inter-system Mobility Failure for Voice Fallback, the re-connect cell should be an E-UTRAN cell and the UE RLF Report should be optional.

The corresponding TP for SON BL CR 38.413 on MRO for voice fallback is given in the Annex.

References

- R3-234625, Introduce a new handover report type for inter-RAT voice fallback, CATT, Nokia, Nokia Shanghai Bell, ZTE, Samsung

Annex

Text Proposal for SON BL CR 38.413 on MRO for voice fallback

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.40	Inter-system HO Report

This IE contains the inter-system HO report to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Handover Report Type | M |  |  |  |  |  |
| >Too early Inter-system HO |  |  |  |  | - |  |
| >>Source Cell ID | M |  | E-UTRA CGI 9.3.1.9 | CGI of the source cell for the HO. |  |  |
| >>Failure Cell ID | M |  | NG-RAN CGI 9.3.1.73 | CGI of the target cell for the HO. |  |  |
| >>UE RLF Report Container | O |  | 9.3.3.41 |  |  |  |
| >Inter-system Unnecessary HO |  |  |  |  | - |  |
| >>Source Cell CGI | M |  | NG-RAN CGI 9.3.1.73 | Source NR cell in NG-RAN |  |  |
| >>Target Cell CGI | M |  | E-UTRA CGI 9.3.1.9 | Target cell in E-UTRAN |  |  |
| >>Early IRAT HO | M |  | ENUMERATED (true, false, ...) | Is set to “true” if the measurement period expired due to an inter-RAT handover towards NR executed within the configured measurement duration and otherwise set to “false” |  |  |
| >>Candidate Cell List |  | 1 |  |  |  |  |
| >>>Candidate Cell Item |  | 1..<maxnoofCandidateCells> |  |  |  |  |
| >>>>CHOICE Candidate Cell Type | M |  |  |  |  |  |
| >>>>>Candidate CGI |  |  |  |  |  |  |
| >>>>>>Candidate Cell ID | M |  | NR CGI 9.3.1.7 | This IE contains an NR CGI. |  |  |
| >>>>>Candidate PCI |  |  |  |  |  |  |
| >>>>>>Candidate PCI | M |  | INTEGER (0..1007, …) | This IE includes the NR Physical Cell Identifier of detected cells not included in the Candidate Cell List IE and for which an NR CGI could not be derived. |  |  |
| >>>>>>Candidate NR ARFCN | M |  | INTEGER (0.. maxNRARFCN) | RF Reference Frequency as defined in TS 38.104 [39], section 5.4.2.1. The frequency provided in this IE identifies the absolute frequency position of the reference resource block (Common RB 0) of the carrier. Its lowest subcarrier is also known as Point A. |  |  |
| >Inter-system Mobility Failure for Voice Fallback |  |  |  |  | YES | ignore |
| >>Source Cell ID | M |  | NG-RAN CGI 9.3.1.73 | CGI of the source cell for HO |  |  |
| >>Failure Cell ID | M |  | E-UTRA CGI 9.3.1.9 | CGI of the failure cell for HO |  |  |
| >>Re-connected Cell ID | O |  | E-UTRA CGI 9.3.1.9 | CGI of the re-connected cell after HO |  |  |
| >>UE RLF Report Container | O |  | 9.3.3.41 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCandidateCells | Maximum no. of candidate cells. Value is 32 |
| maxNRARFCN | Maximum value of NRARFCNs. Value is 3279165. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

InterSystemHandoverReportType ::= CHOICE {

tooearlyIntersystemHO				TooearlyIntersystemHO,

intersystemUnnecessaryHO			IntersystemUnnecessaryHO,

choice-Extensions		ProtocolIE-SingleContainer { { InterSystemHandoverReportType-ExtIEs} }

}

InterSystemHandoverReportType-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-intersystemMobilityFailureforVoiceFallback  CRITICALITY ignore	TYPE  IntersystemMobilityFailureforVoiceFallback 	PRESENCE mandatory },

...

}

IntersystemMobilityFailureforVoiceFallback ::= SEQUENCE {

sourcecellID			NGRAN-CGI,

targetcellID			EUTRA-CGI,

reconnectedCellID	EUTRA-CGI                      OPTIONAL,

uERLFReportContainer	UERLFReportContainer		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { IntersystemMobilityFailureforVoiceFallback-ExtIEs} }			OPTIONAL,

...

}

IntersystemMobilityFailureforVoiceFallback-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>