3GPP TSG-RAN WG3 Meeting #122	R3-237716

Chicago , US, 13 Nov – 17 Nov 2023

Title: 	(TP to BL CR of 38.300 38.423 37.340) QoE in NR-DC

Source: 	China Unicom

Agenda item:	11.3

Document Type:	Discussion

1	Introduction

This paper provides the text proposal for the BL CR of 38.300, based on the discussion in R3-234811.

This paper provides the text proposal for the BL CR of 38.423, based on the discussion in R3-235993.

This paper provides the text proposal for the BL CR of 37340, based on the BLCR in R3-235989.

2	TP to BL CR of 38.300

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 21.2.3	Handling of QMC during RAN Overload

The QoE measurement collection pause/resume procedure is used to pause/resume reporting of one or multiple QoE measurement configurations in a UE in RAN overload situation.

For a QoE measurement configuration, the assistance information provided by the OAM may be considered by the gNB for deciding whether to pause/resume the measurement reporting of certain QoE measurement configurations in case of RAN overload.

In NR-DC senario, the network explicitly indicates to the UE whether to send QoE reports via SRB4 or SRB5, per QoE reference.The SRB for QoE reporting can be changed during the application session in case of RAN overload. The command for changing the SRB used for reporting may be sent to the UE by the node that configured that specific QoE configuration. The node that currently receives the QoE reports via the Uu can request from the peer node that the QoE reporting leg is switched to the peer node per QoE reference. The leg switch for each QoE reporting needs to be approved by both nodes serving the UE.

Editor’s Note: Whether the assistance information for QoE Measurement Collection will be used by UE to decide which QoE reports to discard in case the UE’s buffer becomes full depends on RAN2 decision.

The gNB can use the RRCReconfiguration message to temporarily stop the UE from sending application layer measurement reports associated with one or multiple application layer measurement configurations. When the UE receives the QoE measurement collection pause indication, the UE temporarily stores application layer measurement reports in AS layer. When the UE receives the QoE measurement collection resume indication, the UE sends the stored application layer measurement reports to the gNB.

-------------------------------------------Next change-------------------------------------------

### 21.x.2	Support for RAN visible QoE measurements and reporting in NR-DC

Either the MN or the SN can generate and send a RAN visible QoE configuration to the UE. The gNB that has initially configured a UE in NR-DC with an RVQoE configuration can modify and release the RAN visible QoE configuration as long as the UE is connected to this gNB. The gNB that configures the encapsulated QoE to UE is the RAN visible QoE-configuring gNB, the peer node is the non-RAN visible QoE-configuring gNB. Upon mobility, the RAN visible QoE-configuring gNB may be changed.

The UE may send RAN visible QoE reports to the network using either SRB4 or SRB5. In addition, the gNB that received a RAN visible QoE report can forward the report to the other gNB (the SN or the MN). QoE reports and RAN visible QoE reports pertaining to the same QoE Reference can be sent over the same SRB or they can be sent over different SRBs.

The RAN visible QoE-configuring gNB can configure RAN visible QoE measurements at a UE without a priori knowledge about which gNB(s) will provide the bearer(s) for a future application session. During the lifetime of an application session, to ensure that the RAN visible QoE reports are sent to the gNB(s) that provide the bearer(s) which carry the data flow(s) associated with the RAN visible QoE measurement result in a RAN visible QoE report, the gNB receiving the RAN visible QoE reports determines the bearer(s) used to deliver the application session data flow(s) and the associated gNB (s). The determination may be based on the PDU session ID(s) and the QoS flow ID(s) indicated in a received RAN visible QoE report.

When the RAN visible QoE-configuring gNB receives a RAN visible QoE measurement report and determines that the non-RVQoE-configuring gNB provides at least one bearer for the application session, the RVQoE-configuring gNB indicates to the non-RVQoE-configuring gNB about the available RAN visible QoE metrics and current reporting periodicity, and initiates the coordination procedure for the reporting path of the subsequent RAN visible QoE reports . The non-RVQoE-configuring gNB can then, if needed, indicate to the RAN visible QoE-configuring gNB its preference with respect to the reporting path for the subsequent RAN visible QoE reports and its preferred RAN visible QoE configuration parameters.

If a gNB receives a RAN visible QoE report from a UE in NR-DC, and determines that the bearer(s) for the application session data flow(s) is (are) also provided by the other gNB, or only, provided by the other gNB, the gNB that received the RAN visible QoE measurement report may forward the received RAN visible QoE report to the other gNB. The RAN visible QoE configuration may also be modified or released.

Editor Note: FFS on further refinements and the placement of the above text.The naming “RVQoE-configuring gNB” and “non-RVQoE-configuring gNB” are to be further discussed.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

3 TP to BL CR of 38.423

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.x1	QMC Coordination Request

This IE contains the information that the S-NG-RAN node needs to provide to the M-NG-RAN node, or the information that the M-NG-RAN node needs to provide to the S-NG-RAN node, for managing configuration and reporting of one or more QoE and/or RAN visible QoE measurements.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MN to SN QMC Coordination Request List |  | 0..1 |  |  |
| >MN to SN QMC Coordination Request Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) |  |
| >>Measurement Configuration Application Layer ID | O |  | INTEGER (0..15, ...) |  |
| >>Measurement Collection Entity IP Address | O |  | Transport Layer Address 9.2.3.29 | The IP address of the entity receiving the QoE measurement report. |
| >>QoE Reporting Path Request | O |  | ENUMERATED (SRB4, SRB5, …) | This IE indicates the preferred SRB for receiving the QoE reports. |
| >>RAN Visible QoE Reporting Path Request | O |  | ENUMERATED (SRB4, SRB5, …) | This IE indicates the preferred SRB for receiving the RAN Visible QoE reports. |
| >>Further RAN Visible QoE Interest Inquiry | O |  | BOOLEAN | This IE is used to request the S-NG-RAN node to indicate whether it is interested in receiving further RVQoE reports. |
| >>Current RAN Visible QoE Configuration | O |  | 9.2.3.x5 | This IE is to indicate the current RAN Visible QoE configuration and inquire about the RAN Visible QoE Configuration preference of the S-NG-RAN node. |
| SN to MN QMC Coordination Request List |  | 0..1 |  |  |
| >SN to MN QMC Coordination Request Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) |  |
| >>QoE Reporting Path Request | O |  | ENUMERATED (SRB4, SRB5, ...) | This IE indicates the preferred SRB for receiving the QoE reports. |
| >>RAN Visible QoE Reporting Path Request | O |  | ENUMERATED (SRB4, SRB5, ...) | This IE indicates the preferred SRB for receiving the RAN Visible QoE reports. |
| >>Further RAN Visible QoE Interest Inquiry | O |  | BOOLEAN | This IE is used to request the M-NG-RAN node to indicate whether it is interested in receiving further RVQoE reports. |
| >>Current RAN Visible QoE Configuration | O |  | 9.2.3.x5 | This IE is to indicate the current RAN Visible QoE configuration and inquire about the RAN Visible QoE Configuration preference of the M-NG-RAN node. |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

Editor’s Note: Whether to also include QoE configuration container in the IE is FFS.

#### 9.2.3.x2	QMC Coordination Response

Editor’s Note: The messages used to convey this IE are FFS.

This IE contains the information that the M-NG-RAN node needs to provide to the S-NG-RAN node, or the information that the S-NG-RAN node needs to provide to the S-NG-RAN node in response to the QMC Coordination Request.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MN to SN QMC Configuration Response List |  | 0..1 |  |  |
| >MN to SN QMC Coordination Response Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) |  |
| >>Measurement Configuration Application Layer ID | O |  | INTEGER (0..15, ...) |  |
| >>QoE Configuration Sending Path | O |  | ENUMERATED (MN, SN, …) |  |
| >>QoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  |
| >>RVQoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  |
| >>Further RAN Visible QoE Interest Response | O |  | ENUMERATED (interested, not-interested, ...) | This IE is to indicate whether the M-NG-RAN node is interested in receiving further RAN Visible QoE reports. |
| >>RAN Visible QoE Configuration Preference | O |  | 9.2.3.x5 | The RAN Visible QoE Configuration preference generate by the M-NG-RAN node. |
| SN to MN QMC Configuration Response List |  | 0..1 |  |  |
| >SN to MN QMC Coordination Response Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) |  |
| >>QoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  |
| >>RVQoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  |
| >>Further RAN Visible QoE Interest Response | O |  | ENUMERATED (interested, not-interested, ...) | This IE is to indicate whether the S-NG-RAN node is interested in receiving further RAN Visible QoE reports. |
| >>RAN Visible QoE Configuration Preference | O |  | 9.2.3.x5 | The RAN Visible QoE Configuration preference generate by the S-NG-RAN node. |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

4	TP to BL CR of 37.340

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 13.x.2	QoE Measurement Configuration

#### 13.x.2.1	QoE Measurement Collection Activation and Reporting in NR-DC

For a UE in NR-DC, the MN and the SN may coordinate QoE measurement collection activation and reporting as follows:

For management-based QoE activation, the MN:

- Allocates the measurement configuration application layer ID, and indicates to SN if needed;

- Determines whether the MN or the SN sends the QoE configuration to the UE, in case SN enquires MN.

For management-based QoE measurement configuration received directly by the SN from the OAM, the SN may perform UE selection. For a selected UE, the SN indicates to the MN the QoE reference of the management-based QoE session and, separately for the QoE reports and RAN visible QoE reports, the SN indicates whether it is going to receive the corresponding reports via the MN (using SRB4) or using SRB5. Upon receiving the request, the MN can decide and notify the SN whether the MN sends the QoE and RAN visible QoE configuration to the UE, or whether the SN should send the configuration(s) to the UE. The SN can send a QoE and a RAN visible QoE measurement configuration directly to the UE via SRB3, or in a transparent container to the MN, which then sends it to the UE via SRB1.

For management-based QoE configurations received from the OAM and for the signalling based QoE configurations, the MN can only send the configuration to the UE via SRB1, and the UE can send the QoE reports via SRB4 or SRB5.

The network explicitly indicates to the UE whether to send QoE reports via SRB4 or SRB5, per QoE reference, separately for QoE reports and RAN visible QoE reports. The SRB for QoE reporting can be changed during the application session. The command for changing the SRB used for reporting may be sent to the UE by the node that configured that specific QoE configuration. The node that currently receives the QoE reports via the Uu can request from the peer node that the QoE reporting leg is switched to the peer node per QoE reference. The leg switch for each QoE reporting needs to be approved by both nodes serving the UE. RAN visible QoE reports can be sent over the same leg, as the QoE reports pertaining to the same QoE reference, or over the other leg.

The MN should inform the SN that a UE is configured with a management-based QoE/RAN visible QoE measurement configuration.

If the MN has configured the UE with QoE measurements, and if the UE is configured to send the QoE reports to the SN, then, if the MN decides that the SN forwards the reports directly to the MCE, the MN should indicate to the SN the QoE reference, the MCE IP address and the measConfigAppLayerId.

If the SN has configured the UE with QoE measurements, and if the UE is configured to send the QoE reports to the MN, then, if the SN decides that the MN forwards the reports directly to the MCE, the SN should indicate to the MN the QoE reference and the MCE IP address.

RAN visible QoE reports can be sent to the SN directly via the SRB5 if SRB5 is configured, or via the MN using SRB4.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>