3GPP TSG-RAN WG3 Meeting #122	R3-237736

Chicago, IL, USA, November 13th – 17th, 2023

Agenda item:	14.4

Source: 	LG Electronics Inc.

Title: 	Response to R3-237168 and solutions under discussion

Document for:	Discussion and Decision

# Introduction

SA3 and RAN2 have discussed on the suitable solution for S-CPAC security mechanism via LSes [1][2], based on which 4 options are currently on the table in SA3 [3].

However, we have some concerns because all the solutions under discussions in SA3 are either wrongly designed or prone to potential key mismatch. In this contribution, we provide our view and analysis, and seek the community to consider the right solution approach from the beginning.

# Discussion

With respect to S-CPAC security mechanism, RAN2 agreed the following and sent the reply LS to SA3 [2]:

- Rel-18 Conditional-Reconfiguration Information element may include - List of Group-ID (mapping to SN) and associated SK-counter values outside the candidate conditional configurations. - The Group-ID parameter is included within each candidate conditional configuration(CondConfigAddMod) marked for subsequent CPAC. - UE include the selected SK-counter value in the MN RRC Reconfiguration Complete message when UE selects new SK-counter value as part of SCPAC execution. Additional Note : RAN2 assumes that, during inter-SN SCPAC execution, the UE selects the first unused SK counter and the target SN selects the first unused SN key corresponding to this SK counter so the target SN and the UE are expected to use the same key in typical scenarios. In current PSCell change and CPAC scenarios, SN is provided with the security keys upon configuration of SCPAC as UL data transmission may start in parallel with the transmission of the RRC Reconfiguration Complete message. While RAN2 did not identify scenario where key mismatch can occur, as per RAN2 understanding the UE cannot detect whether the failure due to key mismatch at SN. Hence RAN2 concluded to include SK-counter in MN RRC Reconfiguration Complete message as indicated in Agreement 2. The network will use the received SK-counter value in the case of failure. - For Pcell-change /PSCell-change /SCG Release scenarios, if the SCPAC configuration is maintained, UE also maintains the unused SK-counter values. - RAN2 Understanding: The NW configuration ensures that The SK-counter lists assigned for SCPAC configurations and the SK-counter value assigned for CPAC configurations are uniquely different. No specification changes are needed in this regard. - No specification changes are needed for UE behavior for the Scenario where free SK-Counter not available at the time of execution. This scenario can be avoided by NW configuration.

Namely, RAN2 agreed that a list of unique SK counter values (per each candidate SN) will be configured to the UE, based on which the UE selects the first unused SK counter of the selected SN to derive the corresponding SN key to use whenever inter-SN S-CPAC execution happens (to that selected SN).

With that, from the RAN2’s agreements and responses in [2], we can observe the following:

- Seamless inter-SN mobility: RAN2 assumes the support of UL data transmission that may start in parallel with the transmission of RRCReconfigurationComplete to the MN. As mentioned, this has been supported in the legacy PSCell change and Rel-16/17 CPAC, because the selected SN has been provided with the SN key to use “before” the event happens. Because of this, when the event happens, UL and subsequent DL data can immediately communicate with the right SN key, together with sending RRCReconfigurationComplete, and the same behavior is expected for Rel-18 S-CPAC.

- Failure detection: The purpose of including SK counter in MN RRCReconfigurationComplete upon S-CPAC execution is for the failure detection by NW. As mentioned, the UE cannot detect the failure due to key mismatch at the selected SN – that’s why there should be some mechanism for NW to detect and thus RAN2 concluded that the UE supplies the selected SK counter value to MN. In other words, the selected SK counter value provided from the UE via RRCReconfigurationComplete was NOT designed for MN to configure the right SN key (or indicate which one to use among pre-configured) to the selected SN. If so, there always will be a delay and the seamless inter-SN mobility won’t be guaranteed.

Observation 1: From RAN2, it is observed that “Seamless inter-SN mobility” (as same for the legacy PSCell change and Rel-16/17 CPAC) is an important consideration, for which requires the SN key to use to be configured to the selected SN “before” the inter-SN execution happens.

Observation 2: From RAN2, it is also observed that the failure detection mechanism (the selected SK counter value provided from the UE via RRCReconfigurationComplete) was not designed for MN to configure the right SN key (or indicate which one to use among pre-configured) to the selected SN. If so, there always will be a delay and the seamless inter-SN mobility won’t be guaranteed.

Have these in mind, if we look at the 4 options under discussion in SA3 [3] (the figures excerpted from [4]):

| Option 1 | Option 2-1 |
| --- | --- |
| Option 2-2 | Option 3 |

Figure 1: The snapshot of 4 options currently under discussions in SA3 (excerpted from [4])

First, we can observe that three options (Options 1, 2-1, 2-2) rely on the SK counter value provided from the UE via RRCReconfigurationComplete in order for MN to configure the right SN key to use (or indicate which one to use among pre-configured) to the selected SN. In other words, these options wrongly rely on the failure detection mechanism and thus inevitably creates a delay to the selected SN. The seamless inter-SN mobility that RAN2 assumes cannot be achieved, because the selected SN knows the right SN key to use only “after” the event happens.

On the other hand, Option 3 doesn’t rely on the failure detection mechanism for the selected SN to know the right SN key to use. MN pre-configures a list of SN keys to each candidate SN in advance, which gives a room to at least allow seamless inter-SN mobility upon inter-SN S-CPAC execution. However, we still see that this option is prone to potential key mismatch unfortunately, because S-CPAC mobility is controlled by the UE and this option leaves each SN to decide on inter-SN mobility on its own. For example, if we assume that the UE was in SN1 and S-CPAC execution happens to another PSCell in the same SN1, then from SN1 point of view, it is hard to make sure whether the UE has remained in SN1, or whether there was another execution in-between where the UE accessed to a candidate PSCell in another SN and comes back to SN1. In the former case, the SN key doesn’t have be refreshed, while the latter requires new SN key, for which SN1 is oblivious.

One may argue that in the latter case, SN1 would receive the Xn-U ADDRESS INDICATION message from MN for the purpose of data forwarding upon the UE accessing to a candidate PSCell in another SN. But please note that data forwarding is a “optional” feature, and thus should not be used for such distinction of which SN key to use upon inter-SN S-CPAC execution.

In sum, we have reasonable concerns that none of the options under discussions are viable, either doesn’t support seamless inter-SN mobility as SN key to use (or which one) is configured or known to the selected SN only “after” the event happens (Options 1, 2-1, 2-2), or even if multiple SN keys are pre-configured, prone to potential key mismatch (Option 3).

Observation 3: The 4 options under discussion in SA3 (shown in Figure 1) are concerned because

- Three options (Options 1, 2-1, 2-2) wrongly rely on the failure detection mechanism and creates a delay to the selected SN. The seamless inter-SN mobility that RAN2 assumes cannot be achieved, because the selected SN knows the right SN key to use only “after” the event happens.

- On the other hand, Option 3, though it doesn’t rely on the failure detection mechanism for the selected SN to know the right SN key to use, MN pre-configuring a list of SN keys to each candidate SN in advance is still prone to potential key mismatch, because S-CPAC mobility is controlled by the UE and this option leaves each SN to decide on inter-SN mobility on its own (for which it is oblivious).

As mentioned, the reason why Option 3 could be prone to key mismatch is because it leaves up to each candidate SN to decide on whether inter-SN S-CPAC is executed or not, for which it is oblivious and the execution is completely controlled by the UE. But in our current design, MN was already made to know this due to MN RRCReconfigurationComplete, which carries the information to identify the SN of the selected candidate PSCell (see TS 37.340 [5]). For example, the following was excerpted from TS 37.340 Section 10.5.2. MN-initiated conditional SN change and SN-initiated conditional SN change:

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

5.	The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRCReconfiguration* message corresponding to the selected candidate PSCell, and sends an MN RRCReconfigurationComplete* message, including an NR RRCReconfigurationComplete** message for the selected candidate PSCell, and information enabling the MN to identify the SN of the selected candidate PSCell.

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

10.	The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRCReconfiguration* message corresponding to the selected candidate PSCell, and sends an RRCReconfigurationComplete* message, including an RRCReconfigurationComplete** message for the selected candidate PSCell, and information enabling the MN to identify the SN of the selected candidate PSCell.

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

Observation 4: In fact, in our current design, MN was already made to know whether inter-SN S-CPAC is executed or not, because MN RRCReconfigurationComplete carries the information to identify the SN of the selected candidate PSCell (see TS 37.340).

Considering this aspect and the observations above, the right solution we believe should be as follows – whenever inter-SN S-CPAC execution happens (e.g. from SN1 to SN2), then MN configures the next SN key to use to the previous serving SN (i.e. SN1):

Figure 2: The right solution approach proposed (MN configures the next SN key to use to the previous serving SN at inter-SN S-CPAC execution)

This approach essentially enables that a right SN key to use (and only one key) is pre-configured to each candidate SN at any point of time, to be used when the subsequent execution happens to one of its candidate PSCells. From this sense, the SN key handling is essentially the same to the legacy PSCell change and Rel-16/17 CPAC, and the seamless inter-SN mobility can be guaranteed.

Moreover, key mismatch can be avoided because MN RRCReconfigurationComplete is always sent whenever S-CPAC execution happens, and MN “always” knows for sure whether the execution was intra-SN or inter-SN (from the current serving SN). If intra-SN, then there is no need to update the SN key as the UE and the current serving SN have been already communicating based on the right SN key already. Only when inter-SN execution happens (i.e. only when necessary), MN can then go ahead to derive new key and update the next SN key to use to the previous serving SN, for which will be used when the subsequent execution happens to one of its candidate PSCells and the previous serving SN becomes the serving SN again.

Furthermore, the proposed solution has no RAN3 signalling impact. Unlike Option 3, it doesn’t require MN to pre-configure a list of SN keys during SN Addition. Given that MN just needs to update the SN key to the previous serving SN at the right time and only when necessary (inter-SN execution happens), the existing MN-initiated SN modification procedure can be re-used without any changes.

Only caveat could be that MN needs to store the SK counter values configured to the UE as it may use them later (from the first unused, per each candidate SN) to derive new SN key and update to the previous serving SN upon inter-SN execution. However, the SK counter value has been managed by “MN” from the legacy DC, where SN simply receives the SN key to use, derived and provided by MN. We don’t see that storing the SK counter values in MN’s UE context is not a critical show stopper for this elegant solution.

Observation 5: Based on the observations above, the right solution we believe should be as follows – whenever inter-SN S-CPAC execution happens (e.g. from SN1 to SN2), then MN configures the next SN key to use to the previous serving SN (i.e. SN1), as shown in Figure 2.

Observation 6: The following benefits are observed:

- A right SN key to use (and only one key) is pre-configured to each candidate SN at any point of time, to be used when the subsequent execution happens to one of its candidate PSCells. From this sense, the SN key handling is essentially the same to the legacy PSCell change and Rel-16/17 CPAC, and the seamless inter-SN mobility can be guaranteed.

- Key mismatch can be avoided because MN RRCReconfigurationComplete is always sent whenever S-CPAC execution happens, and MN “always” knows for sure whether the execution was intra-SN or inter-SN (from the current serving SN). Only when inter-SN execution happens (i.e. only when necessary), MN can then go ahead to derive new key and update the next SN key to use to the previous serving SN. No need for MN to derive and prepare multiple SN keys from the beginning.

- No RAN3 signalling impact is foreseen, given that MN just needs to update the SN key to the previous serving SN. The existing MN-initiated SN modification procedure can be re-used without any changes.

Observation 7: Only caveat could be that MN needs to store the SK counter values configured to the UE as it may use them later (from the first unused, per each candidate SN) to derive new SN key and update to the previous serving SN upon inter-SN execution. However, the SK counter value has been managed by “MN” from the legacy DC, where SN simply receives the SN key to use, derived and provided by MN. Storing the SK counter values in MN’s UE context should not be a show stopper.

Based on the discussions and observations above, we thus propose to consider the better solution for S-CPAC security mechanism described above, and request SA3 to take this into account.

Proposal 1: For S-CPAC security mechanism, RAN3 to agree the proposed better solution that MN simply configures the next SN key to use to the previous serving SN at inter-SN S-CPAC execution.

Proposal 2: Request SA3 to consider the proposed solution and send LS to SA3 as proposed in Annex.

# Conclusion

In the present contribution we make the following observations:

Observation 1: From RAN2, it is observed that “Seamless inter-SN mobility” (as same for the legacy PSCell change and Rel-16/17 CPAC) is an important consideration, for which requires the SN key to use to be configured to the selected SN “before” the inter-SN execution happens.

Observation 2: From RAN2, it is also observed that the failure detection mechanism (the selected SK counter value provided from the UE via RRCReconfigurationComplete) was not designed for MN to configure the right SN key (or indicate which one to use among pre-configured) to the selected SN. If so, there always will be a delay and the seamless inter-SN mobility won’t be guaranteed.

| Option 1 | Option 2-1 |
| --- | --- |
| Option 2-2 | Option 3 |

Figure 1: The snapshot of 4 options currently under discussions in SA3 (excerpted from [4])

Observation 3: The 4 options under discussion in SA3 (shown in Figure 1) are concerned because

- Three options (Options 1, 2-1, 2-2) wrongly rely on the failure detection mechanism and creates a delay to the selected SN. The seamless inter-SN mobility that RAN2 assumes cannot be achieved, because the selected SN knows the right SN key to use only “after” the event happens.

- On the other hand, Option 3, though it doesn’t rely on the failure detection mechanism for the selected SN to know the right SN key to use, MN pre-configuring a list of SN keys to each candidate SN in advance is still prone to potential key mismatch, because S-CPAC mobility is controlled by the UE and this option leaves each SN to decide on inter-SN mobility on its own (for which it is oblivious).

Observation 4: In fact, in our current design, MN was already made to know whether inter-SN S-CPAC is executed or not, because MN RRCReconfigurationComplete carries the information to identify the SN of the selected candidate PSCell (see TS 37.340).

Observation 5: Based on the observations above, the right solution we believe should be as follows – whenever inter-SN S-CPAC execution happens (e.g. from SN1 to SN2), then MN configures the next SN key to use to the previous serving SN (i.e. SN1), as shown in Figure 2.

Figure 2: The right solution approach proposed (MN configures the next SN key to use to the previous serving SN at inter-SN S-CPAC execution)

Observation 6: The following benefits are observed:

- A right SN key to use (and only one key) is pre-configured to each candidate SN at any point of time, to be used when the subsequent execution happens to one of its candidate PSCells. From this sense, the SN key handling is essentially the same to the legacy PSCell change and Rel-16/17 CPAC, and the seamless inter-SN mobility can be guaranteed.

- Key mismatch can be avoided because MN RRCReconfigurationComplete is always sent whenever S-CPAC execution happens, and MN “always” knows for sure whether the execution was intra-SN or inter-SN (from the current serving SN). Only when inter-SN execution happens (i.e. only when necessary), MN can then go ahead to derive new key and update the next SN key to use to the previous serving SN. No need for MN to derive and prepare multiple SN keys from the beginning.

- No RAN3 signalling impact is foreseen, given that MN just needs to update the SN key to the previous serving SN. The existing MN-initiated SN modification procedure can be re-used without any changes.

Observation 7: Only caveat could be that MN needs to store the SK counter values configured to the UE as it may use them later (from the first unused, per each candidate SN) to derive new SN key and update to the previous serving SN upon inter-SN execution. However, the SK counter value has been managed by “MN” from the legacy DC, where SN simply receives the SN key to use, derived and provided by MN. Storing the SK counter values in MN’s UE context should not be a show stopper.

Based on the discussion in the present contribution and the observations above we propose:

Proposal 1: For S-CPAC security mechanism, RAN3 to agree the proposed better solution that MN simply configures the next SN key to use to the previous serving SN at inter-SN S-CPAC execution.

Proposal 2: Request SA3 to consider the proposed solution and send LS to SA3 as proposed in Annex.

# Reference

[1]	S3-233200, LS on Security Solution for Selective SCG, SA3

[2]	R3-237168/R2-2311607, Reply LS on security for selective SCG activation, RAN2

[3] S3-234648, “draftCR on Security of Selective SCG Activation”, Huawei, HiSilicon, Qualcomm Incorporated, Ericsson, Nokia, Nokia Shanghai Bell, Samsung

[4] R3-237285, “Security issue on S-CPAC”, NTT DOCOMO INC.

[5] TS 37.340, E-UTRA and NR; Multi-connectivity; Stage 2

# Annex: [DRAFT] LS to SA3

Title:	[Draft] LS on the solution for S-CPAC security mechanism

Response to:

Release:	Rel-18

Work Item:	NR_Mob_enh2-Core

Source:	LG Electronics Inc. [To be RAN3]

To:	SA3

Cc:	RAN2

Contact Person:

Name: 	Jaemin Han

E-mail Address: 	jaeminh.han@lge.com

Send any reply LS to:           3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN3 thanks SA3 (and RAN2) for discussions on the suitable solution for S-CPAC security mechanism via the LSes communicated by S3-233200 and R2-2311607.

While RAN3 honors those discussions and agreements already made, RAN3 have concerns that none of the 4 options cunrrently under discussions in SA3 are viable, either doesn’t support seamless inter-SN mobility as SN key to use (or which one) is configured or known to the selected SN only “after” the event happens (Options 1, 2-1, 2-2), or even if multiple SN keys are pre-configured, prone to key mismatch (Option 3).

Considering the design’s potential impact on RAN3 signalling, RAN3 has evaulated the options and agreed for the better solution, i.e. whenever inter-SN S-CPAC execution happens (e.g. from SN1 to SN2), then MN configures the next SN key to use to the previous serving SN (i.e. SN1), as shown in the following figure:

Figure 1: The proposed solution from RAN3 (MN configures the next SN key to use to the previous serving SN at inter-SN S-CPAC execution)

RAN3 believes that this approach enables that a right SN key to use (and only one) is pre-configured to each candidate SN at any point of time, to be used when the subsequent execution happens to one of its candidate PSCells. From this sense, the SN key handling is essentially the same to the legacy PSCell change and Rel-16/17 CPAC, and the seamless inter-SN mobility that RAN2 assumes can be guaranteed.

In addition, RAN3 also believes that key mismatch can be avoided because RRCReconfigurationComplete is always sent to MN whenever S-CPAC execution happens, and MN “always” knows for sure whether the execution was intra-SN or inter-SN (from the current serving SN). Only when inter-SN execution happens (i.e. only when necessary), MN can then go ahead to derive new key and update the next SN key to use to the previous serving SN. There is no need for MN to derive and prepare multiple SN keys from the beginning. Furthermore, RAN3 also sees that no RAN3 signalling impact is foreseen with this approach, given that MN just needs to update the SN key to the previous serving SN. The existing MN-initiated SN modification procedure can be re-used without any changes.

With this, RAN3 respectively requests SA3 to consider the proposed solution described above when discussing the solution for S-CPAC security mechanism in SA3.

2. Actions:

To SA3

ACTION: 	RAN3 kindly asks SA3 to take the above information and the proposed solution into account when discussing the solution for S-CPAC security mechanism in SA3.

3. Date of Next TSG-RAN WG3 Meetings:

TSG RAN3 Meeting #123		February 26th – March 1st, 2024 		Athens, Greece

TSG RAN3 Meeting #123bis	April 15th – 19th, 2024 					China (TBC)