3GPP TSG-RAN WG3 Meeting #122	R3-237907

# Chicago, USA, November 13th – 17th 2023

Agenda Item:	10.2.2

Source:	Ericsson

Title:	(TP for SON BL CR for TS 38.413) SON enhancements for Mobility Robustness Optimization

Document for:	Discussion and Decision

# 1	Introduction

The TP in Annex captures agreements of RAN3#122.

# Annex: TP to SON BL CR to TS 38.413

<<<<<<<<<<<<<<<<<<<< 1st Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.40	Inter-system HO Report

This IE contains the inter-system HO report to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CHOICE Handover Report Type | M |  |  |  |  |  |  |
| >Too early Inter-system HO |  |  |  |  | - |  |  |
| >>Source Cell ID | M |  | E-UTRA CGI 9.3.1.9 | CGI of the source cell for the HO. |  |  |  |
| >>Failure Cell ID | M |  | NG-RAN CGI 9.3.1.73 | CGI of the target cell for the HO. |  |  |  |
| >>UE RLF Report Container | O |  | 9.3.3.41 |  |  |  |  |
| >Inter-system Unnecessary HO |  |  |  |  | - |  |  |
| >>Source Cell CGI | M |  | NG-RAN CGI 9.3.1.73 | Source NR cell in NG-RAN |  |  |  |
| >>Target Cell CGI | M |  | E-UTRA CGI 9.3.1.9 | Target cell in E-UTRAN |  |  |  |
| >>Early IRAT HO | M |  | ENUMERATED (true, false, ...) | Is set to “true” if the measurement period expired due to an inter-RAT handover towards NR executed within the configured measurement duration and otherwise set to “false” |  |  |  |
| >>Candidate Cell List |  | 1 |  |  |  |  |  |
| >>>Candidate Cell Item |  | 1..<maxnoofCandidateCells> |  |  |  |  |  |
| >>>>CHOICE Candidate Cell Type | M |  |  |  |  |  |  |
| >>>>>Candidate CGI |  |  |  |  |  |  |  |
| >>>>>>Candidate Cell ID | M |  | NR CGI 9.3.1.7 | This IE contains an NR CGI. |  |  |  |
| >>>>>Candidate PCI |  |  |  |  |  |  |  |
| >>>>>>Candidate PCI | M |  | INTEGER (0..1007, …) | This IE includes the NR Physical Cell Identifier of detected cells not included in the Candidate Cell List IE and for which an NR CGI could not be derived. |  |  |  |
| >>>>>>Candidate NR ARFCN | M |  | INTEGER (0.. maxNARFCNA) | RF Reference Frequency as defined in TS 38.104 [39], section 5.4.2.1. The frequency provided in this IE identifies the absolute frequency position of the reference resource block (Common RB 0) of the carrier. Its lowest subcarrier is also known as Point A. |  |  |  |
| > Inter-system Mobility Failure for Voice Fallback |  |  |  |  |  | YES | ignore |
| >>Source Cell ID |  | M |  | NG-RAN CGI 9.3.1.73 | CGI of the source cell for HO |  |  |
| >>Failure Cell ID |  | M |  | E-UTRA CGI 9.3.1.9 | CGI of the failure cell for HO |  |  |
| >>Re-connect Cell ID |  | O |  | E-UTRA CGI 9.3.1.9 | E-UTRA CGI of the cell where UE attempted re-establishment or where the UE successfully re-connected after the failure. |  |  |
| >>UE RLF Report Container |  | O |  | 9.3.3.41 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCandidateCells | Maximum no. of candidate cells. Value is 32 |
| maxNARFCN | Maximum value of NR carrier frequency, defined in TS 38.331 [18] |

<<<<<<<<<<<<<<<<<<<< Next  Change >>>>>>>>>>>>>>>>>>>>

IntersystemMobilityFailureforVoiceFallback ::= SEQUENCE {

sourcecellID			NGRAN-CGI,

targetcellID			EUTRA-CGI,

reconnectCellID	EUTRA-CGI                      OPTIONAL,

uERLFReportContainer	UERLFReportContainer		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { IntersystemMobilityFailureforVoiceFallback-ExtIEs} }			OPTIONAL,

...

}

IntersystemMobilityFailureforVoiceFallback-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>