3GPP TSG-RAN WG3 Meeting #122	R3-237944

Chicago, USA, 13th – 17th Nov 2023

Agenda item:	 10.2.1

Source:		Samsung

Title:	(TP for SON BLCR for TS38.300) SON enhancement for SHR

Document for:		Discussion and Decision

1	Introduction

This contribution provided a TP on SHR based on the agreements in RAN3#122 meeting.

Annex 2: TP for TS38.300

15.5.2.7	Successful HO

One of the functions of Mobility Robustness Optimization is to detect a sub-optimal successful handover event. The aim is to identify underlying conditions during successful ordinary handovers, successful DAPS handovers, or successful Conditional handovers.

For analysis of successful handover, the UE may collect Successful Handover Report (SHR) based on configuration by network, if stored, and makes the SHR available to the network as specified in TS 38.331 [12].

The UE stores the SHR until it is fetched by the network or for 48 hours after the SHR is recorded.

For SHR collected during intra-NR handover, if the target NR node fetches the SHR from the UE and the trigger of SHR is T310/T312, it forwards the information to the source NR node, i.e. the node handling the cell reported as source cell in this SHR, by using the ACCESS AND MOBILITY INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

If the NG-RAN node that fetches the SHR from the UE is neither the source node nor the target node of the handover, it forwards the information to the node(s) which configured the SHR trigger causing the SHR to be generated, by using the ACCESS AND MOBILITY INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

Upon retrieval of a SHR, the receiving node may analyse whether its mobility configuration needs adjustment.

For SHR collected during intra-system inter-RAT HO from NR to LTE, the correlation between the SHR and the RLF Report can be done based on the target C-RNTI and timing information (e.g., time between handover execution and the retrieval of the SHR, the time since failure to the retrieval of RLF Report) included in the RLF Report and the SHR.

The SHR report can be used to detect one case of Intra-system Too Late Handover, namely when DAPS HO is configured but an RLF is detected in the source cell during a successful DAPS HO.