3GPP TSG-RAN3 Meeting #122	R3-237972

Chicago, US, 13th – 17th November, 2023

Title: 	(TP to BL CR of 38.300) on support of QoE measurement enhancements for MBS service

Source: 	Huawei

Agenda item:	11.2

Document Type:	for approval

1. Introduction

In this contribution, we provide the TP to 38.300 based on the discussion in [1] [2] [3] [4].

2. TP to 38.300

-------------------------------------------Start change-------------------------------------------

## 21.3	QoE Measurement Continuity for Mobility

QoE measurement collection continuity for intra-system intra-RAT handover is supported, with the Area Scope parameters configured by the OAM, where the network is responsible for keeping track of whether the UE is inside or outside the area scope. A UE continues an ongoing QoE measurement even if it leaves the area scope, unless the network indicates to the UE to release the application layer measurement configuration.

For handover, the source gNB may transmit the information related to one or more application layer measurement configurations of the UE to the target gNB via XnAP or NGAP. For signalling-based QoE, the service type indication, QoE reference, and, optionally, the MCE IP address, measurement configuration application layer ID, MDT alignment information, area scope, slice support list for QMC, available RAN visible QoE metrics and measurement status are passed to the target gNB. For management-based QoE, the service type indication, measurement configuration application layer ID, the MCE IP address and QoE measurement status are passed to the target gNB. For RRC_INACTIVE state mobility, QoE measurement configuration(s) of a specific UE can be retrieved from the gNB hosting the UE context when it resumes to the RRC_CONNECTED state.

For signalling-based QoE, at handover to a target gNB that supports QoE measurement collection, the target gNB decides which of the application layer measurement configurations should be kept or released, e.g., based on application layer measurement configuration information received from the source gNB in Xn/NG signalling.

For QoE sessions pertaining to data flows received via MBS broadcast, QoE measurement collection may continue during the RRC_INACTIVE and RRC_IDLE states, and the measurement results, if collected, may be provided to the network when the UE returns to the RRC_CONNECTED state.

Upon UE’s transition from the RRC_IDLE to the RRC_CONNECTED state, the gNB serving the UE should ensure that it does not release an already configured signaling based QoE measurement configuration for the sake of configuring a new management-based QoE measurement configuration.

QoE measurements for ongoing sessions should be continued when switching between multicast and unicast transmission modes.

When the UE resumes the connection with a gNB that does not support QoE, the UE releases all application layer measurement configurations.

--- SKIP UNCHANGED PART ---

-------------------------------------------End of change-------------------------------------------

3. Reference

[1]	R3-237910, Summary of offline discussion on CB on QoE RRC_IDLE/INACTIVE, Huawei