3GPP TSG-RAN WG3#123bis 	R3-241529

Changsha, China, 15 - 19 April 2024

Agenda item:	13.2 Support for inter-CU LTM

Source:	Nokia

Title:	Discussion on LTM when NR-DC is Configured

Document for:	Discussion and Decision

# 1	Introduction

In regard to L1/L2 triggered mobility (LTM), a new Work Item has been approved (RP-234036 NR mobility enhancements Phase 4), including the following related objectives and scope impacting RAN3:

- Specify support for inter-CU Layer 2 Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - As secondary priority, support the case when NR-DC is configured and CU is acting as SN and MCG is unchanged - As secondary priority, support the case when NR-DC is configured, CU is acting as MN and SCG is unchanged or SCG is released - Note: The case that LTM is configured in both MCG and SCG is excluded - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1] - Specify support of conditional LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Prioritise intra-CU LTM - Checkpoint to review objective at RAN#105. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

This paper highlights LTM dual connectivity related aspects to be discussed in the scope of Rel-19.

# 2	LTM when NR-DC is Configured

LTM when NR-DC is configured is part of the objectives of Rel-19 WID for inter-CU LTM work.

In TS 37.340, different multi-connectivity scenarios and their respective signalling for PSCell addition, change, release or retention are described.  These cell changes or reconfigurations can be MN or SN initiated. Likewise, in Rel-18 various issues related to inter-SN PSCell change were discussed and agreements were made under S-CPAC feature that could be considered for LTM work in Rel-19.

Observation 1: In Rel-18 various issues related to inter-SN PSCell change were discussed and agreements were made under S-CPAC feature.

Proposal 1: RAN3 to study whether and to what extent the Rel-18 S-CPAC framework could be re-used in context for LTM when NR-DC is configured in Rel-19.

The following are NR-DC scenarios applicable for Rel-19 work scope:

- Inter-MN PCell change

  - With SCG released

  - With SCG retained

- Inter-SN PSCell change with unchanged PCell

  - MN initiated

  - SN initiated

Based on WID [1] scope, the scenario of “Inter-SN PSCell change with simultaneous PCell change” is excluded from Rel-19 work.

Proposal 2: RAN3 to prioritize the NR-DC scenarios to be specified under inter-CU LTM work in Rel-19.

An additional aspect is in regard the Note in the Rel-19 WID description [1], which indicates the following:

- As secondary priority, support the case when NR-DC is configured, CU is acting as MN and SCG is unchanged or SCG is released - Note: The case that LTM is configured in both MCG and SCG is excluded

This Note is ambiguous and can be interpreted in different ways, such as

- (Interpretation 1) In Rel-19 simultaneous MCG and SCG LTM configuration shall be deprioritized.

- (Interpretation 2) In Rel-19 simultaneous MCG and SCG LTM configuration shall not be supported.

Proposal 3: RAN3 to clarify understanding of the meaning of the note in inter0CU LTM WID that indicates  “The case that LTM is configured in both MCG and SCG is excluded”.

# 4	Conclusions

Observation 1: In Rel-18 various issues related to inter-SN PSCell change were discussed and agreements were made under S-CPAC feature.

Proposal 1: RAN3 to study whether and to what extent the Rel-18 S-CPAC framework could be re-used in context for LTM when NR-DC is configured in Rel-19.

Proposal 2: RAN3 to prioritize the NR-DC scenarios to be specified under inter-CU LTM work in Rel-19.

Proposal 3: RAN3 to clarify understanding of the meaning of the note in inter0CU LTM WID that indicates  “The case that LTM is configured in both MCG and SCG is excluded”.

# References

- RP-234036 NR mobility enhancements Phase 4, Intel (moderator)