3GPP TSG-RAN WG3 Meeting #123-bis	R3-241684

Changsha, China, 15 – 19 April, 2024

Title:	Access to local services from the 5G Femto via collocated local UPF

Agenda Item:	12.3

Source:	Huawei

Document for:	pCR

# 1	Introduction

The SID [1] provides the following objectives for 5G Femto:

The objectives of the 5G Femto study are as follows:

-	Study the overall RAN architecture and required functional and procedural impacts for supporting 5G Femto deployments [RAN3].

-	Study how to define the 5G access control mechanism by (re-)using the existing CAG functionality and identify needed enhancements (if any) [RAN3].

-	Clarify the access to local services from the 5G Femto via collocated local UPF and identify issues, if any [RAN3].

NOTE 1: The study involves a gap analysis of existing 5G functionality with HomeNB functionality.

NOTE 2: No impact on the UE.

NOTE 3: Coordination with other WGs (e.g. SA2) when needed.

This contribution mainly discusses on the access to local services from the 5G Femto via collocated local UPF.

# 2	Discussion

In LTE, the HeNB can support SIPTO (Selected IP Traffic Offload) or LIPA (Local IP Access) at the local network with a collocated L-GW. The L-GW is the gateway towards the local IP networks associated with the HeNB and can be viewed as a PDN GW. In this way, a UE accessing the HeNB can obtain the local services via the HeNB without the user plane traversing the mobile operator's network.

Fig.1 shows the architecture for the 4G local network service deployment.

Fig.1 4G Local network service logical architecture

According to TS 23.401, the HeNB with a collocated L-GW provides its L-GW IP address over S1-MME interface to the EPC. For example, the HeNB transfers the collocated L-GW IP address to the MME in every INITIAL UE MESSAGE message and every UPLINK NAS TRANSPORT message. The PDN GW selection function uses the L-GW IP address proposed by the HeNB for the local UE.

In TS 23.501, it is specified that the UPF function provides the UE with the user-plane service, when a UE requests a service, the PDU Session Establishment Request is transmitted transparently to the SMF via AMF, along with the UE location information containing the TAI, the Cell ID of the serving cell received by the N2 message. The SMF can map the UE location to the corresponding DNAI(s) associated with the requested service and select an appropriate UPF for the specific PDU session by considering the UE location and the corresponding DNAI(s).

The following are excerpted from TS 23.501:

DN	Data Network

DNAI	DN Access Identifier

### 5.6.2	Interaction between AMF and SMF

The AMF and SMF are separate Network Functions.

…

In order to support charging and to fulfil regulatory requirement (in order to provide NPLI (Network Provided Location Information) as defined in TS 23.228 [15]) related with the set-up, modification and release of IMS Voice calls or with SMS transfer the following applies

-	At the time of the PDU Session Establishment, the AMF provides the SMF with the PEI of the UE if the PEI is available at the AMF.

-	When it forwards UL NAS or N2 signalling to a peer NF (e.g. to SMF or to SMSF) or during the UP connection activation of a PDU Session, the AMF provides any User Location Information it has received from the 5G-AN as well as the Access Type (3GPP - Non 3GPP) of the AN over which it has received the UL NAS or N2 signalling.

…

### 5.6.7	Application Function influence on traffic routing

#### 5.6.7.1	General

…

The DNAIs are related to the information considered by the SMF for UPF selection and (I )SMF (re)selection, e.g. for diverting (locally) some traffic matching traffic filters provided by the PCF.

…

The SMF is responsible for handling the mapping between the UE location (TAI / Cell-Id) and DNAI(s) associated with UPF and applications and the selection of the UPF(s) that serve a PDU Session. This is described in clause 6.3.3.

…

### 6.3.3	User Plane Function Selection

#### 6.3.3.1	Overview

The selection and reselection of the UPF for PDU session establishment, UE mobility or UE traffic offloading are performed by the SMF by considering UPF deployment scenarios such as centrally located UPF and distributed UPF located close to or at the Access Network site. The selection of the UPF shall also enable deployment of UPF with different capabilities, e.g. UPFs supporting no or a subset of optional functionalities.

…

### 6.3.3.3	Selection of an UPF for a particular PDU Session

…

The following parameter(s) and information may be considered by the SMF for UPF selection and re-selection:

…

-	UPF location available at the SMF.

-	UE location information.

…

-	DNAI as included in the PCC Rules and described in clause 5.6.7.

…

-	Information related to user plane topology and user plane terminations, that may be deduced from:

-	5G-AN-provided identities (e.g. CellID, TAI), available UPF(s) and DNAI(s);

…

-	Information regarding the User plane termination(s) corresponding to DNAI(s).

…

The above extracted part illustrates that currently it is supported to deploy distributed UPF close to or at a gNB. Once the collocated UPF is online, the PCF provides the SMF with a PCC rule including the DNAI(s) generated based on the AF request related to the local service. Hence, when a local UE accessing the gNB initiates the local service request, the SMF can use the collocated UPF information for the corresponding PDU session by following the exsiting UPF selection principle.

Conclusion: 5G system supports local UPF deployment, there seems no need to introduce further enhancements on this objective.

# 3	Conclusion

This paper discusses on the access to local services from the 5G Femto via collocated local UPF, and gets the following conclusion:

Conclusion: 5G system supports local UPF deployment, there seems no need to introduce further enhancements on this objective.

The corresponding TP to TR 38.799 is provided in section 5.

# 4. Reference

- RP-234041, New SID: Study on additional topological enhancements for NR.

# 5. TP for TR 38.799 v0.0.0

------------Start of the First Change------------

[xx]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

------------Start of the Next Change------------

DNAI				Data Network Access Identifier

------------Start of the Next Change------------

## x.x	Local services access via collocated local UPF

The access to local services from the 5G Femto via collocated local UPF is supported in exiting 5G system, the distributed UPF can be deployed located close to or at the Access Network site, and during the UPF (re)selection, the SMF considers the UE location information, the UPF location and the corresponding DNAI(s) associated with a particular service, as specified in TS 23.501 [xx].

------------End of the Changes------------