3GPP TSG-RAN WG3 Meeting #123-bis	R3-241888

Changsha, China, 15 Apr – 19 Apr, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0382 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (CR to 38.401) Corrections on LTM procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, Huawei, CATT, Nokia, Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024.04.08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For intra-DU and inter-DU LTM, the LTM configuration ID and LTM configuration ID mapping list doesn’t need to be provided to the (source) DU in Step3, only the LTM configuration ID Mapping List should be send to the (source) DU in Step5. And for LTM with gNB-CU-UP change, the description of Step 5 and Step 6 does not align with the Figure 8.2.1.6-1. Therefore, in this CR, we updated some procedure description of LTM, and we also provided some editorial updates. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For intra-DU LTM: - In Step 3: Remove the LTM configuration ID and LTM configuration ID mapping list sent from the gNB-CU to the gNB-DU. - In Step 5: Add the LTM configuration ID Mapping List sent from the gNB-CU to the gNB-DU. - In Step 14: Update the “TCI state ID” to “TCI state ID(s)”. For inter-DU LTM: - In Step 3: Remove the LTM configuration ID and the LTM configuration ID mapping list sent from the gNB-CU to the candidate gNB-DU(s), add “(s)” to Candidate DU. - In Step 5: Update the description, clarify that the LTM configuration ID Mapping List is sent from the gNB-CU to the gNB-DU. - In Step 14, Remove “to the source gNB-DU” since the LTM related information is sent to the gNB-CU. - In Step 19&20: Update the “TCI state ID” to “TCI state ID(s)”. LTM with gNB-CU-UP change: - In Step 2&3&4: Update the “target gNB-CU-UP” to “candidate gNB-CU-UP”. - In Step 5&6: Update the procedure description. - In Figure 8.2.1.6-1: Update the “Souce DU” to “Source DU, update the “target gNB-CU-UP” to “candidate gNB-CU-UP”. Some editorial updates including: - Update the “target candidate cell(s)” to “candidate cell(s)”. - Update the “candidate target cell” to “target cell”. - Change “command” from lowercase to uppercase. - Remove extra comma. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because the changes only impact the procedure for LTM functions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguties in correspoding LTM procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.4, 8.2.1.5, 8.2.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

8.2.1.4	Intra-gNB-DU LTM

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for LTM. Figure 8.2.1.4-1 shows the intra-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.4-1: Intra-gNB-DU LTM

1.	The UE sends a MeasurementReport message (L3 measurement result) to the gNB-DU containing measurements of neighbouring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The gNB-CU determines to initiate LTM configuration.

3.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU for each candidate cell, containing one candidate cell ID of the candidate cell, and the CSI resource configuration. The gNB-CU may request PRACH resources from the gNB-DU. The gNB-CU may request the gNB-DU to provide the lower layer configuration for the purpose of generating the reference configuration or provide the lower layer reference configuration to the gNB-DU.

4.	If the gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations for the accepted candidate cell.

NOTE 1:	Steps 3 and 4 may be initiated multiple times for LTM candidate cell preparation of multiple cells including the source cell.

5.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message including the LTM configuration ID Mapping List to the gNB-DU. The message may include the updated CSI resource configuration.

6.	The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell.

NOTE 2:	In case of subsequent LTM, the CU-initiated UE Context Modification procedure may be invoked per each candidate cell to transfer to the gNB-DU the updated CSI resource configuration.

7.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

8.	The gNB-DU forwards the received RRCReconfiguration message to the UE.

9.	The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

10.	The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

11.	Early synchronization to the candidate cell(s) may be performed as specified in TS 38.300 [2].

12.	The UE sends the L1 measurement result to the gNB-DU. The gNB-DU decides to execute LTM.

13.	The gNB-DU sends the Cell Switch Command to the UE.

14.	The gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to the gNB-CU to indicate the initiation of the Cell Switch Command to the UE including the target cell ID and the TCI state ID(s).

15.	The gNB-DU detects the UE access in the target cell as specified in TS 38.300 [2].

16.	The gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

17.	The UE sends an RRCReconfigurationComplete message to the gNB-DU.

18.	The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

19.	The gNB-CU may send the UE CONTEXT MODIFICATION REQUEST message to the gNB-DU to release the resources of prepared cells.

20.	The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

8.2.1.5	Inter-gNB-DU LTM

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for LTM. Figure 8.2.1.5-1 shows the inter-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.5-1: Inter-gNB-DU LTM

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of neighbouring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The gNB-CU determines to initiate LTM configuration.

3.	The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s) for each candidate cell, containing one candidate cell ID of the candidate cell, and the CSI resource configuration. The gNB-CU may request PRACH resources from the candidate gNB-DU(s). The gNB-CU may request the candidate gNB-DU(s) to provide the lower layer configuration for the purpose of generating the reference configuration or provide the lower layer reference configuration to the candidate gNB-DU(s).

4.	If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configurations for the accepted target candidate cell.

NOTE 1:	The CU-initiated UE Context Modification procedure may be initiated for preparing candidate cells in the source gNB-DU as specified in step 3 and 4 in 8.2.1.4 Intra-gNB-DU LTM.

5.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the the information related to early sync and the LTM configuration ID Mapping List for the accepted candidate cell(s). The gNB-CU may send the updated CSI resource configuration to the source gNB-DU.

6.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell.

7.	The gNB-CU may send a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU(s) containing the information for subsequent LTM or for updating the configurations of candidate cells. The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s).

8.	The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the updated lower layer configuration (e.g., the updated CSI report configuration).

NOTE 2: Step 7 may also be triggered after step 19, or after step 22 by implementation for subsequent LTM.

9.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

10.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

11.	The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

12.	The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

13.	Early synchronization to the candidate cell(s) may be performed as specified in TS 38.300 [2].

14.The candidate gNB-DU sends the TA value, the associated CFRA resource information, the candidate cell ID and the source gNB-DU ID in the DU-CU TA INFORMATION TRANSFER message to the gNB-CU.

15.	The gNB-CU forwards the TA value, and the associated CFRA resource information to the source gNB-DU in the CU-DU TA INFORMATION TRANSFER message.

16.	The UE sends the L1 measurement result to the source gNB-DU.

17.	The source gNB-DU decides to execute LTM to a target cell.

18.	The source gNB-DU sends the Cell Switch Command to the UE.

19.	The source gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to the gNB-CU to indicate the initiation of the Cell Switch Command to the UE, for which the message includes the target cell ID and the TCI state ID(s).

20.	The gNB-CU forwards the target cell ID and the TCI state ID(s) to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

21.	The target gNB-DU detects the UE access as specified in TS 38.300 [2].

22.	The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

23.	The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

24.	The target gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

25.	The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

26.	The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

8.2.1.6	LTM with gNB-CU-UP change

Figure 8.2.1.6-1 shows the procedure used for LTM with the change of gNB-CU-UP within a gNB.

Figure 8.2.1.6-1 LTM with the change of gNB-CU-UP

0.	The source gNB-DU forwards the Measurement Report to the gNB-CU-CP.

1.	The gNB-CU-CP decides to initiate LTM configuration.

2.	The gNB-CU-CP sends a BEARER CONTEXT SETUP REQUEST message containing UL TNL address information for NG-U to setup the bearer context in the candidate gNB-CU-UP.

3.	The candidate gNB-CU-UP responds with a BEARER CONTEXT SETUP RESPONSE message containing the UL TNL address information for F1-U, DL TNL address information for NG-U, and the TNL address information for data forwarding to the candidate gNB-CU-UP.

4.	LTM configuration procedures are performed between gNB-CU and candidate gNB-DUs, and between gNB-CU and source gNB-DU as specified from step 3 to step 8 in section 8.2.1.5.

5 - 6.	The gNB-CU-CP sends the RRC Reconfiguration message to the source gNB-DU via DL RRC MESSAGE TRANSFER message, and receives the the RRCReconfigurationComplete message from the source gNB-DU via UL RRC MESSAGE TRANSFER message.

7.	The UE sends the lower layer measurement result to the source gNB-DU, and the source gNB-DU decides to execute LTM to a target cell.

8.	The source gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to the gNB-CU-CP with the selected target cell ID.

9.	The gNB-CU-CP forwards the selected target cell ID to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

10-11.	The gNB-CU-CP performs the Bearer Context Modification procedure to retrieve the PDCP UL/DL status and to exchange the TNL address information for data forwarding for the bearers to the source gNB-CU-UP.

12-13.	The gNB-CU-CP performs the Bearer Context Modification procedure to send the DL TNL address information for F1-U and the PDCP UL/DL status to the target gNB-CU-UP.

14.	Data Forwarding may be performed from the source gNB-CU-UP to the target gNB-CU-UP.

15.	The target gNB-DU detects the UE in the target cell.

16.	The target gNB-DU sends an ACCESS SUCCESS message to the gNB-CU-CP.

17-19.	Path Switch procedure is performed to update the DL TNL address information for the NG-U towards the core network.

20-21.	Bearer Context Release procedure may be performed to release the UE context in the source gNB-CU-UP.

END OF CHANGE