3GPP TSG-RAN WG3 Meeting #123bis	R3-242018

# Changsha, China, 15th - 19th April 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0387 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on stage 2 of PSCell List of RA report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, CATT, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current stage 2 textual description of how to use PSCell contained in E-UTRA RA Report refer to routing to gNB as SN is not correct. Based on meeting minute RAN2#124 in R2-2313567: UE report all available NR RACH report based on request from LTE BS. It is RAN2’s understanding the RA report retrieved by LTE BS could generated from an en-gNB or from a gNB. The current description limit the scope of target RAN node only relate to en-gNB is not correct and therefore need to be updated. In addition, the IE name PSCell List is misleading since the list not only includes PSCell but also include PCell. Impact Analysis: Impact assessment towards the previous version of the specification (same release): - This CR has limited impact since it only updates the textual description of stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update PScell list to NR cell list in section 10.18.5 - Update behavior of NR cell list in section 10.18.5 when the list be retrieved from a LTE node. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current textual description of PSCell List in stage 2 is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.18.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.423 CR 1270 TS36.423 CR 1788 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

10.18.5	RA Report retrieval

In MR-DC, when a UE performs successful random access attempts which are only known by the SN (e.g., beam failure recovery, UL synchronization issue, scheduling request failure, no PUCCH resource available), the SN may inform the MN about the occurrences of successful random access procedures in the SN via a RACH indication. The MN may then retrieve the RA Report from the UE(s) based on the RACH indication received from the SN.

A UE while being in EN-DC and NGEN-DC can collect E-UTRA RA Reports and NR RA Reports upon performing RACH in MN and SN respectively. When a E-UTRAN node retrieves the E-UTRA RA Report, it can also request UE to include the NR RA Report. If available, the UE then includes the NR RA Report in a container along with a list of NR Cells associated to the NR RA Report within the E-UTRA RA Report. The retrieving E-UTRAN node may then forward it to the corresponding NG-RAN nodes serving the NR Cells indicated within the E-UTRA RA Report.

In case of NGEN-DC, in case there is no Xn connectivity between the ng-eNB retrieving the NR RA Report from the UE and the gNB serving the NR Cells indicated by UE in the NR RA Report, the ng-eNB may forward the NR RA Report via Xn to an ng-eNB connected to a gNB serving the NR Cells indicated in the RA Report.

In case of EN-DC, in case there is no X2 connectivity between the eNB retrieving the NR RA Report from the UE and the en-gNB serving the NR Cells indicated by UE in the NR RA Report, the eNB may forward the NR RA Report via X2 to an eNB connected to an en-gNB serving the NR Cells indicated in the RA Report.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>