3GPP TSG-RAN WG3 #123bis	R3-242066

Changsha, China, 15-19 April 2024

Agenda Item:	11.2

Source:	Ericsson, Charter, InterDigital, Telecom Italia

Title:	AIML for Network Slicing

Document for:	Discussion, Agreement

# Introduction

As detailed in RP-234054, it has been agreed that new AI/ML based use cases will be studied as part of a Rel-19 Study Item (SI). Among the objectives of the SI is AI/ML based Network Slicing, as reported below from the Study Item Description (SID):

The detailed objectives of the SI are listed as follows:

- Study two new AI/ML based use cases, i.e., Network Slicing and CCO, with existing NG-RAN interfaces and architecture (including non-split architecture and split architecture).

- Rel-18 leftovers as candidates for normative work, based on the Rel-18 principles, as follows:

-   Mobility optimization for NR-DC

-   Split architecture support for Rel-18 use cases based on the conclusions from Rel-18 WI

-   Energy Saving enhancements, e.g., Energy Cost Prediction

-   Continuous MDT collection targeting the same UE across RRC states

-   Multi-hop UE trajectory across gNBs

Note: RAN3 should take the Rel-18 discussions into account.

In this paper, we discuss the use case of AI/ML based network slicing and identify areas of study for RAN3 to pursue as part of the Rel-19 SI.

# Discussion

Network slicing, as defined in 3GPP TS 28.530, is a paradigm where logical networks/partitions are created, with appropriate isolation, resources and optimized topology to serve a purpose or service category (e.g. use case/traffic category, or for MNO internal reasons) or customers (logical system created "on demand"). A network slice is a logical network that provides specific network capabilities and characteristics, such that the requirements of the services running on top of the network slice are supported and/or satisfied.

As far as the NG-RAN is concerned, as presented in TS 38.300 [1], among many others, the following key principles apply for support of Network Slicing in NG-RAN:

RAN awareness of slices

-	NG-RAN supports a differentiated handling of traffic for different network slices which have been pre-configured. How NG-RAN supports the slice enabling in terms of NG-RAN functions (i.e. the set of network functions that comprise each slice) is implementation dependent.

Resource management between slices

-	NG-RAN supports policy enforcement between slices as per service level agreements. It should be possible for a single NG-RAN node to support multiple slices. The NG-RAN should be free to apply the best RRM policy for the SLA in place to each supported slice.

On the other hand, NG-RAN based AIML as defined in Release 18 plays a key role in support of coordinated radio resources management between neighbor gNBs. As such, the Release 18 AI/ML framework could be extended to enable coordinated radio resource management between neighbor gNBs for the purpose of fulfilling the RAN related requirements mapping to the SLA in place for each supported slice.

Proposal 1: RAN3 to study extensions of the Release 18 AI/ML framework to enable coordination actions between neighbour gNBs for the purpose of RAN related network slice requirements fulfilment.

In order to work towards this direction, the first step is to assume that the RAN is configured with clear requirements concerning each supported network slice. This assumption is in line with what described in TS38.300, which states that:

“NG-RAN supports policy enforcement between slices as per service level agreements. It should be possible for a single NG-RAN node to support multiple slices. The NG-RAN should be free to apply the best RRM policy for the SLA in place to each supported slice.”

Hence the above text confirms that the RAN is aware of per slice requirements mapping to the slice´s SLA. In general, such requirements are agreed upon between the slice vertical and the network/service provider. The contractual framework for this is formally expressed as a Service Level Agreement (SLA) and then translated into requirements on measurable metrics for each sub-domain in the network. The OAM system has the responsibility of translating the SLA to targets that every subsystem of the access and the transport networks can understand and fulfill.

Once the above is established, the solution we propose to study would entail two or more RAN nodes to collaborate with one another, in terms of exchanging information, predictions, triggering actions etc., to achieve fulfilment of the requirements established for commonly supported slices.

Proposal 2: RAN3 to study the extension of the DATA COLLECTION REPORTING procedure to support exchanging of measured/predicted metrics reflecting per slice requirements to enable AI/ML based coordination of actions aimed at achieving per slice requirement fulfilment.

## Network Slice specific requirements for the NG-RAN

Requirements that may be typically configured at a gNB and derived for a network slice could be in the form of measurable targets to be fulfilled for a given percentage of session time.

As an example, for a given S-NSSAI, the RAN may be configured with a throughput target in DL and a throughput target in UL. The requirement for the network slice would be to be at or above such throughput target for at least x% of the slice session time.

Per slice requirements at the gNB may be established on a per cell or per node basis. Examples of such requirements may be as follows:

- Average per slice DL throughput for percentage of session time T

- Average per slice UL throughput for percentage of session time T

- Average per slice DL packet delay for percentage of session time T

- Average per slice UL packet delay for percentage of session time T

- Average per slice DL packet loss rate for percentage of session time T

- Average per slice UL packet loss rate for percentage of session time T

The parameter T may be specific and different for each requirement.

Proposal 3: RAN3 to study RAN specific per slice requirements based on measurable metrics used to determine the level of fulfillment of such requirements.

## Requirement fulfilment status in RAN nodes

A slice may be supported in across the entire network, or it may be geographically confined to a few cells or nodes. Regardless, whenever a node/cell cannot offer the required performance for traffic belonging to a slice, the overall network wide performance for the slice is lowered and the operator is at risk of not fulfilling its SLA for the slice.

In such cases, one potential action that could bring the node/cell back to fulfilling the per slice requirements is to offload part of its traffic to a neighbor cell/node. Such an action will free resources at the source node/cell and therefore allow to serve the network slice affected by poor performance in a better way.

However, to perform such an action, the source RAN node should be aware of a measured/predicted per slice resource status and requirement fulfilment status at the target node. Namely, as a minimum, the source node needs to receive from the target node the following:

- An indication of the measured available capacity per slice in UL and DL (this is already available via the Xn: Resource Status Reporting procedure)

- An indication of the predicted available capacity per slice in UL and DL

- An indication per slice of the measured requirement fulfillment level, e.g. for each network slice requirement, whether it is fulfilled or not fulfilled. This information could also include details about the delta between current measured values and the target requirements for such targets.

- An indication per slice of the predicted requirement fulfillment level, e.g. for each network slice requirement, whether it is predicted to be fulfilled or not fulfilled. This information could also include details about the delta between the predicted values and the target requirements for such targets.

With this information the source node is able to determine whether an offloading action towards a neighbour node is beneficial in terms of fulfilling the per slice requirements. As an example, if a target gNB predicts that its per slice targets will be fulfilled in the future and if the source node calculates that, by offloading a certain amount of traffic towards the target node, it will fulfil its per slice requirements, then the offloading action would be overall beneficial and the source node can carry it out.

Proposal 4: It is proposed to base the AI/ML supported Network Slicing use case on signalling between gNBs of measured/predicted per slice available capacity as well as measured/predicted per slice requirement fulfilment level.

## Cluster-level observability

While having access to information about neighbor nodes’ fulfilment levels for slice specific requirements helps taking decisions on offloading actions with the purpose of improving the overall level of fulfilment of the per slice requirements, a further enhancement to the concept described above consists of enabling the RAN to have visibility on per slice fulfilment for a cluster of gNBs (including itself). Such cluster of nodes may be formed by gNBs that are interconnected via Xn and that cover an area where a given slice is supported.

This information could enable a single gNB to understand the impact of fulfilling/not-fulfilling its own per slice requirements on the per cluster level of requirement fulfilment. Such per cluster level of fulfilment could be signalled to the RAN by the OAM and it could be taken as an input by the AI/ML algorithm that infers the best actions to take to achieve global per slice requirements fulfilment.

Proposal 5: It is proposed to study the benefits of allowing the OAM to signal to the RAN a per cluster level of fulfilment for the RAN requirements assigned to a network slice supported in the whole cluster.

## AI/ML assisted network slicing

Based on the proposals above, the concept described in this paper in support for an AI/ML based network slicing use case relies on the following points:

- RAN nodes are configured with per slice target requirements, consisting of target established for measurable metrics at the RAN

- RAN nodes exchange measured/predicted per slice available capacity and requirement fulfilment levels

- Based on the received information and internal measurements/predictions, RAN nodes infer actions allowing a better overall fulfilment of the per slice requirements

- As an enhancement, RAN nodes receive from the OAM a RAN-node-cluster level of network slice requirement fulfilment. This helps RAN nodes to understand how network slice requirements are fulfilled at a wider area level and how a single RAN node performance affects such wider target

# Conclusions and Proposals

In this paper, new concepts have been presented in support for AI/ML based network slicing use cases. The following proposals have been derived:

Proposal 1: RAN3 to study extensions of the Release 18 AI/ML framework to enable coordination actions between neighbour gNBs for the purpose of RAN related network slice requirements fulfilment.

Proposal 2: RAN3 to study the extension of the DATA COLLECTION REPORTING procedure to support exchanging of measured/predicted metrics reflecting per slice requirements to enable AI/ML based coordination of actions aimed at achieving per slice requirement fulfilment.

Proposal 3: RAN3 to study RAN specific per slice requirements based on measurable metrics used to determine the level of fulfilment of such requirements.

Proposal 4: It is proposed to base the AI/ML supported Network Slicing use case on signalling between gNBs of measured/predicted per slice available capacity as well as measured/predicted per slice requirement fulfilment level.

A TP mirroring the proposals above is available in R3-242065.

# References

TS 36.300  v. 18.0.0.