3GPP RAN WG3 Meeting #123	R3-240219

# Athens, Greece, 26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.415 | CR | 0041 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on PDU Set Information User Plane Protocol |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In section 4.1, the container name of PDU Set Information user plane protocal should be “PDU Set Information Container” not “GTP-U Container.” In section 6.4.1.1, it is stated that the DL PDU SET INFORMATION frame may include the PDU Set Information. However, the PDU Set Information is mandatorily present. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 4.1, the “GTP-U Container” is changed to be “PDU Set Information Container”. In section 6.4.1.1, the ‘may’ is removed in ‘The DL PDU SET INFORMATION frame may include the PDU Set Information …’. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the PDU Set Information user plane protocol related function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some errors exist in the text. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1; 6.4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------1st Change ---------------------------------------------------------------

## 4.1	General aspects

The PDU Session User Plane protocol and PDU Set Information User Plane protocol are located in the User Plane of the Radio Network Layer above the Transport Network Layer of the interface.

Each PDU session User Plane protocol instance and PDU Set Information User Plane protocol instance are associated to one PDU Session.

In this version of the present document, the PDU session user plane protocol data is conveyed by GTP-U protocol means, more specifically, by means of the "PDU Session Container" GTP-U Extension Header as defined in TS 29.281 [3].

In this version of the present document, the PDU set information user plane protocol data is conveyed by GTP-U protocol means, more specifically, by means of the "PDU Set Information Container" GTP-U Extension Header as defined in TS 29.281 [3].

------------------------------------------------------2nd Change ---------------------------------------------------------------

### 6.4.1	Transfer of DL PDU Set Information

#### 6.4.1.1	Successful operation

The purpose of the Transfer of DL PDU Set Information procedure is to send PDU Set information and indication of End of Data Burst related to a QoS flow from UPF to NG-RAN node or between NG-RAN nodes, or from gNB-CU to gNB-DU.

The DL PDU SET INFORMATION frame includes a QoS Flow Identifier (QFI) field associated with the transferred packet. The NG-RAN shall use the received QFI to determine the QoS flow and QoS profile which are associated with the received packet.

The DL PDU SET INFORMATION frame includes the PDU Set Information (i.e., PDU Set Sequence Number, PDU Sequence Number within a PDU Set, PDU Set Size, PDU Set Importance, and End PDU of the PDU Set) as specified in TS 23.501 [5].

The DL PDU SET INFORMATION frame may include indication of End of Data Burst as specified in TS 23.501 [5].

Figure 6.4.1.1-1: Successful Transfer of DL PDU Set Information

Figure 6.4.1.1-2: Successful Transfer of DL PDU Set Information