3GPP TSG-RAN3 Meeting # 123	R3-240568

# Athens, Greece,  26-02-2024 – 02-03-2024

Agenda Item:	8.3

Source:	Ericsson, ZTE, CATT

Title:	Support of Sub One Second Report Interval following CT4 LS

Document for:	Discussion

# Introduction

This paper describes the impacts on NRPPa following the LS from CT4 in [1].

The outcome from last meeting’s discussion in AI 8.1 is to postpone the discussion of introducing the new IE to RAN3 Athens meeting [2]:

RAN3 agrees to introduce millisecond granularity in the UE Reporting Information IE in NRPPa and F1AP. The encoding needs to be aligned with LPP, this is pending RAN2. Topic is postponed to next meeting.

# Discussion

The LS from CT4 states the following:

1. Overall Description: CT4 has agreed following CRs from Rel-17 at CT4#115e, which specified support of periodic deferred location reporting with interval less than 1 second: - GMLC SBI interface (CR0102 of TS 29.515) - AMF SBI interface (CR0892 of TS 29.518) - LMF SBI interface (CR0165 of TS 29.572) - MAP interface (CR1267 of TS 29.002). The CRs were approved at CT#100 and implemented in 3GPP June 2023 public release. The agreed CRs may have LPP and/or NRPPA impacts. 2. Actions: To RAN WG2 and RAN WG3 group: ACTION: CT4 kindly asks RAN2 and RAN3 to take the above information into account and proceed to update the related specifications if needed.

##### 6.1.6.2.24	Type: PeriodicEventInfo

Table 6.1.6.2.24-1: Definition of type PeriodicEventInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| reportingAmount | ReportingAmount | M | 1 | Number of event reports |
| reportingInterval | ReportingInterval | M | 1 | Interval of event reports |
| reportingInfiniteInd | boolean | O | 0..1 | When present, this IE shall be set to the value true, indicating that as many reports as possible to be generated during allowed duration (8639999 seconds). When this IE is present in a request, the reportingAmount should be set to the largest possible value (see NOTE) for backward compatibility consideration. An LMF supporting this IE shall ignore the reportingAmount IE and an LMF not supporting this IE will use reportingAmount IE to generate indicated number of reports as legacy. When this IE is present in a response, it indicates that this IE is supported and accepted by the LMF. |
| reportingIntervalMs | ReportingIntervalMs | O | 0..1 | When present, this IE shall indicate the Interval of event reports in milliseconds. When this IE is present in a request, the reportingInterval is set ot 1 for backward compatible consideration. An LMF supporting this IE shall ignore the reportingInterval IE; an LMF not supporting this IE will use reportingInterval IE to generate report with minimal interval as legacy, i.e. 1 second. When this IE is present in a response, it indicates that this IE is supported by the LMF and it shall indicate the value accepted by the LMF. |
| NOTE: reportingAmount x reportingInterval shall not exceed 8639999 (99 days, 23 hours, 59 minutes and 59 seconds) for compatibility with OMA MLP and RLP. |  |  |  |  |

In RAN3 specs, the Reporting Interval is on seconds. CT4 have added a new IE of reporting interval in ms:

| ReportingIntervalMs | integer | Event reporting periodic interval in milliseconds. Minimum: 1. Maximum: 999. |
| --- | --- | --- |

### 9.2.70	UE Reporting Information

This IE contains the UE Reporting Information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reporting Amount | M |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | Value 0 represents an infinite number of periodic reporting |
| Reporting Interval | M |  | ENUMERATED (none, 1, 2, 4, 8, 10, 16, 20, 32, 64) | Unit: seconds |

Observation 1: In NRPPa reporting amount for a configured UE does not support sub-ms reporting Interval.

To align with CT4, it is needed to add a new IE Reporting Interval in Milliseconds to the UE Reporting Information, which indicates the interval between location information reports and the response time requirement for the first location information report in milliseconds.

| Reporting Interval in Milliseconds | O |  | INTEGER (1..999) | Indicates the interval between location information reports and the response time requirement for the first location information report in milliseconds. Unit: milliseconds |
| --- | --- | --- | --- | --- |

Proposal 1: To align with CT4, the UE configured Reporting Interval should be extended to milliseconds values.

This change affects NRPPa and F1AP. A set of Cat F/A CRs to NRPPa is provided in [3-4].

# Conclusion

Observation 1: In NRPPa reporting amount for a configured UE does not support sub-ms reporting Interval.

Proposal 1: To align with CT4, the UE configured Reporting Interval should be extended to milliseconds values.

# References

- R3-237135 LS on Sub One Second Report Period for Deferred Location over SBI

- R3-237741, Summary of proposals for CB: # 1_ReportPeriod

- R3-240569, CR to NRPPa on addition of milliseconds reporting interval, Cat F, Rel-17

- R3-240570, CR to NRPPa on addition of milliseconds reporting interval, Cat A, Rel-18