3GPP TSG-RAN WG3 Meeting #123	R3-241133

# Athens, GR, 26th Feb – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1131 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Complete Configuation Indicator in XnAP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, ZTE, Ericsson, LG Electronics, Nokia, Nokia Shanghai Bell, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-03-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In R18, in order to support reference configuraiton for S-CPAC, we introduce a new IE named Complete Configuation Indicator IE with emum-type value“complete config” and “delta config“ to indicate the S-CPAC candidate configuration provided by the S-NG-RAN node in S-NODE ADDITION REQUEST ACKNOWLEDGE and S-NODE MODIFICATION REQUEST ACKNOWLEDGE message. The IE name is aligned with what we have introduced in F1AP but the emum-type value is not the same, in F1AP, the enum-type value for the LTM Complete Configuration indicator IE is only “complete”. This CR is to align the enum-type value of the Complete Configuation Indicator IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For section 9.2.3.194, remove the emum-type value “delta config“ in Complete Configuration Indicator IE; - For section 9.3.5, correct the correspionding ASN.1 part. Impact Analysis: This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because the changes only impact the procedure for S-CAPC functions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Redundant value for complete configuration remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.194, 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: R3-240190. Rev1: R3-241106, remove the procedure descriptions updates. Rev2: R3-241133, correct the typo in the cover page. |  |  |  |  |  |  |  |  |

START OF CHANGE

9.2.3.194	Complete Configuration Indicator

This IE indicates that the complete configuration is used at the S-NG-RAN node for S-CPAC.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Complete Configuration Indicator | M |  | ENUMERATED (complete config, ...) |  |

NEXT CHANGE

9.3.5	Information Element definitions

//irrelevant part skipped//

#### -- C

//irrelevant part skipped//

CNsubgroupID ::= INTEGER (0..7, ...)

CompleteConfig-Indicator ::= ENUMERATED {complete-config, ...}

Conditional-Reconfig-List ::= SEQUENCE (SIZE(1..maxnoofPSCellCandidates)) OF Conditional-Reconfig-Item

//irrelevant part skipped//

END OF CHANGE