3GPP TSG-RAN WG3 Meeting #123	R3-240901

Athens, Greece, 26th February – 1st March 2024

Agenda Item:	9.1.9.1

Source:	Lenovo (moderator)

Title:	SoD of CB: # QoE1_Inactive CB: # XR2_ECNMarkingDiscard

Document for:	Approval

Introduction

This contribution provides the summary of offline discussion on:

CB: # XR2_ECNMarkingDiscard

- Check ECN marking corrections

- Discuss the open issues related to PSU Set Discard and capture agreements

(moderator - Lenovo)

Summary of offline disc R3-240901

For the Chairman’s Notes

Discussion

UL PSI based discard

Issue 1: Which procedure is used for the new PSI based discard indication, UE context setup/modification procedure or DL RRC message transfer procedure?

During online discussion, it was agreed that:

For UL: Introduce a per DRB new indication in F1AP from CU to DU

But it is still open for which procedure the new indication should be included:

UE context setup/modification procedure or RRC message transfer procedure?

Proposal 1: the xxxx procedure is used for the new PSI based discard indication.

DL PSI based discard

Basic function:

Proposal 2: Add the PSI based Discard timer in the PDCP configuration IE in E1AP.

How to use the PSI based discard timer for DL PSI based discard is up to gNB-CU-UP’s implementation.

Other enhancements:

| R3-240492 | Support of PSI based discard (Huawei, Qualcomm, Lenovo) | discussion |
| --- | --- | --- |

Proposal 4: The gNB-DU can send the suggestion on DL PSI based SDU discard activation/deactivation to the gNB-CU-UP if DL PSI-based discard is configured.

| R3-240580 | Discussion on PDU Set discarding and other topics (Ericsson) | discussion |
| --- | --- | --- |

Proposal 1: Introduce PSI in F1-U DL USER DATA frame.

Proposal 2: Introduce information on transmitted PDCP PDU Set SN and  discarded PDCP PDU Set SN information in the F1-U DL Data Delivery Status frame.

Proposal 3: Introduce the PDU Set transmission status in the E1 Bearer Modification Request / Response messages and in the Xn SN Status Transfer message

Proposal 5: The gNB-DU provides the discarding status result (on, off) in the UL RRC MESSAGE TRANSFER message.

| R3-240298 | Support of DL PSI-based discard and UL Traffic Information (Xiaomi) | CR0108r, TS 37.483 v18.0.0, Rel-18, Cat. F |
| --- | --- | --- |

Proposal: Introudce a Uplink Traffic Information IE in QoS Flow QoS Parameters List IE

| R3-240217 | Corrections for PDU set related issues (Lenovo) | discussion |
| --- | --- | --- |

- Once gNB-CU reconfigures the PSI-based discard timer for DRBs of the UE, the gNB-CU informs gNB-DU that the PSI-based discard timer is reconfigured for the DRB(s).

ECN Marking

Issue 1:

| R3-240218 | Corrections on ECN Marking (Lenovo, Huawei, CATT) | CR1065r, TS 38.413 v18.0.0, Rel-18, Cat. F |
| --- | --- | --- |
| R3-240438 | Report the ECN Marking or Congestion Information Reporting Status during Xn-HO and NG-HO (Nokia, Nokia Shanghai Bell) | CR1094r, TS 38.413 v18.0.0, Rel-18, Cat. F |

Proposal 1a: Add the ECN Marking or Congestion Information Reporting Status IE in the Path Switch Request Transfer IE and Handover Request Acknowledge Transfer IE.

Proposal 1b: take R3-240438 as starting point for proposal 1a.

Issue 2:

| R3-240497 | Correction on ECN marking for L4S and data forwarding (Huawei, Lenovo, CMCC, Deutsche Telekom) | draftCR |
| --- | --- | --- |

Proposal 2a: Add the description about how the gNB perform ECN marking for L4S in 38.300.

Proposal 2b: Take R3-240497 as starting point for proposal 1b.

Issue 3:

| R3-240697 | Corrections to TS 37.483 to support XR (ZTE, CATT, CMCC, China Telecom) |
| --- | --- |

Proposal 3a: reports the editorial change to E1 rapporteur. The other change can be discussed in PDU set offline discussion.

References