3GPP TSG-RAN WG3 Meeting #124	R3-243125

Fukuoka, Japan, 20 – 24 May 2024

Agenda item:	17.2

Source:	Nokia

Title:	On-Demand SSB SCell operation for UEs in connected mode

Document for:	Discussion

# 1	Introduction

In this paper we provide information for monitoring of potential RAN3 impacts of on-demand SSB SCell operation for UEs in connected mode based on the current discussion state in RAN1 [1].

# 2	Discussion

RAN1 has made following agreements regarding scenarios and signaling for on-demand SSB:

Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. Agreement For a cell supporting on-demand SSB SCell operation, further study the following options. - Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication. - Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided. - FFS: Details of the signaling - Other options are not precluded. - FFS: Details on On-demand SSB transmission indication

Our understanding is that network triggering of SSB transmission will be decided by the gNB-DU in the discussed scenarios. RAN3 impact on e.g. F1 signalling, if any, can only be determined after further down-selection in RAN1 among options listed above and associated UE-related signalling decisions in RAN2.

RAN3 should therefore continue monitoring agreements taken by RAN1 and RAN2 for this topic.

Proposal: RAN3 to continue monitoring agreements taken by RAN1 and RAN2 for on-demand SSB SCell operation for UEs in connected mode.

# 3	Conclusions

We have made the following proposal:

Proposal: RAN3 to continue monitoring agreements taken by RAN1 and RAN2 for on-demand SSB SCell operation for UEs in connected mode.

# References

[1]		R1-2404223, On-demand SSB SCell Operation, Nokia, Nokia Shanghai Bell