3GPP TSG-RAN WG3 Meeting #124	R3-243198

Fukuoka, Japan, from 20 – 24 May 2024

Title:	Xn Gateway in Femto architecture

Source:	Charter Communications

Agenda Item:	12.3

Document for:	Discussion and decision

# 1	Introduction

In RAN3 #123-bis, the first draft of the study item on “Additional Topological Enhancements for NR” (R3-242242) was created post-meeting. In particular section 5.2 deals with the proposed architectures of Femto.  The subject of the Xn Gateway was left as an FFS issue, with contributions to be presented at this meeting.

# 2	Discussion

The 4G Femto architecture is depicted in TS 36.300 as shown below

And, a more detailed logical view of that architecture is depicted in the figure below from TS 36.300.

Furthermore, in section 6.2.3.3 of the Study Item TR 37.803, what was then called “X2 Routing Proxy”, and is today called X2 GW, it was said “Such a solution would provide flexibility to satisfy various deployment requirements”.

Given that Xn in 5G is the equivalent of X2 in 4G, and the equivalent of a X2 GW would therefore be a Xn GW in 5G, it is proposed that:

Proposal 1: An Xn GW may be included in the 5G Femto architecture.

# 3	Conclusion

Based on the discussion in this paper, we make the following proposal:

Proposal 1: An Xn GW may be included in the 5G Femto architecture.

HgNB

gNB

HgNB

SeGW

XnGW

Xn-c

Xn-c

5G Core

NG

NG

HgNB
Mgmt
System

Xn-c