3GPP TSG-RAN WG3 Meeting #124	R3-243228

Fukuoka, Japan, 20 – 24 May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0394 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on BC session path update for network sharing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CBN, Nokia, Qualcomm Incorporated, Samsung, CATT, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_enh-Core |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on previous discussions, the handling of shared NG-U tunnel switch in case the PLMN of the established shared NG-U triggers Broadcast Session Release Request but the session is kept by the other PLMN(s) are quite different in case of MOCN and in case of network sharing with multiple cell-ID broadcast, but the corresponding call flows are missing. The step 10 in existing Figure 8.15.1.1b-1 should be updated from “E1AP BC Bearer Context Setup Response” to “E1AP BC Bearer Context Modification Response”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce call flows to show, in case the PLMN of the established shared NG-U triggers Broadcast Session Release Request but the session is kept by the other PLMN(s), the different handing in case of MOCN and in case of network sharing with multiple cell-ID broadcast. Update the step 10 in existing Figure 8.15.1.1b-1, from “E1AP BC Bearer Context Setup Response” to “E1AP BC Bearer Context Modification Response”. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has no impact with the previous version of the specification (same release) because it adds missing scenarios of broadcast for resource efficiency for network sharing scenarios. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The BC path update procedure is unclear. Error remains in step 10 in existing Figure 8.15.1.1b-1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.15.1.1a, 8.15.1.1b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------Start of the Change----------

#### 8.15.1.1a	Broadcast MBS Session Setup for resource efficiency for MOCN

Figure 8.15.1.1a-1 illustrates an exemplified interaction of NGAP, E1AP, F1AP and RRC protocol functions at Broadcast MBS Session Setup with the support of resource efficiency for MOCN.

Figure 8.15.1.1a-1: Broadcast MBS Session Setup for resource efficiency for MOCN

1.	The 5GC1 triggers the Broadcast MBS Session Setup procedure for TMGI 1 as defined in section 8.15.1.1, and the Associated Session ID x is provided together with TMGI 1 in step 1, 2 and 5 of 8.15.1.1. The broadcast MBS media stream received from 5GC1 is provided to the UEs of PLMN1.

2.	The 5GC2 triggers the Broadcast MBS Session Setup procedure for TMGI 2 with the Associated Session ID x.

3.	Upon receiving the same Associated Session ID x, the gNB-CU-CP may determine to not establish shared NG-U tunnel for PLMN2 with 5GC2.

4.	The gNB-CU-CP triggers the F1AP Broadcast Context Setup Request for TMGI 2 with the Associated Session ID x towards the gNB-DU.

5.	Upon receiving the same Associated Session ID x, the gNB-DU shall determine to not establish shared F1-U tunnel for PLMN2.

6.	The gNB-DU triggers the F1AP Broadcast Context Setup Response for TMGI 2 to the gNB-CU, and indicates that the shared F1-U tunnel is not established.

7.	The gNB-DU provides configuration of both TMGI 1 and TMGI 2 by means of MCCH. The broadcast MBS media stream received from 5GC1 is provided to both UEs of PLMN1 and UEs of PLMN2.

8.	The gNB-CU-CP sends NGAP Broadcast Session Setup Response towards 5GC2.

Figure 8.15.1.1a-2 shows the procedure of shared NG-U tunnel switch in case the PLMN of the established shared NG-U triggers Broadcast Session Release Request but the session is kept by the other PLMN(s).

Figure 8.15.1.1a-2: Broadcast MBS Session shared NG-U tunnel switch for MOCN

NOTE x: In this signalling flow, it is assumed that during step 0, the shared NG-U is only established with 5GC1. And it is assumed that the same gNB-CU-UP is used for PLMN1 and PLMN2.

0.	TMGI 1 and TMGI 2 are established during Broadcast MBS Session Setup for resource efficiency for MOCN as defined in 8.15.1.1a-1.

1.	The 5GC1 triggers the Broadcast Session Release Request for TMGI 1.

2a.	In case of NG-U multicast transport, the gNB-CU-CP triggers BC Bearer Context Modification Request towards the gNB-CU-UP, and provides the IP multicast address and the IP source specific multicast address received in step 0 from 5GC2.

3a~3a1. In case of NG-U multicast transport, the gNB-CU-UP sends BC Bearer Context Modification Response to the gNB-CU-CP, and joins the NG-U multicast group of 5GC2.

2b.	 In case of NG-U unicast transport, the gNB-CU-CP triggers NGAP Broadcast Session Transport Request for TMGI 2 towards 5GC2, and provides the gNB NG-U GTP DL TEID which was allocated by the gNB-CU-UP in step 0.

3b. In case of NG-U unicast transport, the 5GC2 sends Broadcast Session Transport Response towards the gNB-CU-CP, and start to transmit Broadcast MBS Media Stream towards the received gNB NG-U GTP DL TEID.

4.	The gNB-CU-CP triggers F1AP Broadcast Context Release Command with the F1AP MBS association allocated for TMGI 1 towards the gNB-DU.

5.	The gNB-DU keeps the shared F1-U tunnel(s), and removes the MCCH configuration of TMGI 1 over the radio.

6.	The gNB-DU sends Broadcast Context Release Complete towards the gNB-CU-CP.

7.	The gNB-CU-CP sends Broadcast Session Release Response about TMGI 1 towards 5GC1.

NOTE y: step 2~3 and step 4~5 and step 7 can be performed in parallel.

#### 8.15.1.1b	Broadcast MBS Session Setup for resource efficiency for RAN sharing with multiple cell-ID broadcast

Figure 8.15.1.1b-1 illustrates an exemplified interaction of NGAP, E1AP, F1AP and RRC protocol functions at Broadcast MBS Session Setup with the support of resource efficiency for RAN sharing with multiple cell-ID broadcast.

The gNB-DU1/2 entity shown in Figure 8.15.1b-1 is a simplified representation of the gNB-DU1 of PLMN 1, the gNB DU2 of PLMN 2 and respective radio resources of the shared cell.

Figure 8.15.1.1b-1: Broadcast MBS Session Setup for RAN sharing with multiple cell-ID broadcast

1.	The 5GC1 triggers the Broadcast MBS Session Setup procedure towards gNB-CU-CP1 for TMGI 1 as defined in section 8.15.1.1, and the Associated Session ID x is provided together with TMGI 1 in step 1, 2 and 5 of 8.15.1.1. The broadcast MBS media stream received from 5GC1 is provided to the UEs of PLMN1 via the gNB-CU-UP1 and the gNB-DU1.

2.	The 5GC2 triggers the Broadcast MBS Session Setup procedure for TMGI 2 with the Associated Session ID x towards gNB-CU-CP2.

3/4.	The gNB-CU-CP2 sets up the broadcast bearer context with gNB-CU-UP2 including the Associated Session ID x. The gNB-CU UP uses the received information to correlate the MBS sessions and include the shared NG-U termination endpoint if applicable.

5.	The gNB-CU-CP2 triggers the F1AP Broadcast Context Setup Request for TMGI 2 with the Associated Session ID x towards the gNB-DU2.

6.	Upon receiving the same Associated Session ID x, the gNB-DU1/2 may determine to not establish shared F1-U tunnel for PLMN2.

7.	The gNB-DU2 triggers the F1AP Broadcast Context Setup Response for TMGI2 to the gNB-CU-CP2, and indicates that the shared F1-U tunnel is not established.

8.	The gNB-CU-CP2 may determine to not establish shared NG-U tunnel for PLMN2.

9/10.	The gNB-CU-CP2 triggers E1AP BC Bearer Context Modification procedure towards the gNB-CU-UP2, and indicates that the shared F1-U tunnel is not established.

11.	The gNB-DU1/2 provides configuration of both TMGI 1 and TMGI 2 by means of MCCH. The broadcast MBS media stream received from 5GC1 is provided to both UEs of PLMN1 and UEs of PLMN2.

12.	The gNB-CU-CP2 sends NGAP Broadcast Session Setup Response towards 5GC2.

Figure 8.15.1.1b-2 shows the procedure of shared NG-U tunnel and shared F1-U tunnel switch in case the PLMN of the established shared NG-U and shared F1-U triggers Broadcast Session Release Request but the session is kept by the other PLMN(s).

Figure 8.15.1.1b-2: Broadcast MBS Session shared NG-U tunnel and shared F1-U tunnel switch for RAN sharing with multiple cell-ID broadcast

NOTE x: In this signalling flow, it is assumed that during step 0, the shared NG-U and shared F1-U tunnel(s) are only established for PLMN1.

0.	TMGI 1 and TMGI 2 are established during Broadcast MBS Session Setup for resource efficiency for RAN sharing with multiple cell-ID broadcast as defined in 8.15.1.1b-1.

1.	The 5GC1 triggers the Broadcast Session Release Request for TMGI 1 towards the gNB-CU-CP1.

2 and 3. 	The gNB-CU-CP1 triggers the BC Bearer Context Release Command towards the gNB-CU-UP1 and gNB-CU-UP1 provides corresponding response.

4.	The gNB-CU-CP1 triggers Broadcast Context Release Command towards the gNB-DU1.

5.	The gNB-DU1/2 determines to establish shared F1-U of PLMN2, the gNB-DU2 triggers Broadcast Transport Resource Request towards the gNB-CU-CP2, and may provide the F1-U TNL(s) at DU for the corresponding MRB(s).

6.	The gNB-CU-CP2 triggers BC Bearer Context Modification Request towards the gNB-CU-UP2, and provides the F1-U TNL(s) at DU if received in step 5. In case of NG-U multicast transport, the gNB-CU-CP2 also provides the 5GC NG-U TNL received from 5GC2 in step 0.

7.	The gNB-CU-UP2 sends BC Bearer Context Modification Response towards the gNB-CU-CP2, provides the F1-U TNL(s) at CU. In case of NG-U unicast transport, the gNB-CU-UP2 also provides the gNB NG-U TNL.

7a. In case of NG-U multicast transport, the gNB-CU-UP2 joins the NG-U multicast group of 5GC2.

8.	In case of NG-U unicast transport, the gNB-CU-CP2 triggers Broadcast Session Transport Request for TMGI 2 towards 5GC2, and provides the gNB NG-U GTP DL TEID which was allocated by the gNB-CU-UP2 in step 7.

9. In case of NG-U unicast transport, the 5GC2 sends Broadcast Session Transport Response towards the gNB-CU-CP2, and start to transmit Broadcast MBS Media Stream towards the received gNB NG-U GTP DL TEID.

10.	The gNB-CU-CP2 triggers Broadcast Context Modification Request towards the gNB-DU2, provides the F1-U TNL(s) at CU received in step 7.

11.	The gNB-DU2 sends Broadcast Context Modification Response to the gNB-CU-CP2.

11a, 11b. If F1-U TNL at DU is received in step 11, the gNB-CU-CP further triggers BC Bearer Context Modification Request towards the gNB-CU-UP, and includes the F1-U TNL at DU received in step 11. Otherwise, these two steps are skipped.

12.	The gNB-DU1/2 removes the MCCH configuration of TMGI 1 over the radio.

13.	The gNB-DU1 sends Broadcast Context Release Complete towards the gNB-CU-CP1.

14.	The gNB-CU-CP1 sends Broadcast Session Release Response towards the 5GC1.

NOTE y: step 3 and step 4 can be performed in parallel. Step 5~11 and step 12 and step 13~14 can be performed in parallel.

------------End of the Change-----------