3GPP TSG-RAN WG3 #124 meeting	R3-243247

# Fukuoka city, Japan, 20th May– 24th May, 2024

Agenda item:		12.2

Source:		NEC

Title:		Consideration on architecture and mobility for WAB

Document for:		Discussion and decision

# 1. Introduction

In RAN3#123bis meeting, the general requirements and architecture for WAB were discussed.

In this contribution, we will further discuss aspects related to the architecture and mobility enhancements for WAB.

# 2. Discussion

2.1 WAB architecture and requirements.

TP for TR 38.799 [2] has captured the requirements for the WAB-node. However, it does not include any requirements for WAB-donor (i.e., gNB serving the WAB-MT). From our understanding, The WAB-donor provides NR Uu link for backhauling of WAB-node, and shall serve not only WAB-MT but also legacy UEs. It’s better to capture it in the TP to support the scenario that UEs may handover from WAB-node to WAB-donor.

Proposal 1: RAN3 to capture the following requirement in the TP:

- The WAB-donor (i.e., gNB serving the WAB-MT) provides NR Uu link for backhauling of WAB-node.

- The WAB-donor shall serve not only WAB-MT but also legacy UEs.

Similar to IAB, WAB-donor may also broadcast an indication of whether supporting WAB (e.g., wab-Support) via system information. Based on it, the WAB-MT can prioritize or only consider a cell broadcasting wab-Support as a candidate for cell selection, for example, for the selected PLMN or SNPN. In addition, WAB-MT may ignore cell-barring or cell-reservation indications contained in cell system information broadcast.

Proposal 2: WAB-donor to broadcast an indication of whether supporting WAB via system information.

Proposal 3: WAB-MT may ignore cell-barring or cell-reservation indications contained in cell system information broadcast.

For the access of WAB-MT, it can use the same way as legacy UEs to connect to the network. In order to let the network know that the connected UE is a WAB-MT and to assist the network selecting an AMF supporting WAB, the WAB-MT may indicate a WAB-node indication to the network in an early phase, for example, during random access procedure or in Msg5 (e.g., RRCSetupComplete message).

Proposal 4: WAB-MT to indicate a WAB-node indication to the network during random access procedure or in Msg5.

A WAB-node may serve one or more cells and all the data scheduled by the cells shall be aggregated on WAB-MT, the Uu link between WAB-MT and WAB-donor is highly likely to be the bottleneck for data transmission, and congestion may happen if the data load is huge. Hence, flow control for WAB-MT should be considered to improve user experience. For example, the traffic load/estimated traffic load of the cell served by a WAB-node may be reported to WAB-donor.

Proposal 5: Flow control for WAB-MT should be considered to avoid congestion on the link between WAB-MT and WAB-donor.

2.2 Mobility for UE or WAB-MT.

UE’s Handover

Figure 1: Scenarios for UE’s handover

As illustrated in Figure 1, irrespective of whether the 5GC node is changed, a UE served by WAB-node (i.e., on-board UE) may handover from WAB-node to WAB-donor (Scenario 1) or gNB other than WAB-donor (Scenario 2), for example, when UE gets off the bus. To support UE’s mobility, RAN3 should study how to manage the Xn interface between WAB-node and WAB-donor, and the Xn interface between WAB-node and RAN node other than WAB-donor.

Observation 1: To support UE’s mobility, RAN3 should study how to manage the Xn interface between WAB-node and WAB-donor, and the Xn interface between WAB-node and RAN nodes other than WAB-donor.

Figure 2: Protocol stacks for Xn-C/Xn-U interfaces between WAB-node and WAB-donor

For the Xn interface between WAB-node and WAB-donor, the protocol stack could be simple, and the data may be exchanged directly between these two nodes. As illustrated in Figure 2, XnAP or GTP-U encapsulation is performed by WAB-node or WAB-donor. Based on the IP address of the packet, WAB-donor can determine whether the destination of the packet is itself.

Proposal 6: RAN3 to agree the protocol stacks for Xn interface between WAB-node and WAB-donor in Figure 2.

Figure 3: Protocol stacks for Xn interface between WAB-node and RAN node other than WAB-donor (option 1)

For the Xn interface between WAB-node and RAN node other than WAB-donor, backhauling of XnAP or GTP-U packets over WAB-donor is required. One option for the protocol stack is illustrated in Figure 3, XnAP or GTP-U encapsulation is performed by WAB-node or RAN node 2. The IP packets carried XnAP or GTP-U data are forwarded as user plane data over NR Uu interface between WAB-node and WAB-donor, and then forwarded as user plane data over Xn-U interface between WAB-donor and RAN node 2. In this option, Xn-U interface between WAB-donor and RAN node 2 should be setup before the data forwarding.

Figure 4: Protocol stack for Xn-C interface between WAB-node and RAN node other than WAB-donor (option 2)

There is another option of protocol stack for enabling the XnAP interface between WAB-node and RAN node other than WAB-donor, which is illustrated in Figure 4. In this option, The XnAP message (encapsulated in IP) shall be transmitted via an RRC signalling (e.g., in a transparent container) between WAB-node and WAB-donor. Compared with option 1, transmission over RRC signalling may provide more reliability over the Uu interface.

Proposal 7: RAN3 to agree the protocol stacks for Xn interface between WAB-node and RAN node other than WAB-donor in Figure 3 and/or Figure 4.

WAB-MT’s Handover

For the mobility of WAB-MT, the legacy handover procedure for UE may be used as a baseline. Furthermore, RAN3 should consider the necessary WAB-related information exchanged between RAN nodes over Xn-AP to support WAB-MT’s handover:

- WAB Node Indication which indicates that the handover is for a WAB-node, e.g., in HANDOVER REQUEST message.

- BH PDU Session Indication may also be introduced to let the target RAN node know that a BH PDU Session should be setup for the WAB-node, e.g., in HANDOVER REQUEST message.

- WAB Support Indication for a cell, e.g., in XN SETUP REQUEST and XN SETUP RESPONSE message. Based on it, the RAN node can determine whether to choose the corresponding cell as a candidate target cell for WAB-MT.

- The information of cells served by the WAB-node or the WAB-donor, e.g., in XN SETUP REQUEST and XN SETUP RESPONSE message. The RAN nodes may consider this information (e.g., cell-specific signal/channel configuration and multiplexing info) for scheduling, resource coordination, or resource multiplexing. And regarding the resource multiplexing for WAB, we think the solution for IAB can be used as a baseline for the WAB resource multiplexing study.

Proposal 8: RAN3 to consider the following information exchanged between RAN nodes over Xn-AP to support mobility of WAB-MT:

- WAB Node Indication and/or BH PDU Session Indication in HANDOVER REQUEST message.

- WAB Support Indication and/or the information of cells served by the WAB-node or the WAB-donor in XN SETUP REQUEST and XN SETUP RESPONSE message.

# Summary

This contribution provides our analysis on potential enhancements for WAB, and has the following proposals:

Proposal 1: RAN3 to capture the following requirement in the TP:

- The WAB-donor (i.e., gNB serving the WAB-MT) provides NR Uu link for backhauling of WAB-node.

- The WAB-donor shall serve not only WAB-MT but also legacy UEs.

Proposal 2: WAB-donor to broadcast an indication of whether supporting WAB via system information.

Proposal 3: WAB-MT may ignore cell-barring or cell-reservation indications contained in cell system information broadcast.

Proposal 4: WAB-MT to indicate a WAB-node indication to the network during random access procedure or in Msg5.

Proposal 5: Flow control for WAB-MT should be considered to avoid congestion on the link between WAB-MT and WAB-donor.

Observation 1: To support UE’s mobility, RAN3 should study how to manage the Xn interface between WAB-node and WAB-donor, and the Xn interface between WAB-node and RAN nodes other than WAB-donor.

Proposal 6: RAN3 to agree the protocol stacks for Xn interface between WAB-node and WAB-donor in Figure 2.

Proposal 7: RAN3 to agree the protocol stacks for Xn interface between WAB-node and RAN node other than WAB-donor in Figure 3 and/or Figure 4.

Proposal 8: RAN3 to consider the following information exchanged between RAN nodes over Xn-AP to support mobility of WAB-MT:

- WAB Node Indication and/or BH PDU Session Indication in HANDOVER REQUEST message.

- WAB Support Indication and/or the information of cells served by the WAB-node or the WAB-donor in XN SETUP REQUEST and XN SETUP RESPONSE message.

# References

[1] RP-240319, Revised SID on Study on additional topological enhancements for NR

[2] R3-242236, TP for TR 38.799 on WAB general requirements and architecture

# Annex: TP for TR38.799

Start of Change

# X	Wireless Access and Backhaul (WAB)

## X.1	General

The study is based on the following requirements:

- The WAB-donor (i.e., gNB serving the WAB-MT) provides NR Uu link for backhauling of WAB-node.

- The WAB-donor shall serve not only WAB-MT but also legacy UEs.

## X.2	WAB Architecture

Figure X.2-X shows protocol stack examples of Control plane and User plane transport for an Xn interface between WAB-node and WAB-donor.

Figure X.2-X: Protocol stack examples of Control plane and User plane transport for an Xn interface between WAB-node and WAB-donor.

Figure X.2-Y shows protocol stack examples of Control plane and User plane transport for an Xn interface between WAB-node and RAN node other than WAB-donor.

Figure X.2-Y: Protocol stack examples of Control plane and User plane transport for an Xn interface between WAB-node and RAN node other than WAB-donor (option 1)

Figure X.2-Z shows another protocol stack examples of Control plane transport for an Xn interface between WAB-node and RAN node other than WAB-donor.

Figure X.2-Z: Protocol stack examples of Control plane transport for an Xn interface between WAB-node and RAN node other than WAB-donor (option 2)

End of Change