3GPP TSG-RAN WG3 Meeting #124	R3-243316

Fukuoka, Japan, 20th May – 24th May 2024

Agenda Item:		12.3

Source:		Baicells

Title:		(TP to TR 38.799) NG connection and interface for Option 2 for NR Femto Architecture

Document for:		Other

- Introduction

This contribution is to provide TP about NG connection and interface between NR Femto node and NR Femto GW for the Option 2 of the NR Femto architecture based on tdoc[1]

- Reference

- R3-243315, Discussion on NG connection and interface for Option 2 for NR Femto Architecture, Baicells

- TP for TR 38.799

5.2.1.2	Option 2

Figure 5.2.1.2-1 shows a logical architecture for the NR Femto that has a set of NG interfaces to connect the NR Femto node to the 5GC.

Figure 5.2.1.2-1: Option 2 for NR Femto Architecture

NOTE:	The SeGW and the NR Femto Management System are out of RAN3 scope.

The NG-RAN architecture may deploy an NR Femto Gateway (NR Femto GW) to allow the NG interface between the NR Femto node and the 5GC to support a large number of NR Femto nodes in a scalable manner. The NR Femto GW serves as a concentrator for the C-Plane, specifically the NG-C interface.

The NG interface is defined as the interface:

-	Between the NR Femto GW and the 5GC;

-	Between the NR Femto node and the NR Femto GW;

-	Between the NR Femto node and the 5GC;

The NR Femto GW appears to the AMF as a gNB. The NR Femto GW appears to the NR Femto node as an AMF. The NG interface between the NR Femto node and the 5GC is the same regardless of whether the NR Femto node is connected to the 5GC via an NR Femto GW or not.

5.2.1.2.1	Option 2